3GPP TSG-RAN WG4 Meeting #97-e	    R4-2016092

Electronic Meeting, 2nd – 13th November, 2020

Agenda item:	7.10.1.3

Source:	Ericsson

Title:	Discussion on FR2 DL 256QAM​ UE CQI testing

Document for:	Approval

# Introduction

In this paper, we present our views on UE CQI requirement tests for FR2 DL 256QAM, based on the WF from RAN4#96-e [1].

# Discussion

## Simulation assumptions

The simulations are based on the assumptions found in WF.

Table 1 Simulation assumptions

- Whether to define FR2 CQI reporting requirements for CQI table 2 - In the next meeting, decide whether to introduce the requirements for AWGN and/or fading conditions based on the simulation results under 50MHz CBW: - For AWGN condition, companies are encouraged to simulate the required SNR for achieving median CQI of [11, 12 and 13] in CQI table 2. - For fading condition, companies are encouraged to simulate the required SNR where CQI indices corresponding to 256QAM (i.e., 12 and higher) in CQI table 2 can be reported with at least [10%] probability. - If it is agreed to define FR2 CQI reporting test for CQI table 2, use channel bandwidth of 50MHz. - If it is agreed to define FR2 CQI reporting for CQI table 2, consider the following test applicability: - If UE passes the test with CQI Table 2, then it can skip the corresponding test with CQI Table 1 - Note: Similar to the existing CQI table 1 test, the test for CQI Table 2 will cover both lower SNR and higher SNR.

According to the WF, CQI index 11 maps to 64QAM modulation order which is outside of scope of the 256QAM demodulation work. Furthermore, the CQI table in and of itself does not verify performance of 256QAM operation in FR2. As has been discussed under the demodulation work, we’ve agreed to specify performance requirements with MCS20 from MCS table 2. MCS20 is the lowest configuration where 256QAM modulation order is employed. From a functionality perspective the testing points may be of concern when specifying performance requirements for CQI values which maps to higher MCS indices.

From Rel-15 eMBB FR2 CQI requirements under AWGN conditions and 100MHz channel bandwidth the testing points are specified at 14dB, and 15dB SNR. As we’ve seen in the initial simulations, these testing points are too low for FR2 256QAM operation. In such scenarios we’ve seen under fading conditions that 18dB is the lower edge for demodulation requirements under fading conditions at the 70% throughput mark.

For CQI requirements the configured BLER threshold for changing MCS may be selected at the 10% BLER causing even more stringent requirements for the required SNR. At the 90% (10% BLER) throughput mark under fading conditions demodulation requirements is somewhere over the 20dB SNR under ideal conditions. Bearing in mind that the CQI performance is testing the ability of the UE to report different CQI indices which maps to different 256QAM coding rates i.e., different MCS values.

## Simulation results and proposals

Simulation results for AWGN and TDLA channel conditions for 50MHz CBW are found in [2] based on the above simulation assumptions and 38.101-4 Table 8.2.2.2.1.1-1 and Table 8.2.2.2.2.1-1.

### Observations and proposals

According to 38.214 Table 5.2.2.1-3 CQI table2, CQI 12 is the lowest CQI index that employs 256QAM, where the corresponding SNR is 17 dB in both AWGN and TDLA30-35 channel conditions as shown in our simulation results. Given additional impairment margin and IM, this could be too high to be feasible as SNR testing point in FR2.

Observation 1: SNR testing points for 256QAM in CQI table 2 could be too high to be feasible in FR2, in both AWGN and fading conditions.

Furthermore, the reported median CQI accuracy for wideband fading scenario at 17dB SNR is quite unreliable. More than 41% of the reported CQI falls outside of the range median CQI ±1. This can be mitigated by setting the testing points at higher SNR. At 22dB SNR the CQI accuracy is quite reliable, only 12% of the CQI falls outside of the range median CQI ±1. However, 22dB SNR is very close to the upper testability limit for 50MHz FR2 scenario.

Observation 2: For fading environment there is a measurement uncertainty of upwards of 42% of CQI reported outside of the range median CQI ±1. For CQI values corresponding to 256QAM modulation order.

Proposal 1: Do not define CQI reporting requirements for FR2 DL 256QAM.

# Conclusion

In summary, we have performed UE CQI reporting simulations for FR2 DL 256QAM in accordance with simulation assumptions found in WF [1]. We have the following observation and proposal.

Observation 1: SNR testing points for 256QAM in CQI table 2 could be too high to be feasible in FR2, in both AWGN and fading conditions.

Observation 2: For fading environment there is a measurement uncertainty of upwards of 42% of CQI reported outside of the range median CQI ±1. For CQI values corresponding to 256QAM modulation order.

Proposal 1: Do not define CQI reporting requirements for FR2 DL 256QAM

# References

- R4-2012666, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom

- R4-2016094, “Simulation results for FR2 256QAM UE CQI performance requirements”, Ericsson