3GPP TSG-RAN WG4 Meeting #97-e		R4-2016095

Electronic Meeting, 2nd – 13th November, 2020

Agenda item:

Source:	Ericsson

Title:	Simulation results for FR2 DL 256QAM​ UE demodulation tests

Document for:	Information

# Introduction

In this paper, we present our simulation results for UE demodulation requirement tests for FR2 DL 256QAM, based on the WF from RAN4#96-e [1].

# Simulation results

## Simulation assumptions

The simulations are based on the following simulation assumptions.

- Tx EVM​: 3% ​

- Rank​: Rank 1

- Channel bandwidth and PRB allocation​50MHz CBW with full PRB allocation​

- Propagation condition​

  - Option 1A: TDLA30-300 ​

  - Option 2: Static channel​

- MCS​

  - MCS 20 for rank 1

- MIMO configuration​: 2Tx 2Rx ULA low​ for fading channel, 1Tx2Rx for static channel

- DM-RS configuration​ : Type 1 single symbol front loaded, 1 additional DMRS​

- PRB bundling size: 2​

- Precoding model: Random Precoding, per slot , WB granularity​

- HARQ process number​: 8​

- SCS: 120kHz

- PT-RS configuration​: per 2PRB in frequency domain, per symbol in time domain​

- Overhead for TBS determination: 6

- PDSCH mapping type​: Type A

- TDD configuration: DDDSU, S=10D:2G:2U​

- PDCCH duration​: Symbol #0 in each slot​

- PDSCH configuration​:

  - Full DL slots: Start symbol 1, Duration 13

  - Special slots: Start symbol 1, Duration 9

- SSB configuration: Periodicity 20 ms, allocated in first slot within 20ms​

- TRS configuration​: 20 ms periodicity, 2 slots, Offset 10 ms​

- HARQ RV sequence: {0, 2, 3, 1}​

- HARQ process number: 8

- Receiver assumption: MMSE-IRC

- Test metric: 70% of max TP

## Simulation results and proposals

Simulation results for TDLA30 and AWGN channel conditions for 50MHz CBW are given below based on the above simulation assumptions.

Figure 1 Throughput curve for 50MHz CBW

Table 1 SNR at 70% Throughput target

| CBC [MHz] | Channel Model | SNR @ 70% throughput mark [dB] |
| --- | --- | --- |
| 50 | TDLA30-300 | 17.3 |
|  | AWGN | 15.3 |

# Conclusion

In summary, we have performed UE demodulation simulations for FR2 DL 256QAM in accordance with simulation assumptions found in WF [1].

# References

- R4-2012666, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom