3GPP TSG-RAN WG4 Meeting#97-e                                 R4-2016132

E-meeting,2nd Nov – 13th Nov, 2020

Agenda item:	11.1

Source: 	ZTE Corporation

Title: 	Maintenance TP to TR38.921

Document for:	Approval

Introduction

In this contribution, we share some further updates for TR 38.912 based on the latest discussion and agreement.

Discussion

In section 4.2.3, it is better to mention the antenna pattern assumed for UE side keeping similar information as BS side.

In section 4.2.8, as uplink scheduled UE is assumed as 1 user for evaluation work, the corresponding parameters should be updated. In addition, 43dBm/24dBm BS max output power might cause confusion, one more Note is added;

In section 4.2.9, to clarify the UE with single isotropic antenna instead of single antenna element;

In section 5.4,  it is proposed to removed as this is duplicated information as section 4.2.7.

In section 5.2.1, remove the the column for rural area.

For other corrections are just editorial.

- Text proposal

4	Co-existence study

4.1	Co-existence simulation scenarios

Table 4.1-1 summarizes the proposed scenarios to be considered for 6.425-7.125GHz and 10.0-10.5 GHz.

Table 4.1-1: Summary of considered scenario

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Indoor hotspot |  |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Urban macro |  |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Indoor hotspot |  |
| 6 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Dense urban | Down-prioritized |
| 7 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Urban macro |  |
| 8 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Indoor hotspot |  |
| 9 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Dense urban | Down-prioritized |
| 10 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Urban macro |  |
| 11 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Indoor hotspot |  |
| 12 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Dense urban | Down-prioritized |

4.2	Co-existence simulation assumption

### 4.2.3		Antenna and beam forming pattern modelling

The BS antenna is modelled as described in subclause 8.1.1 using parameters for different BS deployments listed in subclause 8.1.2.

The UE antenna is modelled as described in subclause 8.2 using  isotropic antenna pattern.

### 4.2.7		Link level performance for 5G NR coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN		Minimum SNIR of the code set, dB
SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 4.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 4.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 4.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 4.2.7-1 are targeted for eMBB coexistence scenario.

### 4.2.8		Other simulation parameters

Table 4.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Carrier frequency | 7GHz, 10GHz | 7GHz, 10GHz | Down-prioritized |
| Channel bandwidth | 100MHz | 100MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (DL) | 98.28MHz | 98.28MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (UL) | 98.28MHz | 98.28MHz | Down-prioritized |
| The number of active UE (DL) (Note 1) | 1 | 1 | Down-prioritized |
| The number of active UE (UL) (Note 1) | 1 | 1 | Down-prioritized |
| Traffic model | Full buffer | Full buffer | Down-prioritized |
| DL power control | NO | NO | Down-prioritized |
| UL power control | YES | YES | Down-prioritized |
| BS max TX power in dBm | 24 (Note 3) | 43 (Note 3) | Down-prioritized |
| UE max TX power in dBm | 23 or 20 (Note 2) | 23 or 20 (Note 2) | Down-prioritized |
| UE min TX power in dBm | -33 | -33 | Down-prioritized |
| BS Noise figure in dB | 14 (@7GHz) 15 (@10GHz) | 6 (@7GHz) 7 (@10GHz) | Down-prioritized |
| UE Noise figure in dB | 9 | 9 | Down-prioritized |
| Handover margin | 3dB | 3dB | Down-prioritized |
| Note 1 Same as the number of BS beam(s) Note 2: 20dBm as optional case where CLx-ile should be reduced by 3dB Note 3: BS max TX power is defined per polarization. |  |  |  |

### 4.2.9		Co-existence simulation methodology

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at BS and UE with isotropic antenna.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	, where  is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	, where  is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	.

5	General parameters

5.3	Signal Bandwidth

The signal bandwidth for a 100MHz channel bandwidth signal is calculated based on the NR spectrum utilization for 30kHz SCS:

Signal bandwidth = NRB x SCS x 12

with NRB: Number of Resource block for 100 MHz channel bandwidth and 30kHz SCS, as specified in TS 38.104 [3].

6	BS parameters

6.1	Transmitter characteristics

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 6.1.5-1 below.

Table 6.1.5-1 The total Radiated power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

6.2	Receiver characteristics

### 6.2.1	Noise figure

From the TR 38.820 [5] for 7-24 GHz, the typical Noise Figure for a Wide Area BS operating at 10 GHz is 7dB (12dB for Medium Range BS and 15dB for Local Area BS).

Table 5.5.1.1-1 of TR 38.820: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

For 6.425-7.125 GHz, the typical Noise Figure for a Wide Area BS operating at 7 GHz was agreed to be 6dB (11dB for Medium Range BS and 14dB for Local Area BS).

8	Antenna characteristics

8.1	BS antenna characteristics

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain.

Table 8.1.2-1: BS array antenna parameters

| Parameter |  | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- | --- |
| Am (dB) |  | 30 | 30 | 30 |
| SLAv (dB) |  | 30 | 30 | 30 |
| 3dB (deg.) |  | 90 | 90 | 90 |
| 3dB (deg.) |  | 65 | 90 | 90 |
| GE,max (dBi) |  | 6.4 | 5.5 | 5.5 |
| LE (dB) |  | 2.0 | 2.0 | 2.0 |
| (M, N) |  | (16, 8) | (16, 8) | (8,8) |
| dh (m) |  | 0.5 | 0.5 | 0.5 |
| dv (m) |  | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) |  | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) |  | 90 to 100 | 90 to 120 | 90 to 120 |
| Total Radiated Power (dBm) |  | 46 | 46 | 37 |
| Mechanical downtilt (deg.) |  | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: TRP includes the powers from two polarizations Note 4: The vertical coverage range includes the mechanical downtilt. |  |  |  |  |