3GPP TSG-RAN WG4 Meeting # 97-e	R4-2016210

# Electronic Meeting, 2 November - 13 November 2020

Agenda item:	12.1.3.2

Source:	Keysight Technologies

Title:	On Probe Configurations and Channel model vs. OTA test system coordinate systems for FR2 MIMO OTA

Document for:	Approval

# Introduction

This contribution is proposing an alternate probe configuration as agreed during the last meeting. Additionally, it discusses the channel model and test system coordinate systems. While the change of probe configuration provides simpler definitions for 3D scanning it has consequences on the channel model definitions. This paper discusses the issue and makes a proposal for avoiding problems and redefinition of angle and polarization parameters of the channel model specified in [4].

# Probe Configuration

In the last RAN4#96-e meeting, discussions were held regarding the probe locations with respect to the positioner axes and coordinate system [1][2]. Various options were discussed but no concrete probe configuration was agreed. Instead, it was agreed to rotate the probes towards the z axis so that the DL directions perceived by the DUT are three dimensional as intended

- Probes Configuration - Rotate the probes towards the z axis direction, which probe is along z axis will be decided next meeting - CE vendors encouraged to discuss probe configurations, blocking, re-positioning, impact on channel model realization w.r.t. 38.901 and 38.827 and harmonize on a joint probe configuration proposal prior to the next RAN4 meeting - Detailed position of probes will be decided next meeting

The current probe configuration defined in [4] is shown in Figure 1 from various angles.

Figure 1: Probe Locations as defined in 38.827

The proposed probe location, which have been previously presented to CE vendors for comments, is shown in Figure 2 and tabulated in Table 1.

Figure 2: Proposed Probe Locations towards z direction.

Table 1: Proposed Probe Locations

| Probe Number | Theta/ZoA [deg] | Phi/AoA [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

The rotation angles of the probes are as follows:

- around z axis: -75o

- around y axis: -90o

The resulting probe configuration has probe #1 centred along the z direction so that this probe can be used for QoQZ validation purposes and only has one probe, i.e., probe #3, in the hemisphere with y<0, i.e., the blocking by the positioner is somewhat limited. The fully documented QoQZ validation approach in [7][8][9] would be applicable for NR FR2 MIMO OTA as the UE RF and RRM 2AoA QoQZ validation procedures are also assuming the measurement probe in the z direction.

Observation 1: The new proposed probe configuration has the probes aligned towards the z axis with probe #1 along the z direction.

Observation 2: The QoQZ validation can readily be performed with probe #1 as the fully documented UE RF and RRM 2AoA QoQZ validation procedures are also assuming the measurement probe in the z direction.

Observation 3: The blocking with this probe configuration is rather limited.

Clearly, the relative spacing between probes is the same even though the absolute difference in Theta/ZoA and Phi/AoA between probes has changed. The relative differences between probes for the two probe configurations in Figure 1 and Figure 2 is tabulated in Table 1; therefore, no additional PSP% simulations are required.

Table 2: Angular Differences between Probes

|  | Angular Difference Between Probes [deg] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Probe # | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 0.0 | 11.2 | 20.6 | 20.6 | 20.6 | 30.0 |
| 2 | 11.2 | 0.0 | 29.9 | 10.0 | 14.1 | 20.6 |
| 3 | 20.6 | 29.9 | 0.0 | 39.8 | 41.2 | 50.2 |
| 4 | 20.6 | 10.0 | 39.8 | 0.0 | 10.0 | 11.2 |
| 5 | 20.6 | 14.1 | 41.2 | 10.0 | 0.0 | 11.2 |
| 6 | 30.0 | 20.6 | 50.2 | 11.2 | 11.2 | 0.0 |

Proposal 1: Adopt the revised probe configuration in Table 1 for NR FR2 MIMO OTA

# Channel Model Coordinate System

Clustered delay line (CDL) channel models are specified by parameters such as cluster azimuth and zenith angles, and per cluster angle spreads in azimuth and zenith. Moreover, polarizations of antenna models and rays are defined by  and  unit vectors. The coordinate system has impact on these parameters.

The channel model definitions of [4] or any preceding CDL specification are not perfectly unambiguous. Namely, angle spreads of clusters are specified uniformly, regardless of cluster zenith angles. Even though we know that poles of the angular coordinate system, i.e., zenith = 0 and 180, are singularity points where all azimuth angles overlap, and the azimuth dimension disappears. Not only on poles, but also in vicinity, any angular ranges have substantially different span as compared to the equator (zenith = 90).

The first 3GPP model following the geometry based stochastic (GSCM) principle was SCM [5]. It was 2-dimensional only, i.e., only azimuth angles were defined. The sub-sequent models added elevation dimension to parameterization. Always the silent assumption was, that the propagation paths are confined around horizon just as in real life. No motivation for specifying model parameters considering poles was identified. For historical reasons, the channel model definitions are targeted for equator of  , angular coordinate system. Furthermore, cluster angle spreads have been inspired by physical objects that have spanned certain angular ranges around horizon in propagation measurements.

Polarization parameters of channel models reflect man-made (constructed) environments with mainly vertical and horizontal surfaces. The original parameters of GSCM models are obtained from propagation measurements in such environments. Thus,  and  polarization originally mean vertically and horizontally polarized components in, e.g. urban spot with vertical walls and horizontal ground. For mathematical completeness they are defined by unit vectors of azimuth and elevation (or zenith) increment direction.

Now if the centre of gravity of channel models is rotated to zenith, the original meaning of angle spreads is distorted. Moreover, the polarization definitions lose their original meaning around the singularity point. E.g. if there is a cluster with 20 rays around the pole, the polarization vectors in channel model definitions point to 20 different directions.

We propose to keep separate coordinate systems for the channel models and for the DUT test orientation definition in the OTA chamber. Any channel model discussions, definitions, validation activities, angle spread calculations, etc. are performed using the current angle definitions and coordinate system of the channel model. The coordinate system of OTA test system can be changed, but it must not influence any channel model related issues. The relation of two coordinate axes are specified in Figure 3. Channel model coordinate axes xCM, yCM, and zCM correspond to the OTA test system coordinate axes z, y, and -x, respectively, i.e.

x=-zCMy=yCM   z=xCM

Figure 3. New OTA test system coordinate axes (x, y, z) and a proposal for corresponding channel model coordinate axes (xCM, yCM, zCM).

Proposal 2: Define channel model coordinate axes xCM, yCM, and zCM which correspond to the OTA test system coordinate axes z, y, and -x, respectively

# Conclusion

This contribution made the following observations and proposals

Observation 1: The new proposed probe configuration has the probes aligned towards the z axis with probe #1 along the z direction.

Observation 2: The QoQZ validation can readily be performed with probe #1 as the fully documented UE RF and RRM 2AoA QoQZ validation procedures are also assuming the measurement probe in the z direction.

Observation 3: The blocking with this probe configuration is rather limited.

Proposal 1: Adopt the revised probe configuration in Table 1 for NR FR2 MIMO OTA

Proposal 2: Define channel model coordinate axes xCM, yCM, and zCM which correspond to the OTA test system coordinate axes z, y, and -x, respectively

# References

- R4-2012743, Email discussion summary for [96e][329] NR_MIMO_OTA, CAICT, 3GPP TSG-RAN WG4 Meeting # 97-e, August 2020

- R4-2010774, 3D-MPAC implementation of measurement probe and positioner, Oppo, 3GPP TSG-RAN WG4 Meeting # 97-e, August 2020

- R4-2012707, WF on NR MIMO OTA, vivo, CAICT, Spirent, 3GPP TSG-RAN WG4 Meeting # 97-e, August 2020

- TR 38.827, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE); Release 16”, V2.0.0, June 2020.

- 3GPP TR 25.996, “Spatial channel model for Multiple Input Multiple Output (MIMO) simulations”

- TR38.810, Study on test methods, V16.5.0 (2019-12)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.40 (2020-06)

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment, V16.4.0 (2020-06)