3GPP RAN WG4 Meeting #96-e		R4-2016297

Electronic Meeting, 02nd – 13th November 2020

Agenda item:	17.1

Source:	Apple

Title:	CA/DC Band configurations notations and usage in 3GPP

Document for:	Approval

# 1	Introduction

Since several meetings RAN4 is discussing a simplification of the band combinations tables in the 38.101 specs. Meanwhile there are about 7799 band combinations in these tables and it is very difficult to keep track of all specified combinations. As there have been many bugs in the specifications as well as the combination request sheets and the WIDs, this paper summarizes the notation and the procedures for adding band combinations to specs and WIDs as a reference for future RAN4 work to improve the quality of the specs and WIDs.

Also MCC is working on a solution to simplify the spec tables using .CSV files, which also has a prerequisite to use the correct notations.

All these notations and rules have been already discussed and agreed previously and are just summarized here.

# 2	Notation of band combinations

The correct notation of the band combinations is a real problem in the 38.101 specs as well as the combination request lists by the operators and the basket WIDs. If there are bugs in the notation, it is unclear what combination is really meant in the request or spec. Then the combination may be misinterpreted, not taken into account when implementing combinations in BS or UEs or even removed from specs or requests. Also automated processing of the tables will become difficult with too many bugs.

Generally the notation of band combinations starts with the type of the configuration (mainly CA or DC), followed by one list (either LTE or NR) or two lists (first LTE, then NR) of bands with bandwidths.

#### 2.1 Types of band combinations

We have the following types of band combinations:

Carrier Aggregation: Starts with “CA_” as the first three characters. Then either a list of LTE or NR carriers is following, where the carriers or bands are always separated by “-“. LTE and NR carriers cannot be combined, that would be a DC combination. Examples:

- CA_1A-2A (LTE)

- CA_n1A-n2A (NR)

- Examples for wrong notations: CA_1A_2A (“_” instead of “-“ between the carriers/bands), CA-1A-2A (no “_” but “-“ after “CA”, CA_1A_n2A (this would need to be a DC combination)

Dual Connectivity: Starts with “DC_” as the first three characters, then for EN-DC configurations followed by the list of LTE carriers, a “_” as separation between the LTE and NR carriers and then the list of NR carriers. There are exceptions for SUL EN-DC combinations, which separate the LTE and NR carriers by “_SUL_” instead of “_”, and contiguous intra-band EN-DC combinations using “(n)” instead of “_” and the “n” of the first NR band in the list, as discussed in the next chapter. DC configurations within LTE or NR just list the carriers after “DC_”. For EN_DC combinations for V2X the “DC_” at the beginning is replaced by V2X_”, even if it is still a EN-DC combination. Examples:

- DC_1A_n2A (EN-DC)

- DC_1A-2A (LTE-DC)

- DC_n1A-n2A (NR-DC)

- DC_(n)1AA (EN-DC with contiguous intra-band LTE and NR carriers)

- DC_1A-(n)2AA (EN-DC with one LTE carrier followed by contiguous intra-band LTE and NR carriers)

- Examples for wrong notations: DC_1A-n2A (“-” instead of “_“ between the LTE and NR carriers/bands for EN-DC combinations), DC-1A-2A (no “_” but “-“ after “DC”, DC_n1A_n2A (“_” instead of “-“ between the NR carriers/bands for NR-DC combinations)

Supplementary UL: NR SA configurations start with SUL_ as the first four characters, as it is only NR without LTE, if it is within an EN-DC combination there is a _SUL_ between the LTE and the NR part instead of the “SUL_” at the beginning, using the usual “DC_” as the first characters. Examples:

- SUL_n2A-n80A (n80 being the SUL band)

- DC_1A_SUL_n2A-n80A (n80 being the SUL band)

In summary we have these types and notations:

- CA_ …: A Carrier Aggregation configuration followed by the list of either LTE or NR carriers

- DC_ …: A Dual Connectivity configuration followed by the list of either LTE carriers for LTE-DC or NR carriers for NR-DC or for EN-DC first LTE carriers, then “_”  and the NR carriers. In case of a DC combination for V2X, the “DC_” is replaced with “V2X_”.

- SUL_ …: A Carrier Aggregation configuration including one SUL band followed by the list of NR carriers, one of them being a SUL carrier. In case of a DC configuration with SUL, the “SUL_” is shifted behind the “_” separating the LTE and NR carriers and the configuration starts with “DC_” as usual for DC configurations

#### 2.2 Notation of lists of bands and bandwidths within a configuration

#### 2.2.1 Band numbers

A list of LTE or NR carriers within a CA or DC configuration is a either a single or multiple LTE or NR carriers. The simplest one is just a single carrier. It consists of the band number followed by the bandwidth class, which is “A” for a single carrier. For LTE the band number is just the number of the band, for NR carriers the numerical part of the band notation is preceded by a “n”, indicating this is a NR band, not a LTE band. NR bands above n256 are FR2 bands, below n256 are FR1 bands. The band number is always followed by the bandwidth class, which can be quite complicated for NR combinations with intra-band CA. Bandwidth classes other than “A” indicate multiple carriers in that band. In the list there can be multiple entries for inter-band CA configurations (in LTE also intra-band non-contiguous CA), which are always separated by “-“. The band numbers are sorted in increasing numbers. LTE and NR bands in the same frequency range usually have the same band number. Examples:

- Notation of a single LTE carrier: 1A, 2A, 3A etc.

- Notation of a single NR carrier: n1A, n2A, n3A etc.

- List of multiple LTE carriers on different bands: 1A-2A-3A

- List of multiple NR carriers on different bands: n1A-n2A-n3A

#### 2.2.2 Bandwidth classes

2.2.2.1 Bandwidth classes for LTE

An entry within the list of carriers always starts with the band number followed by the bandwidth class. In LTE the bandwidth classes (if not “A”) mean this is a contiguous CA configuration with multiple carriers. They are specified in table 5.6A-1 in 36.101 and can range from “A” for a single carrier  up to F for 5 carriers. BW class I is specified for 8 carriers, but is not used. Non-contiguous CA combinations are just listing multiple single carriers separated by “-“. Examples:

- CA_1B: Two contiguously aggregated LTE carriers with 20MHz or less in band 1.

- CA_2F: Five contiguously aggregated LTE carriers with up to 100MHz in band 2.

- CA_3A-3A: Two non-contiguously aggregated LTE carriers in band 3

- CA_4A-4E: A single carrier followed by a gap and then followed by four contiguously aggregated carriers with up to 80MHz

- A single carrier is no CA configuration as there is nothing aggregated, so there is no CA_5A, this is just 5A.

2.2.2.2 Bandwidth classes for NR

NR bandwidth classes are much more complicated. Also here an entry within the list of carriers always starts with the band number followed by the bandwidth class. But in NR the bandwidth class includes contiguous and non-contiguous CA and a mixture of contiguous and non-contiguous CA. For contiguous CA the bandwidth classes are specified similar to LTE, but separate for FR1 and FR2. For FR1 contiguous CA BW classes are specified in table 5.3A.5-1 in 38.101-1 ranging from A to O, for FR2 in 5.3A.4-1 in 38.101-2 ranging from A to Q.

A special kind or BW class specification is when there are intra-band contiguous LTE and NR carriers within a DC combination like DC_(n)1AA. In this case the LTE and NR carriers within that band are combined to a single entry of the list of carriers starting with (n) indicating that it can be “n” for the NR carrier, or no “n” for the LTE carrier. This is followed by the numerical value of the band (here “1”) and then the contiguous BW class for the LTE part and the contiguous BW class for the NR part. So DC_(n)1AA means that there is a single carrier for LTE and a single carrier for NR side-by-side contiguously aggregated in band 1. This can be extended by more contiguous carriers on the LTE or NR side or both, for example DC_(n)41DA means three contiguous carriers for LTE besides a single carrier for NR. This can be extended by other LTE carriers in front of  the combination of carriers with (n) or with other NR carriers behind the (n) part, for example DC_1A-(n)2AA or DC_(n)2AA-n3A. The (n) part is considered as the last TLE combination in the list or the first NR combination in the list, therefore adding it with a “-“ instead of a “_”. However, we currently have some kind of mix-up in the spec, as both versions appear there. This will need to be corrected with a CR to get it consistent in the spec.

However, the BW class part of a NR configuration also includes non-contiguous intra-band CA. For a combination containing any non-contiguous CA, i.e. a gap between any aggregated carriers, each block of single or contiguously aggregated carriers is called a sub-block, where a sub-block can also consist of the contiguously aggregated carriers as stated above. While in LTE single non-contiguously aggregated carriers are just duplicated like CA_1A-1A, in NR the number of non-contiguous carriers of a BW class is counted and put in brackets with the number of subblocks of this type preceding the bandwidth class. Therefore a configuration with two non-contiguous carriers will have a BW class (2A) in NR, so the combination will be named CA_n1(2A), meaning there are two non-contiguous carriers with BW class A in band n1.

However, there can also be the combination of contiguous and non-contiguous intra-band CA in NR. In. this case the sub-blocks of each BW class are separately counted and added within the brackets. For example if there are in a n260 FR2 CA combination two sub-blocks of BW class “A” (single carriers), three of BW class “G” (two contiguous carriers up to 100MHz) and one of BW class O (two carriers with 50 or 100MHz), the full combination will be named CA_n260(2A-3G-Q), having 6 sub-blocks with in total 10 carriers. Examples:

- CA_n1B: Two contiguously aggregated NR carriers with 100MHz or less in band n1 (FR1).

- CA_n2D: Three contiguously aggregated NR carriers with up to 300MHz in band n2.

- CA_n3(2A): Two non-contiguously aggregated NR carriers in band n3

- CA_n260G: Two contiguously aggregated NR carriers with 150 or 200MHz in band n260 (FR2).

- CA_n260M: Eight contiguously aggregated NR carriers with 750 or 800MHz in band n260 (FR2).

- CA_n260(2A): Two non-contiguously aggregated NR carriers in band n260 with up to 800MHz (2x400MHz)

- CA_n260(A-M): A single carrier followed by a gap and then followed by eight contiguously aggregated carriers with up to 100MHz each

- CA_n260(2A-3G-Q): Two single carriers up to 400MNHz each, three sub-blocks with two carriers each of 150 or 200MHz per sub-block followed by another subblock with two carriers of 50 or 100MHz each

#### 2.3 Rules to be used for the notation of CA or DC configurations

Besides what has been discussed above, there need to be some rules for generating the configuration notations:

- Each configuration needs to start with “CA_”, “DC_”, “SUL_” or “V2X_

- DC combinations include a list of LTE carriers first, followed by the list of NR carriers

- Entries within a list of either LTE carriers or NR carriers need to be separated by “-“, not “_”

- The list of LTE carriers and the list of NR carriers within an EN_DC combination need to be separated by “_”, for contiguous intra-band EN-DC the two lists are connected with the (n)xxAA like notation, not “_” (xx is the band number). In specific cases “_SUL_” connects the two lists.

- Contiguous LTE+NR intra-band carriers within a DC combination are using the notation (n)xxAA (xx is the band number)

- No other characters than “A” to “Z”, “0” to “9”, “(“, “)”, “-“ “_” and “n” are allowed within the notation, especially no spaces “ “, “/”, “.”, “.”, LineFeed, CR, other special characters

- Entries within the list of carriers need to be sorted in numerical order, i.e. first band n1, then n2, then n3, then n260, i.e. CA_1A-2A, not CA_2A-1A, but LTE and NR combinations are separately sorted, i.e. DC_2A_n1A, entries with (n) are always between the LTE and NR lists.

- Bandwidth notations are either a single character according to the BW class lists of contiguously aggregated carriers, two of these characters in case of  combinations with (n) or for NR non-contiguous intra-band combinations specific expressions listing multiple carriers within “()”

- Within the “()” of non-contiguous NR combinations there will only be BW class letters for the BW class of contiguous sub-blocks preceded by a number indicating the number of sub-blocks of this BW class, if there are multiple different BW classes they are listed in ascending BW class order separated by “-“

Proposal 1:	RAN4 Confirms the usage of the above rules in chapter 2.3 for the notation of CA or DC configurations

Here are some examples of correct notations:

- DC_1A-2A_n260(A-M)

- DC_1A-2A-2A-2A_n3(3A)

- DC_1A-(n)2AA-n3A

- DC_1A-2A-3A-4A-5A_n6A-n260(2A-3G-Q)

- Some incorrect examples we have seen: DC_1A-2A_n3A(3A) (no “A” before the bracket); DC_2A-1A_n3(3A) (wrong sort order of LTE bands); DC_1A- 2A_n260(A-M) (a “ “ (space) between the “-“ and the “2”; DC_1A-2A_n260A/G/H/I/J/K/L/M (no “/” allowed within a configuration, multiple configurations not allowed within the notation, use separate configuration notations for each configuration)

# 3	Usage of band combination notation in the specs

Currently we use this notation for the CA/DC configurations specified in 36.101 for LTE and 38.101 for NR. However, we have seen multiple bugs within the specs, which require to remind everyone working with the specs, how to handle the CA/DC configurations in the -101 specs. Here are some general rules how to implement these band combinations within the CA/DC configurations within the tables in clause 5.5:

- Each cell of the configuration table should contain only one combination of bands in the first column with the exception that combinations having the same bands but different intra-band contiguous BW classes can be listed in the same cell. Also all non-contiguous combinations can be listed in one cell, but separated from the contiguous combinations in another cell.

- In the UL column there shall only be UL configurations that belong to the configurations in the first column. Unfortunately this means that in the UL column there can be higher order configurations than some of the configurations in the first column, however, they cannot be used with such a lower order combination. This was agreed some time ago as a “table simplification”, but creates some hassle as there are UL combinations listed that cannot be supported with the DLs.

- Multiple configurations with different bandwidth classes shall be separated by pressing the return key, you will see the sign at the end of the line if you activated the button to view these special characters, no other special characters to separate configuration shall be used

- There shall be no special characters not belonging to the combinations in any configuration cell, no spaces “ “, “/”, “.”, “.”, or any other special characters

- If there are notes for a specific configuration, the note shall be using superscript font and added at the end of the configuration list within a cell, not anywhere within the configuration or separated with any other characters, multiple notes shall be separated just by a comma, all in superscript.

Proposal 2:	RAN4 confirms the usage of the above rules in chapter 3 for applying the notation of CA or DC configurations within the 36.101 and 38.101 specifications

# 4	Usage of band combination notations in the request sheets and WIDs

As agreed some years ago, we use the configurations notation discussed above also for the CA/DC configurations in the columns for the CA/DC configurations for DL and UL in the request sheets and the combination tables within the WIDs. However, we have seen multiple bugs within the specs, which require to remind everyone working with the request sheets and WIDs how to handle the CA/DC configurations in the WIDs. Here are some general rules how to implement these band combinations within the CA/DC configurations lists within the excel tables in the request sheets and WIDs 5.5:

- Each cell of the CA or DC configuration column in the Excel tables shall contain only one single CA/DC configuration using the notation of the configurations as discussed above

- Similar CA or DC configurations with different bandwidth classes shall use another row in the same column of the table.

- The UL configurations column shall only contain the UL configurations valid for the CA/DC configuration in the same row, if there are multiple valid UL configurations they can be listed one after the other separated with “, “ (a comma followed by a space), but they can also be using a single completely filled row for each of the valid UL configurations

- There shall be no merged cells in the table

- The WI rapporteur checks if the notation of the CA/DC configurations is correct and if not returns the request to the requestor. Incorrect requests should not be added to the table in the WID

Proposal 3:	RAN4 confirms the usage of the above rules in chapter 4 for applying the notation of CA or DC configurations within CA/DC request sheets and the WIDs

# 5	Conclusions

We draw the following conclusions and have these proposals:

The rules above just summarize in more detail, what already has been agreed in RAN4. Therefore we propose the following:

Proposal 1:	RAN4 Confirms the usage of the above rules in chapter 2.3 for the notation of CA or DC configurations

Proposal 2:	RAN4 confirms the usage of the above rules in chapter 3 for applying the notation of CA or DC configurations within the 36.101 and 38.101 specifications

Proposal 3:	RAN4 confirms the usage of the above rules in chapter 4 for applying the notation of CA or DC configurations within CA/DC request sheets and the WIDs