3GPP TSG-RAN WG4 Meeting #97-e	R4-2016331

Electronic Meeting, 02 November – 13 November 2020

Source:	Ericsson, Verizon

Title:	TP to TR 38.717-01-01: Addition of CA_n77(3A) and CA_n77(4A)

Agenda item:	10.1.2

Document for:	Approval

# Background

This text proposal for TR 38.717-01-01 to add Addition of CA_n77(3A) and CA_n77(4A) configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

### 6.x.1	Channel bandwidths per operating band for CA_n77(3A) and CA_n77(4A)

Table 6.x.1-1: Supported bandwidth combinations for CA_2DL_n48(3A)_1UL _n48A

| NR CA configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n77(3A) | - | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 300 | 0 |

Table 6.x.1-2: Supported bandwidth combinations for CA_2DL_n48(4A)_1UL _n48A

| NR CA configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(4A) | - | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 400 | 0 |

### 6.x.2	Co-existence studies

There are no co-existence issues for this combination.

### 6.x.3	REFSENS

There are no REFSENS exceptions for this combination.

##### ---End of changes---

# Reference

[1]		RP-201571, “Revised WID: Rel-17 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)”, Ericsson