3GPP TSG-RAN WG4 Meeting #97-e 	R4-2016397

# Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1378 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CSSF for positioning measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incomplete requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Missing requirements are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	Intra-frequency measurement object with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	Intra-frequency measurement object with measurement gap in clause 9.2.6.

-	Inter-frequency measurement with no measurement gap in clause 9.3.9, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16.

-	Inter-frequency measurement object with measurement gap in clause 9.3.4.

-	E-UTRA Inter-RAT measurement object in clauses 9.4.2 and 9.4.3.

-	NR PRS-based measurements for positioning in clause 9.9.

-	E-UTRA Inter-RAT RSTD and E-CID measurements in clauses 9.4.4 and 9.4.5.

-	NR Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.4).

-	E-UTRAN Inter-frequency measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.3) and by the E-UTRAN PSCell (TS 36.133 [15] clause 8.19.3).

-	E-UTRAN Inter-frequency RSTD measurement configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.15).

-	UTRA Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.5 to 8.17.12).

-	GSM Inter-RAT measurements configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.13 and 8.17.14).

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

##### 9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

The scaling value CSSFwithin_gap,i below has been derived without considering GSM inter-RAT carriers.

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR carriers, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra,i,j: Number of intra-frequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency measurement objects or NR inter-RAT measurement objects configured by E-UTRA PCell, EUTRA inter-frequency measurement objects configured by E-UTRA PCell, UTRA inter-RAT measurement objects configured by E-UTRA PCell which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

##### 9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1,

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

Table 9.1.5.2.2-1: PRS configurations for long-periodicity NR measurements for positioning

| PRS periodicity Tprs (ms) | DL-PRS-MutingPattern configuration |
| --- | --- |
| >160 | With or without muting |
| 160 | With muting |

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

-	An inter-frequency SFTD measurement object, if to be measured with measurement gaps, is a candidate to be measured in all measurement gaps.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

-	Mintra,i,j: Number of intra-frequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT objects and frequency layers for PRS-based positioning measurements  which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object or frequency layers for PRS-based positioning measurements, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in clause 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in clause 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to clause 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in clauses 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, and 9.4.2.3 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

##### 9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicty measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j: Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured inter-frequency and inter-RAT measuerement objects and frequency layers for PRS-based positioning measurements is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR positioning measurement corresponding to a positioning frequency layer with configurations of maximum DL-PRS-Periodicity and maximum bitmap size of DL-PRS-MutingPattern among its PRS resource sets as in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects and frequency layers for PRS-based positioning measurements belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

When NR PRS-based measurements for positioning are configured on one or more frequency layers within measurement gaps, the carrier specific scaling factor for a target measurement on a frequency layer with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If the PRS-based measurement on frequency layer i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1,

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all meausrement gaps.

-	An inter-frequency SFTD measurement object, if to be measured with measurement gaps, is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

-	Mintra,i,j: Number of intra-frequency measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT objects and frequency layers for PRS-based positioning measurements  which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object or frequency layers for PRS-based positioning measurements, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in clause 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in clause 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to clause 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in clauses 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, and 9.4.2.3 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

##### 9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

When NR PRS-based measurements for positioning are configured on one or more frequency layers within measurement gaps, the carrier specific scaling factor for a target measurement on a frequency layer with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If the PRS-based measurement on frequency layer i refers to a long-periodicty measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR measurement for positioning based on PRS configurations in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j: Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured inter-frequency and inter-RAT measuerement objects and frequency layers for PRS-based positioning measurements is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

When NR PRS-based measurements for positioning are configured on one or more frequency layers within measurement gaps, the carrier specific scaling factor for a target measurement on a frequency layer with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If the PRS-based measurement on frequency layer i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR positioning measurement corresponding to a positioning frequency layer with configurations of maximum DL-PRS-Periodicity and maximum bitmap size of DL-PRS-MutingPattern among its PRS resource sets as in Table 9.1.5.2.2-1

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured within the gap j.

-	An NR PRS-based measurement is a candidate to be measured in a gap if at least one PRS symbol is covered by the MGL excluding RF swtiching time.

-	An NR measurement object is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects and frequency layers for PRS-based positioning measurements belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects and frequency layers for PRS-based positioning measurements which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects and frequency layers for PRS-based positioning measurements is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.