3GPP TSG-RAN WG4 Meeting#97-e	R4- 2016462

Online, 2nd Nov – 13th Nov, 2020

Source: 	Qualcomm Incorporated

Title: 	Views on URLLC High BLER Demodulation Test Cases

Agenda item:	7.8.1.2.1

Document for:	Discussion

- Introduction

In the last meeting, WF [1] listed different options for testing URLLC features. In this paper, we discuss our views on test cases for URLLC features.

- High Reliability Requirements

In WF [1], it was agreed to choose the MCS for the fading tests for high reliability requirements based on evaluation results. However, we observed below results from our simulations for FDD 4Rx with 2 slot aggregation.

| Case | SNR in dB at 1e-2 BLER | Impairment SNR in dB | Requirement SNR in dB |
| --- | --- | --- | --- |
| FDD 4Rx, MCS 19 | -3.70 | -1.7 | -1.2 |
| FDD 4Rx, MCS 16 | -5.16 | -3.16 | -2.66 |
| FDD 4Rx, MCS 13 | -7.47 | -5.47 | -4.97 |

Based on above results, it can be noticed that requirement SNR is greater than -4dB for MCS 16 and 19. Therefore, we propose the following.

Proposal 1: Define high reliability high BLER tests with MCS 16 or 19 in Low SE MCS Table.

- DL Pre-emption Requirements

Based on our simulation results for 10% pre-emption probability with fixed scheduling, we see 1.4dB and 1.6dB loss for MCS4 and MCE13 respectively, when UE doesn’t do correct pre-emption processing. In our opinion, MCS4 has big enough loss to define the requirements for DL pre-emption. Therefore, we propose the following.

Proposal 2: Define DL pre-emption test for eMBB with 10% pre-emption probability, fixed scheduling and MCS 4.

- Conclusions

This paper discusses the testability of certain URLLC features and test cases. Following has been observed and proposed:

Proposal 1: Define high reliability high BLER tests with MCS 16 or 19 in Low SE MCS Table.

Proposal 2: Define DL pre-emption test for eMBB with 10% pre-emption probability, fixed scheduling and MCS 4.

# References

[1] R4-2012648, “Way forward for NR UE URLLC performance requirements”, 3GPP TSG RAN WG4 Meeting #96-e, Online, Aug 17 - Aug 28, 2020.