3GPP TSG-RAN4 WG4 Meeting # 96-e	R4-2016529

# Electronic meeting, 17 - 28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	On new FWA UE RF requirement for FR2

Agenda Item:	10.24.1

Document for:	Approval

Introduction

Remaining issues for the FR2 new FWA UE RF requirement can be summarized as below:

- Peak EIRP/EIS

- UE capability on power class

- Beam correspondence and MBR requirement

- MPRnarrow requirement

This paper provides further proposal on FR2 new FWA UE RF requirement in Rel-17.

Discussion

Peak EIRP and EIS

Uplink limited system:FR2

Considering high frequency of FR2 and high pathloss/penetration loss, FR2 network coverage is limited. For handheld type UE, the output power is limited by antenna array size, PA number, power consumption and form factor. Hence FR2 Handheld UE is highly limited by uplink power, it makes hot-spot coverage for FR2 handheld more popular.

Meanwhile, such limitation triggers the FWA application into commercial usage on FR2. It has few limitation on power consumption, form factor and antenna array scale. In Rel-15, FWA UE PC1 is specified with minimum EIRP 40dBm which requires for more than 16 elements per polarization to reach. Such FR2 high power application can introduce 5G signal outdoor into building, and enhance on the uplink coverage distance. By professional install, the UL coverage radius can be more than 2km.

Under regulatory requirement with max TRP/EIRP 23dBm/43dBm, Rel-17 New FWA UE targets for enhance FR2 uplink coverage under such condition.

To have evaluation on the performance of UL coverage, we provide UL and DL budget for each modulation order in Table 1. The comparison on UL and DL coverage can be seen clearly.

Table 1-1. UL coverage performance evaluation parameters for 28GHz

| UE Tx Parameter(28GHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Modulation | QPSK |  | 16QAM |  | 64QAM |  |
| Bandwidth | 200MHz |  |  |  |  |  |
| UE min peak EIRP 1 | 32dBm[1] |  |  |  |  |  |
| UE min peak EIRP 2 | 28.7dBm[2] |  |  |  |  |  |
| UE min peak EIRP 3 | 26dBm[3] |  |  |  |  |  |
| MPR(inner allocation for CP-OFDM) | 3.5dB |  | 5dB |  | 7.5dB |  |
| UE antenna height(hUT) | 1.5m |  |  |  |  |  |
| gNB Rx parameter |  |  |  |  |  |  |
| Element number | 256 |  |  |  |  |  |
| NF | 9dB |  |  |  |  |  |
| Thermal noise floor | -89dBm |  |  |  |  |  |
| Element gain | 8dBi |  |  |  |  |  |
| Array gain | 24dB |  |  |  |  |  |
| Total antenna Gain | 32dB |  |  |  |  |  |
| Rx total noise floor | -80dBm |  |  |  |  |  |
| BS height(hBS) | 10m |  |  |  |  |  |
| Demod threshold | QPSK |  |  | 16QAM |  | 64QAM |
|  | 2dB |  |  | 10dB |  | 15dB |
| Max path loss | 135dB |  |  | 125dB |  | 118dB |
| Path Loss for Link-Umi-LOS |  |  |  |  |  |  |
| Shadow fading std | 4dB |  |  |  |  |  |
| d’BP | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL1 | 32.4+21log(d3D)+20log(fc) |  |  |  |  |  |
| PL2 | 32.4+21log(d3D)+20log(fc)-9.5log( (d’BP^2)+ (hBS-hUT)^2) |  |  |  |  |  |
| PL |  |  |  |  |  |  |
| Path Loss for Link-Umi-NLOS |  |  |  |  |  |  |
| Shadow fading std | 7.8dB |  |  |  |  |  |
| d’BP | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL | for |  |  |  |  |  |
| Other penetration loss assumption |  |  |  |  |  |  |
| Glass |  | 10dB(single film) or 20dB(bi-layer film) |  |  |  |  |
| Foliage |  | 10dB |  |  |  |  |

The relation among d3D, d2D, hUT and hBS can be provided as below from TR 38.901:

From the budget calculation based on above assumptions, we get the following UL coverage radius for LOS and NLOS respectively:

Table 1-2 UL coverage radius for different UE Tx power

|  | Umi-LOS(single film glass) |  |  | Umi-NLOS(single film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE min peak EIRP | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| 32dBm | 222m | 78m | 33m | 32m | 15.6m | 6.8m |
| 28.7dBm | 142m | 49m | 20m | 24m | 10.7m | N/A |
| 26dBm | 114.6m | 40m | 15.5m | 20.5m | 8.5m | N/A |

|  | Umi-LOS(bi-film glass) |  |  | Umi-NLOS(bi-film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE min peak EIRP | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| 32dBm | 73.8m | 24.8m | 7.78m | 15m | 3.6m | N/A |
| 28.7dBm | 47m | 14.5m | N/A | 10.14m | N/A | N/A |
| 26dBm | 37.4m | 10.5m | N/A | 7.9m | N/A | N/A |

Table 2-1. DL coverage performance evaluation parameters for 28GHz

| gNB Tx Parameter(28GHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Element number | 256 |  |  |  |  |  |  |
| Element gain | 8dBi |  |  |  |  |  |  |
| Total array gain | 32dB |  |  |  |  |  |  |
| Total EIRP | QPSK |  |  | 16QAM |  |  | 64QAM |
|  | 64dBm |  |  | 63dBm |  |  | 61dBm |
| UE Rx parametere |  |  |  |  |  |  |  |
| UE total noise floor | -82dBm |  |  |  |  |  |  |
| UE max peak EIS 1 | -93.3dBm@50MHz [1] |  |  |  |  |  |  |
| Demod threshold | QPSK |  |  |  | 16QAM | 64QAM |  |
|  | 2dB |  |  |  | 10dB | 15dB |  |
| Path Loss for Link-Umi-LOS |  |  |  |  |  |  |  |
| Shadow fading std |  | 4dB |  |  |  |  |  |
| d’BP |  | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL1 |  | 32.4+21log(d3D)+20log(fc) |  |  |  |  |  |
| PL2 |  | 32.4+21log(d3D)+20log(fc)-9.5log( (d’BP^2)+ (hBS-hUT)^2) |  |  |  |  |  |
| PL |  |  |  |  |  |  |  |
| Path Loss for Link-Umi-NLOS |  |  |  |  |  |  |  |
| Shadow fading std |  | 7.8dB |  |  |  |  |  |
| d’BP |  | 2π hBS hUT fc/c=1680 |  |  |  |  |  |
| PL |  | for |  |  |  |  |  |
| Path Loss |  |  |  |  |  |  |  |
| Other penetration loss assumption |  |  |  |  |  |  |  |
| Glass |  |  | 10dB(single film) or 20dB(bi-layer film) |  |  |  |  |
| Foliage |  |  | 10dB |  |  |  |  |

Table 2-2 DL coverage radius for different UE Tx power

|  | Umi-LOS(single film glass) |  |  | Umi-NLOS(single film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE max peak EIS | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| -93.3dBm | 1.9km | 805m | 373m | 127m | 70m | 44m |

|  | Umi-LOS(bi-film glass) |  |  | Umi-NLOS(bi-film glass) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE max peak EIS | QPSK | 16QAM | 64QAM | QPSK | 16QAM | 64QAM |
| -93.3dBm | 721m | 269m | 124m | 65m | 36m | 21.8m |

From comparison for FR2 UL and DL coverage, we found that it is highly uplink limited system. It makes high Tx power crucial for this network. It is not exaggerated to say: 26~28.7Bm Tx power is far from the basic requirement that the system requires for.

When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission, it will have big impact on throughput and user experience. Without 64QAM indoor experience, NR do not have evident gain compared with fiber cable into building.

Observation 1: When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission on 28GHz.

Min peak EIRP

As we provide the power budget and analysis in [1], our initial proposal on min peak EIRP is 32dBm with 16 antenna elements per polarization.

In the previous meetings, companies provide min peak EIRP summarized as below:

| Contribution No. | Antenna elements assumption | Min peak EIRP |
| --- | --- | --- |
| [1] | 16 | 32dBm |
| [2] | 16 | 28.7/28dBm |
| [3] | 8 | 26dBm |
| [4] | 8 | 28.3dBm |
| [5] | 8 | 27.4dBm |

For 16 antenna elements assumption, the average value is around 30.5dBm. Finally this value is acceptable to most companies.

Considering the cost of UE design, some vendors prefer 8 antenna elements assumption. We fully understand on this choice, and admit that 32dBm is not easy for UE with 8 element to reach with 23dBm TRP upper limit. However, we think 30.5dBm for UE with 8 antenna element is possible from implementation perspective, and such evaluation already considers production variants. However, UE need to do some calibrate work on tolerance control. We provide power budget for 8 antenna elements as below:

Table 1: Power budget for min peak EIRP with 8 antenna element

| Parameters | unit | Nominal | Worst |
| --- | --- | --- | --- |
| Band |  | n257, n258 |  |
| Pout per element | dBm | 14dBm |  |
| # of elements in an array |  | 8 |  |
| Total conducted power per polarization | dBm | 23 |  |
| Avg antenna element gain | dBi | 4 |  |
| Antenna rolloff loss versus frequency | dB | -1 |  |
| Realized antenna array gain | dBi | 12 |  |
| Polarization gain | dB | 2.8 |  |
| Mismatch and transmission line loss including load pull | dB | -2 |  |
| Beam forming loss(phase shifter and amplitude error) | dB | -0.5 |  |
| Finite beam table | dB | -0.1 |  |
| Beam forming loss(one beam table fits all) | dB | -0.25 |  |
| Form factor integration losses | dB | -2 |  |
| Total implementation loss(nominal) | dB | -4.85 |  |
| Total implementation loss(worst case) | dB |  | -7 |
| Total implementation loss(best case) | dB |  |  |
| Max TRP, should≤23dBm |  | 21.8dBm |  |
| Peak EIRP(nominal) | dBm | 32.95dBm |  |
| Tolerance(+/-) | dB |  |  |
| Peak EIRP(Minimum) | dBm | 30.8 |  |

Observation 2: 30.5dBm min peak EIRP could be reached for both UEs with 16 and 8 antenna elements.

Considering companies may have different design consideration, we can compromise to accept following min peak EIRP:

Table 2: UE minimum peak EIRP for power class 5

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30.4 |
| n258 | 30.7 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

Proposal 1: define min peak EIRP for FR2 new FWA UE as in table 2.

Max peak EIS

Similar as for min peak EIRP, max peak EIS evaluation are also divided into 2 groups: 16 elements and 8 elements.

During the discussion, 3 options are raised:

- Take the average value for 16 elements assumption:

- -93.75dBm for n258 (from -94.2/-93.3dBm)

- -93.4dBm for n257 (from -93.5/-93.3dBm)

- Take the average value for 8 elements assumption: -91.9dBm (from -90.8/-92.9/-92 dBm)

- Take all values as average: around -92.6dBm

Max peak EIS evaluation is similar as for min peak EIRP, -93.5dBm is also workable for 8 antenna elements. However it needs more calibration work.

Proposal 2: define min peak EIRP for FR2 new FWA UE as in table 3.

Table 3: Reference sensitivity for power class 5

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -93.4 | -90.4 | -87.4 | -84.4 |
| n258 | -93.7 | -90.7 | -87.7 | -84.7 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

Beam correspondence and MBR requirement

For multi-band relaxation requirement, we can follow the agreement in Rel-16: specify multiband relaxation framework with per-band relaxation. Since this WI is only related to n257 and n258, only 1 multi-band combination (n257+n258) need to be considered.

Assuming 16 or 8 antenna elements for the new FWA UE, the beam width is finer for both DL and UL beam compared with PC3, it means that tiny inaccuracy on DL Beam measurement and corresponding beam selecting will lead to larger difference between best beam and corresponding beam. Meanwhile, larger scale antenna array will deteriorate the wide band antenna performance even worse. Hence, larger value for multiband relaxation would be needed for the new FWA UE.

For MBR value per band, there are still 3 options under discussion[7]:

- 0.7dB per band for both peak and spherical

- 0dB

- 0.5dB

We prefer 0.7dB which follows the Rel-16 MBR requirements which agrees in FR2 enhanced RF requirement WI. With more antenna elements than PC3, same MBR requirement is already an enhancement for antenna design.

FWA UE only have 85% CDF spherical coverage requirement, 1 or 2 beams could cover the required spherical range. Additionally, FWA is installed with direction toward to the gNB. Actually, beam correspondence requirement is not so important compared with PC3. This is why PC1 is successfully commercialized without BC requirement.

However, we are OK to follow BC requirement as defined for PC3, i.e. to define both bit 1 and bit 0 for the new FWA UE.

Proposal 3: For new FR2 FWA UE, specify the multi-band relaxation requirement per band as 0.7dB for both peak and spherical requirement.

Proposal4: Define both bit 0 and bit 1 beam correspondence requirement for the new FWA UE.

MPRnarrow for the new FWA UE

Generally, RAN4 assumes that MPR is defined based on the total TRP transmitted by all PAs. For new FWA UE, the output power for one PA maybe lower than PC3, this is because new FWA UE need more antenna elements to reach the higher EIRP, output power for each PA is reduced to ensure total TRP not exceed 23dBm. However, the total TRP for FWA UE is larger than PC3. For PC3, the max TRP is 23dBm, and min peak EIRP is 22.4dBm, the total TRP do not need to push to the max: 23dBm, it can be less than 23dBm, e.g. 16.5dBm, and UE still can meet the min peak EIRP requirement 22.4dBm.

As mentioned before, since the antenna elements number and the PA output power could be changed to reach the balance between high antenna gain and max TRP limitation, the MPR may need to be re-evaluated based on the PC3 MPR framework. For example, the min peak EIRP for PC3 is 22.4dBm while the max TRP is 23dBm with 4 antenna elements per polarization. Assuming 8.5dB antenna gain provided by most companies, the real TRP is about 25dBm(nominal peak EIRP)-8.5dB=16.5dBm; for the new FWA UE, we propose nominal peak EIRP with 35dBm with 14.5dB antenna gain, then the real TRP would be 20.5dBm. we can see a 4dB real TRP different between PC3 and PC5. Hence, the MPR may need to some re-evaluation.

Additionally, MPRnarrow is defined only for narrow RB allocation which is limited within 1.44MHz and only for the outer 1/3 position in the current spec(0 ≤ RBstart < Ceil(1/3 NRB) or Ceil(2/3NRB) ≤ RBstart ≤ NRB-LCRB). For such case, all the TRP transmission is focused on 1 RB and make PSD very high, then it leads to the emission problem.

Considering the potential TRP difference between PC3 and the new FWA UE, we propose to revise the MPRnarrow as 7dB for the new FWA UE. The MPR requirement other than MPRnarrow can reuse the values defined for PC3.

Proposal 5: Define MPRnarrow=7dB for the new FWA UE, other MPR requirement reuse the values defined for PC3.

Conclusion

In this contribution we discussed on the open issues on new FWA UE RF requirement, according to the analysis, we have the following proposals:

Observation 1: When Tx power is lower than 30dBm, we can see 64QAM is generally not possible for uplink transmission on 28GHz.

Proposal 1: define min peak EIRP for FR2 new FWA UE as in table 2.

Proposal 2: define min peak EIRP for FR2 new FWA UE as in table 3.

Proposal 3: For new FR2 FWA UE, specify the multi-band relaxation requirement per band as 0.7dB for both peak and spherical requirement. Define both bit 0 and bit 1 beam correspondence requirement for the new FWA UE.

Proposal 4: Define MPRnarrow=7dB for the new FWA UE, other MPR requirement reuse the values defined for PC3.

- References

[1] R4-2008175, “New FR2 FWA UE RF requirement”, Huawei, HiSilicon, RAN4 #95-e

[2] R4-2006776, “FR2 23dBm FWA pk. EIRP and REFSENS budgets”, Qualcomm, RAN4 #95-e

[3] R4-2006703, “Spherical EIRP EIS of FR2 FWA UE with maximum TRP of 23dBm”, MediaTek, RAN4 #95-e

[4] R4-2007110, “UE RF requirements for new FR2 FWA use case”, Intel, RAN4 #95-e

[5] R4-2006432, “Discussion on new FR2 FWA UE”, Samsung, RAN4 #95-e

[6] R4-2008921, “WF on FR2 FWA evaluations”, Qualcomm, RAN4 #95-e

[7] R4-2008922, “WF on FR2 FWA RF requirement”, Huawei, HiSilicon, RAN4 #95-e

[8] R4-2008964, “email discussion on new FR2 FWA”, SoftBank, RAN4 #95-e