3GPP TSG-RAN WG4 Meeting #97-e	R4-2016558

# Online Meeting, November 2 – 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1396 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revision of PRS-RSTD measurement period requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarify some aspects of the PRS-RSTD measurement period definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the measurement period calculation should be done according to the type of PRS processing capability reported by the UE. - Define the starting point of the PRS-RSTD measurement period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS-RSTD measurement period requirements would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

#### 9.9.2.5	Measurements Period Requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [4], during TRSTD,Total defined further in this clause.

When measurement gaps and processing time T have overlap between different positioning frequency layers, TRSTD,Total is defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS-RSTD measurement in positioning frequency layer i as defined further in this clause.

Editor’s note:	FFS the RSTD measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

TPRS-RSTD,i is the measurement period for PRS RSTD measurement in i positioning frequency layer as specified below

TPRS-RSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2  NRxBeam,i = [8].

CSSFPRS,i is the carrier-specific scaling factor for the positioning frequency layer i as defined in clause 9.1.5.2 as CSSFwithin_gap,i.

Nsample is the number of PRS RSTD samples and Nsample= [4].

Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + LPRS,i ,

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

LPRS,i is the time duration as defined in clause 5.1.6.5 of TS 38.214 [26, 5.1.6.5]. LPRS,i shall be calculated according to the type of DL PRS processing capability (type 1 or type 2) reported by the UE, as defined in clause 6.4.3 of TS 37.355 [34]

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 6.4.3 of TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 6.4.3 of TS 37.355 [34].

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

The time TPRS-RSTD,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-DL-TDOA-RequestLocationInformation message and NR-DL-TDOA -ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If handover occurs while RSTD measurements are being performed then the UE shall complete the on-going positioning measurement session. The UE shall also meet the RSTD measurement and accuracy requirements. However, in this case the RSTD measurement period TRSTD,total.HO shall be as follows:

TRSTD, total,HO=TRSTD, Total+K*Teffect+THO

Where,

-	K is the number of times handover occurs during TPRS-RSRP,total.HO;

-	Teffect is the largest Teffect,i among all PRS layers;

-	THO is the time during which the RSTD measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

# < End of change >