3GPP TSG-RAN WG4 Meeting # 97-e                                                             R4-2016855

# Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation, CMCC

Title:	TP for TR38.xxx_ PC2 CA_n3A-n41A

Agenda Item:	10.19.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce PC2 2DL/2UL CA_n3A-n41A in TR38.XXX[1].

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n3A-n41A requirements are applied.

- Reference

- TR38.xxx, High power User Equipment (UE) (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink; (Release 17), v0.0.1

- Text Proposal

----- Start of TP -----

  - 5.x			CA_n3-n41

    - 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n41A | CA_n3A-n41A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

    - 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3A-n41A | CA power class | Carrier n3 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

    - 5.x.3	REFSENS requirements

According to the PC3 CA_n3A-n41A study, MSD caused by IMD4 which falls into its own Band 3 Rx, and MSD due to cross-band isolation are defined. The same approach can be applied to PC2 CA_n3A-n41A. In addition, the studies for PC2 ENDC 3+n41 have been completed, therefore the MSD values of PC2 ENDC 3+n41 can be applied to PC2 NR CA n3+n41.

      - 5.x.3.1	Power class 2 case a

Comparing with PC3 CA_n3A-n41A, no additional MSD are expected  for this PC2 CA_n3A-n41A with dual uplink carrier.

      - 5.x.3.2	Power class 2 case b

The additional MSD due to croass band isolation for PC2 CA_n3A-n41A are defined in table 5.x.3.2-1.

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 NR CA in NR FR1

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 |
| n41 | n3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 CA_n3A-n41A are defined in table 5.x.3.2-2.

Table 5.x.3.2-2: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| NR/ Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n3A_n41A | n3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |

    - 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n3A-n41A requirements are applied for PC2 CA_n3A-n41A.

----- End of TP -----