3GPP TSG-RAN WG4 Meeting#97-e                                             R4-2016906

E-meeting,2nd Nov – 13th Nov, 2020

Agenda item:	11.1.2

Source: 	ZTE Corporation

Title: 	TP to TR38.921 : BS spurious emission

Document for:	Approval

Introduction

During the last RAN4#96e meeting, there were some initial discussions on BS spurious emission requirement. In this contribution, we want to share some further inputs for BS spurious emission requirements.

Discussion

Based on the previous discussion, it seems that option 1 for 6.425-7.125GHz is more preferable among companies, only further update needed are just to keep upper frequency of Tx spurious emission to be 26GHz instead of 5 harmonic of DL frequency according to ITU-R SM 329-10 recommendation.

Option 2 for 10-10.5GHz is more preferable among companies which is also aligned with ERC 74-01 report with certain level relaxation between 18GHz and 26GHz.

Based on the above considerations, the BS spurious emission requirement for 7GHz and 10GHz could be updated as following:

  - Option 1: for 7 GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz –26GHz |  | 1 MHz |

  - Option 2: for 10 GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18GHz –26GHz | -20 dBm | 10 MHz |

Text proposal

### 6.1.4	Spurious emissions

Regarding spurious emission requirements for 6.425-7.125GHz, as this frequency range is still within FR1 band definition,  it is agreed to reuse the existing spurious emission requirements defined in TS 38.104 and further update needed are to define upper frequency of Tx spurious emission to be 26GHz instead of 5 harmonic of DL frequency according to ITU-R SM 329-10 recommendation. The general spurious emissions for 6.425-7.125GHz is defined in Table 6.1.4-1.

Regarding spurious emission requirements for 10-10.5GHz, according to ERC 74-01 report, the lower frequency limit could be defined as 30MHz.Without any specified spurious limit for AAS BS in between 6 and 24GHz in ERC 74-01, in relation to the existing frequency ranges specified, with observations of the foreseen implementation of AAS BS and how they are to be deployed and used, the spurious limits specified for above 24GHz with upper frequency limited to 26GHz will be used for 10.0-10.5 GHz as  defined in Table 6.1.4-2.

Table 6.1.4-1: BS spurious emission limits for 6.425-7.125GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 26GHz | -30 dBm | 1 MHz |

Table 6.1.4-2: BS spurious emission limits for 10-10.5GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18GHz –26GHz | -20 dBm | 10 MHz |