3GPP TSG-RAN WG4 Meeting #97-e                                                                R4-2017111

Electronic Meeting, 2 Nov. – 13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on V2X UE Congestion Control Measurement Test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The congestion control measurement test is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new test case for congestion control measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revised from CR R4-2014770. |  |  |  |  |  |  |  |  |

Start of Change

A.9.1.5	Congestion Control Measurement Test for V2X UE

A.9.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the V2X UE makes correct reporting of an event. This test will verify the congestion control measurement requirements in section 12.6.

The test parameters are given in Table A.9.1.5.1-1 and A.9.1.5.1-2 below. In the measurement control information it is indicated to the V2X UE that event-triggered reporting with Event C1 is used. There are 4 active sidelink UEs in this test. The test system shall emulate the active sidelink UE to transmit PSCCH/PSSCH every 50ms. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During T1, all of active sidelink UEs are configured to transmit PSCCH/PSSCH with lower transmission power every 50ms. During T2, all of active sidelink UEs are configured to transmit PSCCH/PSSCH with higher transmission power every 50ms.

[This test is only applicable to UE which supports both SL and Uu Link.]

[Supported test configurations for FR1 NR cell are shown in Table A.9.1.x.1.1-1.]

[Table A.9.1.5.1.1-1: Supported Test Configurations for FR1 NR cell as synchronization reference source]

| Configuration | Description |
| --- | --- |
|  | NR cell |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Supported test configurations for active NR V2X UE are shown in Table A.9.1.x.1.1-2.

Table A.9.1.5.1.1-2: Supported Test Configurations for active V2X UE

| Configuration | Description |
| --- | --- |
|  | Active V2X UE |
| 1 | 30 kHz SSB SCS, 20 MHz bandwidth |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.9.1.5.1-1: General test parameters for Congestion Control Measurement Test for V2X UE

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) |  | V2X UE Config 1 | MHz | 20 |  |
|  |  | V2X UE Config 2 |  | 40 |  |
| SCS |  |  | kHz | 30 |  |
| V2X sidelink communication configuration |  |  |  | As specified in Table A.3.19.2-1 and A.3.19.2-3 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  |  | 1 | ENUMERATED {n1} |
| sl-SubchannelSize included in SL-ResourcePool |  |  |  | 10 | ENUMERATED {n10} |
| sl-StartRB-Subchannel-r16 |  |  |  | 0 |  |
| threshS-RSSI-CBR |  |  |  | 19 | Corresponding -74dBm as defined in Section 6.3.8 in TS38.331 |
| Number of Active Sidelink UEs every 50ms |  |  |  | 4 | Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Active Sidelink UEs (i = 0,1,2,3) | V2X sidelink Communication configuration |  |  | As specified in Table A.3.19.2-1 and A.3.19.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  |  | [1]i Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 |  |
|  | sl-SubchannelSize included in SL-ResourcePool |  |  | 10 |  |
| Timing offset between V2X UE and Active Sidelink UEs |  |  | s | 3 | Synchronous |
| c1-Threshold-r16 |  |  |  | 2 | Corresponding 0.02 as defined in Section 6.3.2 in TS38.331 |
| Hysteresis |  |  |  | 0 |  |
| Time To Trigger |  |  | s | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | 5 |  |
| Note 1: [1]i is a sequence of ninety nine 0’s with one 1 in i+1’th position. Note 2: Only sidelink UE which has both Uu link and sidelink shall be tested. |  |  |  |  |  |

Table A.9.1.5.1-2: Active sidelink UE specific test parameters for Congestion Control Measurement Test for V2X UE

| Parameter |  | Unit | Active Sidelink UE i (i = 0, 1, 2, 3) |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |
| Channel Bandwidth (BWchannel) | V2X UE Config 1 | MHz | 20 |  |
|  | V2X UE Config 2 |  | 40 |  |
| SCS |  | kHz | 30 |  |
| PSCCH RMC (defined in A.3.19.3) | V2X UE Config 1 |  | [CC.1A HD] |  |
|  | V2X UE Config 2 |  | [CC.2A HD] |  |
| PSSCH RMC (defined in A.3.19.3) | V2X UE Config 1 |  | [CD.1A HD] |  |
|  | V2X UE Config 2 |  | [CD.2A HD] |  |
| Note1 |  | dBm/30 KHz | [-103] |  |
|  |  | dB | [4.35] | [10.32] |
| SL-RSRP Note 2 |  | dBm/30 KHz | [-98.65] | [-92.68] |
| S-RSSI1 Note 2 Note3 |  | dBm/3.6 MHz | [-76.5] | [-71.5] |
| S-RSSI2 Note 2 Note4 |  | dBm/3.6 MHz | [-82.21] | [-82.21] |
| Io1 Note 2 Note4 |  | dBm/3.6 MHz | [-76.5] | [-71.5] |
| Io2 Note 2 Note6 |  | dBm/3.6 MHz | [-82.21] | [-82.21] |
| Propagation Condition |  | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSSCH Es/Noc, PSSCH-RSRP, S-RSSI1, S-RSSI2, Io1 and Io2 levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SL-RSSI1 is the SL-RSSI level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 4: SL-RSSI2 is the S-RSSI level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 5: Io1 is the Io level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 6: Io2 is the Io level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. |  |  |  |  |

A.9.1.5.2	Test Requirements

The UE shall not send event C1 triggered measurement reports during T1 and shall send event C1 triggered measurement reports during T2.

The rate of correct events observed during repeated tests shall be at least 98%.

End of Change