3GPP TSG-RAN WG4 Meeting # 97-e	R4-2017451

# Electronic meeting, 2 – 13 November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.107 | CR | 0008 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.107 with update of EDT level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eLAA |  |  |  |  |  | Date: |  |  | 2020-11-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces update for interfering signal of energy detection accuracy (EDT) to align with RAN1 specification TS 37.213. Details of this changes are described in [1]. CR includes outcome of the RAN4#97e meeting discussions. [1] R4-2015375, Further discussion on additional optional EDT level for test, Nokia, Nokia Shaghai Bell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding option of declaration by BS vendor energy detection treshold that depends on regulatory rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification TS 37.107 will be in the conflict with energy detection threshold allowed/mandated by core specification TS 37.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 6.1.4.2, 6.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of R4-2015376 |  |  |  |  |  |  |  |  |

<Start of changes>

### 5.1.1	Channel access parameters

Channel access related parameters for PDSCH are listed in Table 5.1.1-1.

Table 5.1.1-1: Channel access parameters for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| LBT measurement bandwidth | MHz | 10, 20 |
| Energy detection threshold | dBm/20MHz dBm/10MHz | -72 or X (Note) -75 |
| Maximum channel occupancy time | ms | 8 |
| Note: The specific value X is declared by the vendor. |  |  |

<Unchanged sections omitted>

### 6.1.4	Method of test

#### 6.1.4.1	Initial conditions

Test environment:								normal; see Annex D.2 of TS 36.141 [2].

RF channels to be tested for single carrier:		B, M and T; see clause 4.7 of TS 36.141 [2].

Connect the signal analyzer to the base station antenna connector as shown in Annex I of TS 36.141 [2].

#### 6.1.4.2	Procedure

MCOT and minimum idle time

1)	Set the base station to transmit a signal according to E-TM 1.1 at manufacturer’s declared rated output power with corresponding channel bandwidth (i.e. 10 MHz or 20 MHz). Channel Access Priority Class 3 parameters are selected to be tested based on Table 4.1.1-1 in TS 37.213.

2)	Measure the transmitter ON period during the continuous transmission (after the first channel access).

3)	Measure the transmitter OFF period between two consecutive transmitter ON periods.

4)	Verify minimum idle time as follows:

The transmitter OFF period between two consecutive transmitter ON periods shall not be less than 25 µs.

5)	Verify maximum channel occupancy time (MCOT) as follows:

a)	The duration of each transmitter ON period continuous transmission shall not exceed the maximum channel occupancy time (MCOT) requirement specified in clause 6.1.5.

Energy detection accuracy

6)	Generate the interfering signal of AWGN with corresponding channel bandwidth (i.e. 10 MHz or 20 MHz) at the same centre frequency as the tested channel. The interfering signal shall be at a level as specified in table 6.1.5-1. The base station shall stop transmission on the current operating channel and will not resume normal transmissions as long as the interference signal is present.

7)	The step 6) is repeated multiple times considering the following sub-steps:

-	Interferer ON: if the interfering signal is present, the interfering signal should be present for 10ms.

-	Interferer OFF: if the interfering signal is removed, the interfering signal should be absent for 10ms.

-	The total number of interferer ON duration is assumed to be N and the total number of interferer OFF duration is assumed to be M. The value N, M and the sequence of interferer ON/OFF pattern shall be generated randomly for the test.

8)	In the test, a counter is maintained with initial value set to 0 when the test starts.

9)	For every 10ms Interferer ON period, the counter is increased by 1 if there is either an ON/OFF transition or no transmission by the DUT. To pass the test, the counter shall not be less than N*0.9.

### 6.1.5	Test Requirements

In normal conditions, the measurement result shall meet channel access related test requirements for PDSCH as listed in Table 6.1.5-1.

Table 6.1.5-1: Channel access test requirements for PDSCH

| Parameter | Unit | Value |
| --- | --- | --- |
| LBT measurement bandwidth | MHz | 10, 20 |
| Maximum energy detection threshold | dBm/20MHz dBm/10MHz | -72 + 4dB or X + 4dB (Note) -75 + 4dB |
| Maximum channel occupancy time | ms | 8 |
| Note: The specific value X is declared by the vendor. |  |  |

The Base Station shall be able to assess whether the medium is busy or idle with at least 90% probability, using a channel access procedure with the parameters in Table 6.1.5-1.

<End of changes>