3GPP TSG-RAN WG4 Meeting #97-e	R4-2017532

# Electronic Meeting, 2 - 13 Nov, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | <CR#> | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for 38.101-4: Introduction of PDSCH requirement with Single-DCI based SDM scheme |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Perf |  |  |  |  |  | Date: |  |  | 2020-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 agrees to introduce PDSCH requirements of Single-DCI based SDM scheme and the aligned requirements need to be added into the specfication |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PDSCH requirements of Single-DCI based SDM scheme |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH requirements of Single-DCI based SDM scheme are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.5, 5.2.2.2.5, 5.2.3.1.5, 5.2.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2015653 |  |  |  |  |  |  |  |  |

<Start of First change>

5.2.2.1.11	Minimum requirements for PDSCH Single-DCI based SDM scheme

The performance requirements are specified in Table 5.2.2.1.11-3, with the addition of test parameters in Table 5.2.2.1.11-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.11-1.

Table 5.2.2.1.11-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme of Multi-TRP under 2 receive antenna conditions | 1-1,1-2 |

Table 5.2.2.1.11-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRP #1 | TRP #2 |
| Transmit TRP of SSB |  |  |  |  | TRP #1 |  |
| PDCCH configuration |  | TCI state |  |  | [TCI State #1] |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |
|  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  | FDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | [2] |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRP from the first TRP |  |  |  | us | -0.5 for test 1-1 2 for test 1-2 |  |
| Frequency offset of the second TRP from the first TRP |  |  |  | Hz | 200 for test 1-1 0 for test 1-2 |  |
| Number of HARQ Processes |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | 2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRPs, random per slot with PRB bundling granularity. |  |

Table 5.2.2.1.11-3: Minimum performance

| Test num. | Reference channel(Note) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNR (dB) |
| 1-1 | [R.PDSCH.1-3.2 FDD] | 10 / 15 | 64QAM, 0.50 | TDLA30-10 for each TRP | 2x2, ULA Low for each TRP | 70 |  | TBD |
| 1-2 | [R.PDSCH.1-3.2 FDD] | 10 / 15 | 64QAM, 0.50 | TDLA30-10 for each TRP | 2x2, ULA Low for each TRP | 70 | TBD |
| Note: Reference channel is configured for two TRPs |  |  |  |  |  |  |  |

<end of First change>

<Start of Second change>

5.2.2.2.11	Minimum requirements for PDSCH Single-DCI based SDM scheme

The performance requirements are specified in Table 5.2.2.2.11-3, with the addition of test parameters in Table 5.2.2.2.11-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.11-1.

Table 5.2.2.2.11-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme of Multi-TRP under 2 receive antenna conditions. | 1-1,1-2 |

Table 5.2.2.2.11-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRP #1 | TRP #2 |
| Transmit TRP of SSB |  |  |  |  | TRP #1 |  |
| PDCCH configuration |  | TCI state |  |  | [TCI State #1] |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
|  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | [2] |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRP from the first TRP |  |  |  | us | -0.25 for test 1-1 1 for test 1-2 |  |
| Frequency offset of the second TRP from the first TRP |  |  |  | Hz | 300 for test 1-1 0 for test 1-2 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRPs, random per slot with PRB bundling granularity. |  |

Table 5.2.2.2.11-3: Minimum performance

| Test num. | Reference channel(Note) | Bandwidth (MHz) / Subcarrier spacing (kHz) |  | Modulation format and code rate |  | TDD UL-DL pattern |  | Propagation condition |  | Correlation matrix and antenna configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) |  | SNR (dB) |  |
| 1-1 | [R.PDSCH.2-3.2 TDD] | 40 / 30 |  | 64QAM, 0.50 |  | FR1.30-1 |  | TDLA30-10 for each TRP |  | 2x2, ULA Low for each TRP |  | 70 |  | TBD |  |
| 1-2 | [R.PDSCH.2-3.2 TDD] | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 for each TRP | 2x2, ULA Low for each TRP | 70 | TBD |
| Note: Reference channel is configured for two TRPs |  |  |  |  |  |  |  |  |

<end of Second change>

<Start of Third change>

5.2.3.1.11	Minimum requirements for PDSCH Single-DCI based SDM scheme

The performance requirements are specified in Table 5.2.3.1.11-3, with the addition of test parameters in Table 5.2.3.1.11-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.11-1.

Table 5.2.3.1.11-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme of Multi-TRP | 1-1,1-2 |

Table 5.2.3.1.11-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRP #1 | TRP #2 |
| Transmit TRP of SSB |  |  |  |  | TRP #1 |  |
| PDCCH configuration |  | TCI state |  |  | [TCI State #1] |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 20 |  |
|  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |
|  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  | FDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | [2] |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRP from the first TRP |  |  |  | us | -0.5 for test 1-1 2 for test 1-2 |  |
| Frequency offset of the second TRP from the first TRP |  |  |  | Hz | 200 for test 1-1 0 for test 1-2 |  |
| Number of HARQ Processes |  |  |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | 2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRPs, random per slot with PRB bundling granularity. |  |

Table 5.2.3.1.11-3: Minimum performance

| Test num. | Reference channel(Note) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.1-3.2 FDD] | 10 / 15 | 64QAM, 0.50 | TDLA30-10 for each TRP | 2x4, ULA Low for each TRP | 70 | TBD |
| 1-2 | [R.PDSCH.1-3.2 FDD] | 10 / 15 | 64QAM, 0.50 | TDLA30-10 for each TRP | 2x4, ULA Low for each TRP | 70 | TBD |
| Note: Reference channel is configured for two TRPs |  |  |  |  |  |  |  |

<end of third change>

<Start of Forth change>

5.2.3.2.11	Minimum requirements for PDSCH Single-DCI based SDM scheme

The performance requirements are specified in Table 5.2.3.2.11-3, with the addition of test parameters in Table 5.2.3.2.11-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.11-1.

Table 5.2.3.2.11-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme of Multi-TRP under 4 receive antenna conditions. | 1-1,1,2 |

Table 5.2.3.2.11-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRP #1 | TRP #2 |
| Transmit TRP of SSB |  |  |  |  | TRP #1 |  |
| PDCCH configuration |  | TCI state |  |  | [TCI State #1] |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 40 |  |
|  |  | CSI-RS offset |  | Slots | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 | 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 |
|  |  | QCL info |  |  | TCI state #0 |  |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | [2] |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | N/A |
|  |  |  | QCL Type |  | N/A | N/A |
| Resource allocation |  |  |  |  | Full-overlappling |  |
| Timing offset of the second TRP from the first TRP |  |  |  | us | -0.25 for test 1-1 1 for test 1-2 |  |
| Frequency offset of the second TRP from the first TRP |  |  |  | Hz | 300 for test 1-1 0 for test 1-2 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRPs, random per slot with PRB bundling granularity. |  |

Table 5.2.3.2.11-3: Minimum performance

| Test num. | Reference channel(Note) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | [R.PDSCH.2-3.2 TDD] | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 for each TRP | 2x4, ULA Low for each TRP | 70 | TBD |
| 1-2 | [R.PDSCH.2-3.2 TDD] | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 for each TRP | 2x4, ULA Low for each TRP | 70 | TBD |
| Note: Reference channel is configured for two TRPs |  |  |  |  |  |  |  |  |

<end of forth change>