3GPP TSG-RAN WG4 Meeting #97-e 	R4-2014908

# Online, November 2nd – 13th, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0280 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-2: Clarification for NS_202 emission requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NS_202 contains two emission requirements, one is for additional spurious emission requirement at -10 dBm/100 MHz, the other at 1 dBm/200 MHz is meant for protection of satellite passive services. Since the former requirement is tighter and also covers the frequency range of the latter requirement, without clarification on the purpose of the latter requirement, it would look to be redudant for the latter requirement in NS_202. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NOTE 1 to Table 6.5.3.2.3-1 (NS_202) for the emission requirement at 1 dBm/200 MHz for protection frequency range 23600 MHz – 24000 MHz to clarifiy that “This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 – 24000 MHz is meant for protection of satellite passive services.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 1 dBm/200 MHz requirement in NS_202 would look to be redudant without the clarification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changed sections >>>

##### 6.5.3.2.3	Additional spurious emission requirements for NS_202

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |  |
| --- | --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |  |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

#### <<< End of changed sections>>>