3GPP TR 38.717-03-02 V0.2.0 (2020-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 3 bands DL with 2 bands UL

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

- Contents

Foreword	8

1 Scope	9

2 References	9

3 Definitions, symbols and abbreviations	9

3.1 Definitions	9

3.2 Symbols	10

3.3 Abbreviations	10

4 Background	10

4.1 TR Maintenance	10

5 Inter-band Carrier Aggregation for 3 bands DL with 2 bands UL: Specific Band Combination Part	10

5.1 Inter-band within FR1	10

5.1.1 CA_n39-n40-n79	10

5.1.1.1 Operating bands for CA	10

5.1.1.2 Channel bandwidths per operating band for CA	11

5.1.1.3 Co-existence studies	11

5.1.1.4 REFSENS requirements	11

5.1.2 CA_n39-n40-n41	12

5.1.2.1 Operating bands for CA	12

5.1.2.2 Channel bandwidths per operating band for CA	12

5.1.2.3 Co-existence studies	12

5.1.2.4 REFSENS requirements	12

5.1.3 CA_n5-n25-n66	13

5.1.3.1 Operating bands for CA	13

5.1.3.2 Channel bandwidths per operating band for CA	13

5.1.3.3 UE co-existence studies	13

5.1.3.4 REFSENS requirements	15

5.1.4 CA_n5-n25-n78	16

5.1.4.1 Operating bands for CA	16

5.1.4.2 Channel bandwidths per operating band for CA	16

5.1.4.3 Co-existence studies	16

5.1.4.4 REFSENS requirements	18

5.1.5 CA_n1-n77-n79	19

5.1.5.1 Operating bands for CA	19

5.1.5.2 Channel bandwidths per operating band for CA	19

5.1.5.3 UE co-existence studies	19

5.1.5.4 REFSENS requirements	19

5.1.6 CA_n1-n78-n79	20

5.1.6.1 Operating bands for CA	20

5.1.6.2 Channel bandwidths per operating band for CA	20

5.1.6.3 UE co-existence studies	20

5.1.6.4 REFSENS requirements	20

5.1.7 CA_n2-n66-n77	21

5.1.7.1 Operating bands for CA	21

5.1.7.2 Channel bandwidths per operating band for CA	21

5.1.7.3 Co-existence studies	21

5.1.7.4 REFSENS requirements	23

5.1.8 CA_n3-n28-n41	24

5.1.8.1 5.1.8.1	Operating bands for CA	24

5.1.8.2 Channel bandwidths per operating band for CA	24

5.1.8.3 UE co-existence studies	26

5.1.8.4 REFSENS requirements	26

5.1.9 CA_n3-n28-n78	26

5.1.9.1 Operating bands for CA	26

5.1.9.2 Channel bandwidths per operating band for CA	27

5.1.9.3 Co-existence studies	27

5.1.9.4 REFSENS requirements	27

5.1.10 CA_n5-n66-n77	27

5.1.10.1 Operating bands for CA	27

5.1.10.2 Channel bandwidths per operating band for CA	28

5.1.10.3 Co-existence studies	28

5.1.10.4 REFSENS requirements	30

5.1.11 CA_n8-n40-n41	31

5.1.11.1 Operating bands for CA	31

5.1.11.2 Channel bandwidths per operating band for CA	31

5.1.11.3 Co-existence studies	32

5.1.11.4 REFSENS requirements	32

5.1.12 CA_n25-n41-n77	32

5.1.12.1 Operating bands for CA	32

5.1.12.2 Channel bandwidths per operating band for CA	33

5.1.12.3 UE co-existence studies	33

5.1.12.4 REFSENS requirements	33

5.1.13 CA_n25A-n66A-n77A	34

5.1.13.1 Operating bands for CA	34

5.1.13.2 Channel bandwidths per operating band for CA	34

5.1.13.3 UE co-existence studies	35

5.1.13.4 REFSENS requirements	35

5.1.14 CA_n25A-n71A-n77A	36

5.1.14.1 Operating bands for CA	36

5.1.14.2 Channel bandwidths per operating band for CA	36

5.1.14.3 UE co-existence studies	36

5.1.14.4 REFSENS requirements	36

5.1.15 CA_n41A-n66A-n77A	37

5.1.15.1 Operating bands for CA	37

5.1.15.2 Channel bandwidths per operating band for CA	38

5.1.15.3 UE co-existence studies	38

5.1.15.4 REFSENS requirements	38

5.1.16 CA_n41A-n71A-n77A	39

5.1.16.1 Operating bands for CA	39

5.1.16.2 Channel bandwidths per operating band for CA	40

5.1.16.3 UE co-existence studies	40

5.1.16.4 REFSENS requirements	40

5.1.17 CA_n66A-n71A-n77A	41

5.1.17.1 Operating bands for CA	41

5.1.17.2 Channel bandwidths per operating band for CA	42

5.1.17.3 UE co-existence studies	42

5.1.17.4 REFSENS requirements	42

5.2 Inter-band within FR2	43

5.2.1 CA_nX-nY-nZ	43

5.2.1.1 Operating bands for CA	43

5.2.1.2 Channel bandwidths per operating band for CA	43

5.2.1.3 UE co-existence studies	43

5.2.1.4 REFSENS requirements	43

5.3 Inter-band between FR1 and FR2	43

5.3.1 CA_n1-n77-n257	43

5.3.1.1 Operating bands for CA	43

5.3.1.2 Channel bandwidths per operating band for CA	44

5.3.1.3 UE co-existence studies	45

5.3.1.4 REFSENS requirements	45

5.3.2 CA_n1-n78-n257	45

5.3.2.1 Operating bands for CA	45

5.3.2.2 Channel bandwidths per operating band for CA	45

5.3.2.3 UE co-existence studies	46

5.3.2.4 REFSENS requirements	46

5.3.3 CA_n1-n79-n257	47

5.3.3.1 Operating bands for CA	47

5.3.3.2 Channel bandwidths per operating band for CA	48

5.3.3.3 UE co-existence studies	50

5.3.3.4 REFSENS requirements	50

5.3.4 CA_n77-n79-n257	50

5.3.4.1 Operating bands for CA	50

5.3.4.2 Channel bandwidths per operating band for CA	51

5.3.4.3 UE co-existence studies	52

5.3.4.4 REFSENS requirements	52

5.3.5 CA_n78-n79-n257	52

5.3.5.1 Operating bands for CA	52

5.3.5.2 Channel bandwidths per operating band for CA	53

5.3.5.3 UE co-existence studies	54

5.3.5.4 REFSENS requirements	54

6 Dual Connectivity with 3 bands DL: Specific Band Combination Part	54

6.1 6.X	DC_nX-nY-nZ	54

6.1.1 6.x.1   Configurations for DC_nX-nY-nZ	54

Annex A: Change history	55

- Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

- Scope

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with 2 bands UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 3 bands DL with 2 bands UL for the Rel-17 band combinations.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

Definitions, symbols and abbreviations

Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

Background

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with 2 bands UL under Rel-17 time frame. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

Inter-band Carrier Aggregation for 3 bands DL with 2 bands UL: Specific Band Combination Part

Inter-band within FR1

CA_n39-n40-n79

Operating bands for CA

Table 5.1.1.1-1: CA band combination of band n39+n40+n41

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n39A-n40A-n79A | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported channel bandwidths per CA configuration for band n39+n40+n41

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Actually, the co-existence studies for dual uplink operation of two bands, i.e. CA_n39A-n40A, CA_n39A-n79A and CA_n40A-n79A have been captured in TR38.716-02-00, where:

- IMD4 products produced by Band 39 and Band n40 that impact the reference sensitivity of NR band n79.

- No IMD products produced  by Band 39 and Band n79 that impact the reference sensitivity of NR band n40.

- IMD4 and IMD5 products produced by Band 40 and Band n79 that impact the reference sensitivity of NR band n39.

REFSENS requirements

According to clause 5.1.1.3, some IM3 produces of dual uplink operation of two bands will falling into the DL of the third band. However, considering the requirements for TDD-TDD NR CA combinations of CA_n39-n40 are defined without simultaneous Rx/Tx capability in TS38.101-1, i.e. synchronous operation. Therefore it is no need to defined MSD requirements for band n39 due to IMD4 and IMD5 products produced by Band 40 and Band n79.

For the MSD for NR band n79 caused by IMD4 products of Band 39 and Band n40, since the CA_n39-n79 and CA_n40-n79 are operated with mandatory simultaneous Rx/Tx capability in TS38.101-1, i.e. asynchronous operation. Therefore it is need to defined MSD requirements for band n79 due to IMD4 products produced by Band 39 and Band n40.

The required MSD is shown in the table 5.1.1.4-1, where the MSD of EN-DC_39A_n40A-n79A are re-used.

Table 5.1.1.4-1: MSD due to IMD4

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39A-n40A-n79A | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | TDD | IMD4 |

CA_n39-n40-n41

Operating bands for CA

Table 5.1.2.1-1: CA band combination of band n39+n40+n41

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n39A-n40A-n41A | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.2.2-1: Supported channel bandwidths per CA configuration for band n39+n40+n41

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Actually, the co-existence studies for dual uplink operation of two bands, i.e. CA_n39A-n40A, CA_n39A-n41A and CA_n40A-n41A have been captured in TR38.716-02-00, where:

- 	IMD3 products produced by Band n39 and n40 that impact the reference sensitivity of NR band n41.

- 	No IMD3 products produced by Band n39 and n41 that falling into the band n41 Rx.

- 	2nd and 5rd  products produced by Band n40 and n41 may falling into the band n39 Rx.

REFSENS requirements

According to clause 5.1.2.3, some IM3 produces of dual uplink operation of two bands will falling into the DL of the third band. However, considering the requirements for TDD-TDD NR CA combinations of CA_n39-n40, CA_n39-n41 and CA_n40-n41 are defined without simultaneous Rx/Tx capability in TS38.101-1, i.e. synchronous operation. Therefore it is no need to defined MSD requirements due to IMD3 issues, i.e. no specific REFSENS requirements for this combination in 3DL/2UL NR CA operation.

CA_n5-n25-n66

Operating bands for CA

Table 5.1.3.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n25-n66 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.3.2-1: Supported bandwidths per CA band combination of band n5+n25+n66

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n25A-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66A | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n5A-n25A-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n25(2A)-n66(2A) | CA_n5A-n25A CA_n5A-n66A CA_n25A-n66A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

For UE coexistence study of Band n5 + Band n25, Band n5 + Band n66, and Band n25 + band n66, the 2nd, 3rd, 4th and 5th order harmonics are already analyzed in 3DL/1UL WI, where no harmonic issue is identified. The 2nd, 3rd, 4th and 5th order intermodulation products are calculated and presented in Table 5.1.3.3-1, 5.1.3.3-2 and 5.1.3.3-3, respectively.

Table 5.1.3.3-1: Harmonic and IMD analysis for n5+n25

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1850 | 1915 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 3700 | 3830 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 5550 | 5745 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1001 | 1091 | 2674 | 2764 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -267 | -152 | 2851 | 3006 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3498 | 3613 | 4524 | 4679 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 557 | 697 | 4701 | 4921 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4322 | 4462 | 6374 | 6594 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -2182 | -2002 | 5348 | 5528 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6836 | -6551 | -1546 | -1381 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8224 | 8509 | 5146 | 5311 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4097 | -3852 | 1153 | 1358 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7198 | 7443 | 6172 | 6377 |

Table 5.1.3.3-2: Harmonic and IMD analysis for n5+n66

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 3420 | 3560 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 5130 | 5340 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 861 | 956 | 2534 | 2629 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -132 | -12 | 2571 | 2736 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3358 | 3478 | 4244 | 4409 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 692 | 837 | 4281 | 4516 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4182 | 4327 | 5954 | 6189 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1912 | -1722 | 5068 | 5258 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6296 | -5991 | -1686 | -1516 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 7664 | 7969 | 5006 | 5176 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -3692 | -3432 | 873 | 1088 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 6778 | 7038 | 5892 | 6107 |

Table 5.1.3.3-2: Harmonic and IMD analysis for n25+n66

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 1850 | 1915 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 70 | 205 | 3560 | 3695 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1505 | 1710 | 1920 | 2120 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5270 | 5475 | 5410 | 5610 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3215 | 3490 | 3770 | 4035 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 6980 | 7255 | 7260 | 7525 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -410 | -140 | 7120 | 7390 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -5950 | -5620 | -5270 | -4925 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 9110 | 9440 | 8690 | 9035 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -2325 | -1990 | -1640 | -1300 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8970 | 9305 | 8830 | 9170 |

Co-existence studies shows that

- The 4th IMD generated by dual uplink of Band n5 + Band n25 may fall into own Rx of band n66.

- The 2nd IMD generated by dual uplink of Band n5 + Band n66 may fall into own Rx of band n5.

- The 5th IMD generated by dual uplink of Band n5 + Band n66 may fall into own Rx of band n5.

- The 3rd IMD generated by dual uplink of Band n25 + Band n66 may fall into own Rx of band n25.

- The 3rd IMD generated by dual uplink of Band n25 + Band n66 may fall into own Rx of band n66.

- The 5th IMD generated by dual uplink of Band n25 + Band n66 may fall into own Rx of band n66.

REFSENS requirements

The IMD issue specific to 3DL/2UL is that the 4th IMD generated by dual uplink of Band n5 + Band n25 may fall into own Rx of band n66. As this IMD4 issue is similar to CA_2A-5A-66A, the same MSD value is reused in Table 5.1.3.4-1.

Table 5.1.3.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n25A-n66A | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A |  | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 |  | IMD4 |

CA_n5-n25-n78

Operating bands for CA

Table 5.1.4.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n25-n78 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.4.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n25A-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n5A-n25(2A)-n78A | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n5A-n25A-n78(2A) | CA_n5A-n25A CA_n5A-n78A CA_n25A-n78A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Co-existence studies

For UE coexistence study of Band n5 + Band n25, Band n5 + Band n78, and Band n25 + band n78, the 2nd, 3rd, 4th and 5th order harmonics are already analyzed in 3DL/1UL WI, where no harmonic issue is identified. The 2nd, 3rd, 4th and 5th order intermodulation products are calculated and presented in Table 5.1.4.3-1, 5.1.4.3-2 and 5.1.4.3-3, respectively.

Table 5.1.4.3-1: Harmonic and IMD analysis for n5+n25

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1850 | 1915 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 3700 | 3830 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 5550 | 5745 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1001 | 1091 | 2674 | 2764 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -267 | -152 | 2851 | 3006 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3498 | 3613 | 4524 | 4679 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 557 | 697 | 4701 | 4921 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4322 | 4462 | 6374 | 6594 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -2182 | -2002 | 5348 | 5528 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6836 | -6551 | -1546 | -1381 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8224 | 8509 | 5146 | 5311 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4097 | -3852 | 1153 | 1358 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7198 | 7443 | 6172 | 6377 |

Table 5.1.4.3-2: Harmonic and IMD analysis for n5+n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1648 | 1698 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2472 | 2547 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 2451 | 2976 | 4124 | 4649 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -2152 | -1602 | 5751 | 6776 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 4948 | 5498 | 7424 | 8449 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | -1328 | -753 | 9051 | 10576 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 5772 | 6347 | 10724 | 12249 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -5952 | -4902 | 8248 | 9298 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -14376 | -12351 | -96 | 504 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14024 | 16049 | 6596 | 7196 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -9752 | -8202 | 4053 | 5128 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11548 | 13098 | 9072 | 10147 |

Table 5.1.4.3-2: Harmonic and IMD analysis for n25+n78

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 3800 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 3700 | 3830 | 6600 | 7600 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 5550 | 5745 | 9900 | 11400 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 1950 | 5150 | 5715 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -100 | 530 | 4685 | 5750 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 7630 | 8450 | 9515 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1750 | 2445 | 7985 | 9550 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9545 | 11750 | 13315 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -3900 | -2770 | 10300 | 11430 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -13350 | -11285 | -4360 | -3600 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 17115 | 10700 | 11460 |

Co-existence studies shows that

- The 3rd IMD generated by dual uplink of Band n5 + Band n25 may fall into own Rx of band n78.

- The 4th IMD generated by dual uplink of Band n5 + Band n78 may fall into own Rx of band n5.

- The 2nd IMD generated by dual uplink of Band n25 + Band n78 may fall into own Rx of band n25.

- The 4th IMD generated by dual uplink of Band n25 + Band n78 may fall into own Rx of band n78.

- The 5th IMD generated by dual uplink of Band n25 + Band n78 may fall into own Rx of band n78.

REFSENS requirements

The IMD issue specific to 3DL/2UL is that the 3rd IMD generated by dual uplink of Band n5 + Band n25 may fall into own Rx of band n78. As this IMD issue is similar to CA_n3A-n8A-n78A where low and high FDD band IMD3 falls into n78, the same MSD value as 16.1 dB is reused.

Table 5.1.4.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n25A-n78A | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A |  | N/A |
|  | n78 | 3560 | 10 | 50 | 3560 | 16.1 | TDD | IMD3 |

CA_n1-n77-n79

Operating bands for CA

Table 5.1.5.1-1: CA band combination of band n1+n77+n79

| NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.5.2-1: Supported bandwidths per CA band combination of band n1+n77+n79

| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A-n79A | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

UE co-existence studies

Based on co-existence studies of 2UL/2DL CA_n1-n77, CA_n1-n79, and CA_n77-n79, own Rx impact of the 3rd band is the followings

-	3rd, 4th and 5th  order IMD generated by dual uplink of CA_n1-n77 may fall into part of own band n79.

-	5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n77.

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

REFSENS requirements

-	3rd, 4th and 5th  order IMD generated by dual uplink of CA_n1-n77 may fall into part of own band n79.

=> There is no need to have MSD added because n77 and n79 are in sync.

-	5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n77.

=> There is no need to have MSD added because n77 and n79 are in sync.

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

=> The MSD values are shown in the following table. This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSDs due to IM4 and IM5. This is described in NOTE2 and NOTE1.

Table 5.1.5.4-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | [15.6] | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

CA_n1-n78-n79

Operating bands for CA

Table 5.1.6.1-1: CA band combination of band n1+n78+n79

| NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.6.2-1: Supported bandwidths per CA band combination of band n1+n78+n79

| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A | CA_n1A-n78A CA_n1A-n79A CA_n78A-n79A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

UE co-existence studies

Based on co-existence studies of 2UL/2DL CA_n1-n78, CA_n1-n79, and CA_n78-n79, own Rx impact of the 3rd band is the followings

-	3rd and 5th  order IMD generated by dual uplink of CA_n1-n78 may fall into part of own band n79.

-	5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n78.

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n78-n79 may fall into part of own band n1.

REFSENS requirements

-	3rd and 5th  order IMD generated by dual uplink of CA_n1-n78 may fall into part of own band n79.

=>Same MSD apply with DC_1_n78-n79 specified in TS 38.101-3. Note that MSD for 5th order IMD are not specified in TS 38.101-3.

This requirement only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability.

-	5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n78.

=>Same MSD apply with DC_1_n78-n79 specified in TS 38.101-3.

This requirement only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability.

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n78-n79 may fall into part of own band n1.

=>The MSD values are shown in the following table. This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSDs due to IM4 and IM5. This is described in NOTE2 and NOTE1.

Table 5.1.6.4-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n78A-n79A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD31,X1 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5X1 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |

CA_n2-n66-n77

Operating bands for CA

Table 5.1.7.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n66-n77 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.7.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n66A-n77A | CA_n2A-n66A CA_n66A-n77A CA_n2A-n77A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

Co-existence studies

For UE coexistence study of Band n2 + Band n66, Band n5 + Band n77, and Band n66 + band n77, the 2nd, 3rd, 4th and 5th order harmonics are already analyzed in 3DL/1UL WI. The 2nd, 3rd, 4th and 5th order intermodulation products are calculated and presented in Table 5.1.7.3-1, 5.1.4.3-2 and 5.1.7.3-3, respectively.

Table 5.1.7.3-1: Harmonic and IMD analysis for n2+n66

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | -200 | -70 | 3560 | 3690 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1920 | 2110 | 1510 | 1710 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5410 | 5600 | 5270 | 5470 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3770 | 4020 | 3220 | 3490 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7260 | 7510 | 6980 | 7250 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 140 | 400 | 7120 | 7380 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -5270 | -4930 | -5930 | -5620 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8690 | 9030 | 9110 | 9420 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -1640 | -1310 | -2310 | -1990 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8830 | 9160 | 8970 | 9290 |

Table 5.1.7.3-2: Harmonic and IMD analysis for n2+n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1390 | 2350 | 5150 | 6110 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -500 | 520 | 4690 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8020 | 8450 | 10310 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2430 | 7990 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9930 | 11750 | 14510 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -4700 | -2780 | 10300 | 12220 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -14950 | -11290 | -4340 | -3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18710 | 10700 | 11840 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -8900 | -6080 | 870 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16420 | 12150 | 14130 |

Table 5.1.7.3-2: Harmonic and IMD analysis for n66+n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1520 | 2490 | 5010 | 5980 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -780 | 260 | 4820 | 6690 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6720 | 7760 | 8310 | 10180 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 930 | 2040 | 8120 | 10890 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8430 | 9540 | 11610 | 14380 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -4980 | -3040 | 10020 | 11960 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -15090 | -11420 | -3820 | -2640 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14910 | 18580 | 10140 | 11320 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -9180 | -6340 | 1260 | 3270 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13320 | 16160 | 11730 | 13740 |

Co-existence studies shows that

- The 2nd and 4th order IMD generated by dual uplink of Band n2 + Band n66 may fall into own Rx of band n77.

- The 5th order IMD generated by dual uplink of Band n2 + Band n66 may fall into own Rx of band n66.

- The 2nd, 4th and 5th order IMD generated by dual uplink of Band n2 + Band n77 may fall into own Rx of band n2 and n66.

- The 4th and 5th order IMD generated by dual uplink of Band n2 + Band n77 band may fall into own Rx band of n77.

- The 2nd, 4th and 5th order IMD generated by dual uplink of Band n66 + Band n77 may fall into own Rx of band n2.

- The 2nd and 5th order IMD generated by dual uplink of Band n66 + Band n77 may fall into own Rx of band n66.

- The 4th and 5th order IMD generated by dual uplink of Band n66 + Band n77 may fall into own Rx of band n77.

REFSENS requirements

The IMD issue specific to 3DL/2UL for dual uplink of Band n2 + Band n66 is that the 2nd and 4th order IMD may fall into own Rx of band n77. As this IMD issue is the same as DC_2A_n66A-n78A, these MSD values are reused.

The IMD issue specific to 3DL/2UL for dual uplink of Band n2 + Band n77 is that the 2nd, 4th and 5th order IMD may fall into own Rx of band n66. IMD4 issues is the same as DC_2A_n66A-n78A, thus the MSD values are reused. For IMD2 and IMD5, DC_1A-3A_n78A MSD values are reused.

The IMD issue specific to 3DL/2UL for dual uplink of Band n66 + Band n77 is that the 2nd, 4th and 5th order IMD may fall into own Rx of band n2. As this IMD issue is the same as DC_2A_n66A-n78A, these MSD values are reused.

Table 5.1.7.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n2A-n66A-n77A | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 31.2 | FDD | IMD2 |
|  | n77 | 4010 | 10 | 50 | 4010 | N/A | TDD | N/A |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.3 | FDD | IMD4 |
|  | n77 | 3480 | 10 | 50 | 3480 | N/A | TDD | N/A |
| CA_n2A-n66A-n77A | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 2.8 | FDD | IMD5 |
|  | n77 | 3860 | 10 | 50 | 3860 | N/A | TDD | N/A |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |

CA_n3-n28-n41

5.1.8.1	Operating bands for CA

Table 5.1.8.1-1: CA band combination of band n3+n28+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.8.2-1: Supported bandwidths per CA band combination of band n3+n28+n41

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

UE co-existence studies

Co-existence studies of CA_n3-n28-n41 with 2UL have been covered in the constituent fall-back modes, it can get that

- IMD2 and IMD3 of band n3 UL and band n28 UL falling to band n41 DL

REFSENS requirements

Table 5.1.8.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value of DC_3A-41A_n28A.

Table 5.1.8.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |

CA_n3-n28-n78

Operating bands for CA

Table 5.1.9.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n28-n78 | n3 | 1710MHz | – | 1780MHz | 1805MHz | – | 1880MHz | FDD |
|  | n28 | 703MHz | – | 748MHz | 758MHz | – | 803MHz | FDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.9.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n78A | CA_n3A-n28A | n3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n28A | n3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

Co-existence studies

Co-existence studies of CA_n3-n28-n78 with 2UL are already covered in the constituent fall-back modes, it can get that:

- IMD5 of band n3 UL and band n28 UL falling to band n78 DL

REFSENS requirements

Table 5.1.9.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value for DC_3A_n28A-n78A.

Table 5.1.9.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n78A CA_n3A-n28A-n78(2A) | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 |

CA_n5-n66-n77

Operating bands for CA

Table 5.1.10.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n66-n77 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.10.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n66A-n77A | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n5A-n66A-n77(2A) | CA_n5A-n66A CA_n66A-n77A CA_n5A-n77A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Co-existence studies

For UE coexistence study of Band n5 + Band n66, Band n5 + Band n77, and Band n66 + band n77, the 2nd, 3rd, 4th and 5th order harmonics are already analyzed in 3DL/1UL WI, where 2nd and 3rd harmonic issues are identified. The 2nd, 3rd, 4th and 5th order intermodulation products are calculated and presented in Table 5.1.10.3-1, 5.1.10.3-2 and 5.1.10.3-3, respectively.

Table 5.1.10.3-1: Harmonic and IMD analysis for n5+n66

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 861 | 956 | 2534 | 2629 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -132 | -12 | 2571 | 2736 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3358 | 3478 | 4244 | 4409 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 692 | 837 | 4281 | 4516 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4182 | 4327 | 5954 | 6189 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1912 | -1722 | 5068 | 5258 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6296 | -5991 | -1686 | -1516 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 7664 | 7969 | 5006 | 5176 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -3692 | -3432 | 873 | 1088 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 6778 | 7038 | 5892 | 6107 |

Table 5.1.10.3-2: Harmonic and IMD analysis for n5+n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 2451 | 3376 | 4124 | 5049 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -2552 | -1602 | 5751 | 7576 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 4948 | 5898 | 7424 | 9249 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | -1728 | -753 | 9051 | 11776 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 5772 | 6747 | 10724 | 13449 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -6752 | -4902 | 8248 | 10098 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -15976 | -12351 | -96 | 904 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14024 | 17649 | 6596 | 7596 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -10952 | -8202 | 4053 | 5928 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11548 | 14298 | 9072 | 10947 |

Table 5.1.10.3-3: Harmonic and IMD analysis for n66+n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1520 | 2490 | 5010 | 5980 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -780 | 260 | 4820 | 6690 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6720 | 7760 | 8310 | 10180 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 930 | 2040 | 8120 | 10890 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8430 | 9540 | 11610 | 14380 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -4980 | -3040 | 10020 | 11960 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -15090 | -11420 | -3820 | -2640 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 14910 | 18580 | 10140 | 11320 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -9180 | -6340 | 1260 | 3270 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13320 | 16160 | 11730 | 13740 |

Co-existence studies shows that

- The 2nd and 5th order IMD generated by dual uplink of Band n5 + Band n66 may fall into own Rx of band n5.

- The 3rd, 4th, and 5th order IMD generated by dual uplink of Band n5 + Band n66 may fall into own Rx of band n77.

- The 4th and 5th order IMD generated by dual uplink of Band n5 + Band n77 may fall into own Rx of band n5.

- The 2nd order IMD generated by dual uplink of Band n5 + Band n77 may fall into own Rx of band n77.

- The 3rd order IMD generated by dual uplink of Band n5 + Band n77 may fall into own Rx of band n66.

- The 2nd and 5th order IMD generated by dual uplink of Band n66 + Band n77 may fall into own Rx of band n66.

- The 4th and 5th order IMD generated by dual uplink of Band n66 + Band n77 may fall into own Rx of band n77.

REFSENS requirements

The IMD issues specific to 3DL/2UL for dual uplink of Band n5 + Band n66 are that the 3rd, 4th and 5th order IMD may fall into own Rx of band n77. As this IMD issue is similar to CA_n5A-n25A-n78A for IMD3, DC_1A-8A_n79A for IMD4 and DC_1A_n8A-n78A for IMD5, where low and high FDD band IMD3 falls into C-band, those MSD values are is reused.

The IMD issues specific to 3DL/2UL for dual uplink of Band n5 + Band n77 are that the 3rd IMD may fall into own Rx of band n66. As this IMD issue is similar to DC_1A-8A_n77A, where a low FDD and band n77 IMD3 falls into n77, the MSD values are is reused.

Table 5.1.10.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n66A-n77A | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.1 | TDD | IMD3 |
| CA_n5A-n66A-n77A | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4192 | 10 | 50 | 4192 | 8.2 | TDD | IMD4 |
| CA_n5A-n66A-n77A | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n77 | 3590 | 10 | 50 | 3590 | 3.3 | TDD | IMD5 |
| CA_n5A-n66A-n77A | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 14.4 | FDD | IMD3 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

## CA_n8-n40-n41

### Operating bands for CA

Table 5.1.11.1-1: CA band combination of band n8+n40+n41

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8A-n40A-n41A | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### Channel bandwidths per operating band for CA

Table 5.1.11.2-1: Supported channel bandwidths per CA configuration for band n8+n40+n41

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

Actually, the co-existence studies for dual uplink operation of two bands, i.e. CA_n8A-n40A, CA_n8A-n41A and CA_n40A-n41A have been captured in TR38.716-02-00, where:

-    No IMD products produced by Band n40 and n41 that falling into the band n8 Rx.

-    No IMD products produced by Band n8 and n40 that falling into the band n41 Rx.

-    IMD5 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n40.

### REFSENS requirements

According to clause 5.1.11.3, IMD5 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n40. However, considering the requirements for TDD-TDD NR CA combinations of CA_n40-n41 are defined without simultaneous Rx/Tx capability in TS38.101-1, i.e. synchronous operation. Therefore it is no need to defined MSD requirements due to IMD5 issues, i.e. no specific REFSENS requirements for this combination in 3DL/2UL NR CA operation.

CA_n25-n41-n77

Operating bands for CA

Table 5.1.12.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n77 | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.12.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n25A-n41(2A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n25A-n41C-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD3 and IMD5 generated by UL n25-n41 might affect DL n77.

IMD5 generated by UL n25-n77 might affect DL n41.

IMD3 and IMD4 generated by UL n41-n77 might affect DL n25.

REFSENS requirements

CA_n25-n41-n77 need to have the same MSD requirements defined.

MSD values n77 are derived from DC_2A_n38A-n78A for IMD3 and DC_2A_n7A-n78A for IMD5.

MSD values n41 are derived from DC_3A-41A_n77A.

MSD values n25 are derived from DC_3A-7A_n77A for IMD3 and DC_2A-7A_n78A for IMD4.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.12.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n41A-n77A CA_n25A-n41(2A)-n77A CA_n25A-n41C-n77A | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2610 | 5 | 25 | 2610 | N/A | TDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3 |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n77 | 3180 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3475 | N/A | TDD | N/A |

CA_n25A-n66A-n77A

Operating bands for CA

Table 5.1.13.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n66-n77 | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.13.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD2 and IMD4 generated by UL n25-n66 might affect DL n77.

IMD2, IMD4 and IMD5 generated by UL n25-n77 might affect DL n66.

IMD2, IMD4 and IMD5 generated by UL n66-n77 might affect DL n25.

REFSENS requirements

CA_n25-n66-n77 need to have the same MSD requirements defined.

MSD values n66 are derived from DC_2A_n66A-n77A.

MSD values n25 are derived from DC_2A-66A_n78A.

MSD values n77 are derived from DC_2A_n66A-n78A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.13.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n66A-n77A | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |

CA_n25A-n71A-n77A

Operating bands for CA

Table 5.1.14.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n71-n77 | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.14.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD3, IMD4 and IMD5 generated by UL n25-n71 might affect DL n77.

UL n25-n77 IMD will not affect DL n71.

IMD3 and IMD4 generated by UL n71-n77 might affect DL n25.

REFSENS requirements

CA_n25-n71-n77 need to have the same MSD requirements defined.

MSD values n25 are derived from DC_2A-71A_n78A.

MSD values n77 are derived from DC_2A_n71A-n78A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.14.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n71A-n77A | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

CA_n41A-n66A-n77A

Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n66-n77 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.15.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41(2A)-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41C-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD3, IMD4 and IMD5 generated by UL n41-n66 might affect DL n77.

IMD4 generated by UL n41-n77 IMD might affect DL n66.

IMD5 generated by UL n66-n77 might affect DL n41.

REFSENS requirements

CA_n41-n66-n77 need to have the same MSD requirements defined.

MSD values n77 are derived from DC_66A_n7A-n78A.

MSD values n41 are derived from DC_3A-7A_n77A.

MSD values n66 are derived from DC_7A_n1A-n78A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.15.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41A-n66A-n77A CA_n41(2A)-n66A-n77A CA_n41C-n66A-n77A | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

CA_n41A-n71A-n77A

Operating bands for CA

Table 5.1.16.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n71-n77 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.16.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41(2A)-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41C-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD2, IMD3, IMD4 and IMD5 generated by UL n41-n71 might affect DL n77.

IMD2 generated by UL n41-n77 IMD might affect DL n71.

IMD2 and IMD3 generated by UL n71-n77 might affect DL n41.

REFSENS requirements

CA_n41-n71-n77 need to have the same MSD requirements defined.

MSD values n41 are derived from DC_71A_n38A-n78A for IMD2 and DC_8A_n41A-n79A for IMD3.

MSD values n77 are derived from DC_71A_n38A-n78A for IMD2 and DC_8A_n41A-n79A for IMD3.

MSD values n71 are derived from DC_41A_n28A-n77A

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.16.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41A-n71A-n77A CA_n41(2A)-n71A-n77A CA_n41C-n71A-n77A | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

CA_n66A-n71A-n77A

Operating bands for CA

Table 5.1.17.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n66-n71-n77 | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.1.17.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

UE co-existence studies

IMD3, IMD4 and IMD5 generated by UL n66-n71 might affect DL n77.

IMD3 generated by UL n66-n77 IMD might affect DL n71.

IMD3 and IMD4 generated by UL n71-n77 might affect DL n66.

REFSENS requirements

CA_n66-n71-n77 need to have the same MSD requirements defined.

MSD values n66 are derived from DC_71A_n66A-n78A.

MSD values n77 are derived from DC_28A_n3A-n77A.

MSD values n71 are derived from DC_3A-28A_n77A

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.17.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66A-n71A-n77A | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Inter-band within FR2

CA_nX-nY-nZ

Operating bands for CA

Table 5.2.x.1-1: CA band combination of band nX+nY+nZ

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| nX |  | – |  |  | – |  |  |
| nY |  | – |  |  | – |  |  |
| nZ |  | – |  |  | – |  |  |

Channel bandwidths per operating band for CA

Table 5.2.x.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

< Edtor's note: Align with the table format in TS38.101-2 later>

UE co-existence studies

< Edtor's note: Text will be added on whether there are IMD issues due to dual uplink operation falling into the DL of the third band. For example: for CA_nXA-nYA-nZA with 2UL CA_nXA-nYA, intermodulation due to Band nX and Band nY falling into Band nZ shall be verified.>

REFSENS requirements

< Editor's note: Text will be added on reference sensitivity exceptions if IMD issue due to dual uplink operation falling into DL of the third band are identified. >

Inter-band between FR1 and FR2

CA_n1-n77-n257

Operating bands for CA

Table 5.3.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n77-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A-n257A | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n1A-n77A-n257G | CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n77A-n257A CA_n77A-n257G | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A-n257H | CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n77A-n257I | CA_n1A-n77A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n1-n77, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

REFSENS requirements

As mentioned in 5.3.1.3, MSD analysis can be omitted and there is no need to specify additional MSD requirement for the CA combination.

CA_n1-n78-n257

Operating bands for CA

Table 5.3.2.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.3.2.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n257A | CA_n1A-n78A CA_n1A-n257A CA_n78A-n257A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n1A-n78A-n257G | CA_n1A-n78A CA_n1A-n257A CA_n1A-n257G CA_n78A-n257A CA_n78A-n257G | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A-n257H | CA_n1A-n78A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78A-n257I | CA_n1A-n78A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n1-n78, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

REFSENS requirements

As mentioned in 5.3.2.3, MSD analysis can be omitted and there is no need to specify additional MSD requirement for the CA combination.

CA_n1-n79-n257

Operating bands for CA

Table 5.3.3.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n79-n257 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4200 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.3.3.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n79A-n257A | CA_n1A-n79A CA_n1A-n257A CA_n79A-n257A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n1A-n79A-n257G | CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n79A-n257A CA_n79A-n257G | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n79A-n257H | CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n79A-n257I | CA_n1A-n79A CA_n1A-n257A CA_n1A-n257G CA_n1A-n257H CA_n1A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n1-n79, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

REFSENS requirements

As mentioned in 5.3.3.3, MSD analysis can be omitted and there is no need to specify additional MSD requirement for the CA combination.

CA_n77-n79-n257

Operating bands for CA

Table 5.3.4.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n77-n79-n257 | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4200 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.3.4.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77A-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n77A-n79A-n257G | CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257H | CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257I | CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n77-n79, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

REFSENS requirements

As mentioned in 5.3.4.3, MSD analysis can be omitted and there is no need to specify additional MSD requirement for the CA combination.

CA_n78-n79-n257

Operating bands for CA

Table 5.3.5.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n78-n79-n257 | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4200 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

Channel bandwidths per operating band for CA

Table 5.3.5.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n79A-n257A | CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n78A-n79A-n257G | CA_n78A-n79A CA_n78A-n257A CA_n78A-n257G CA_n79A-n257A CA_n79A-n257G | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257H | CA_n78A-n79A CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257I | CA_n78A-n79A CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

UE co-existence studies

Co-existence studies can be omitted because harmonic and intermodulation impact between FR1 bands have been already studied for CA_n78-n79, and harmonic and intermodulation impact between FR1 bands and FR2 band are negligible.

REFSENS requirements

As mentioned in 5.3.5.3, MSD analysis can be omitted and there is no need to specify additional MSD requirement for the CA combination.

Dual Connectivity with 3 bands DL: Specific Band Combination Part

6.X	DC_nX-nY-nZ

< Editor's note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means contribution only to add pure NR DC texts is not allowned.>

6.x.1   Configurations for DC_nX-nY-nZ

Table 6.x.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA-nZA | DC_nXA-nYA DC_nXA-nZA ... |

- Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2010791 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | RAN4#96-e | R4-2010793 |  |  |  | The following approved TPs in RAN4 96-e meeting are included: - R4-2011677 TP to TR 38.717-03-02: CA_n5-n25-n66, Nokia, Bell Mobility - R4-2010531 TP to TR 38.717-03-02: CA_n5-n25-n78, Nokia, Bell Mobility - R4-2010644 TP for TR38.717-03-02_CA_n39A-n40A-n79A, ZTE Corporation - R4-2010645 TP for TR38.717-03-02_CA_n39A-n40A-n41A, ZTE Corporation - R4-2009690 TP for CA_n1-n77-n257 3UL/2DL for TR38.717-03-02, NTT DOCOMO, INC. - R4-2009691 TP for CA_n1-n78-n257 3UL/2DL for TR38.717-03-02, NTT DOCOMO, INC. - R4-2009692 TP for CA_n1-n79-n257 3UL/2DL for TR38.717-03-02, NTT DOCOMO, INC. - R4-2009693 TP for CA_n77-n79-n257 3UL/2DL for TR38.717-03-02, NTT DOCOMO, INC. - R4-2009694 TP for CA_n78-n79-n257 3UL/2DL for TR38.717-03-02, NTT DOCOMO, INC. | 0.1.0 |
| 2020-11 | RAN4#97-e | R4-2015185 |  |  |  | The following completed band combinations of inter-band CA for 3 bands DL with 2 bands UL are added from RAN4 #97-e: - R4-2016755 TP for TR 38.717-03-02 CA_n3-n28-n41 - R4-2016756 TP for TR 38.717-03-02 CA_n3-n28-n78 - R4-2016757 TP for CA 3DL2UL n1-n77-n79 for TR 38.717-03-02 - R4-2016758 TP for CA 3DL2UL n1-n78-n79 for TR 38.717-03-02 - R4-2015052 TP for TR38.717-03-02_ CA_n8A-n40A-n41A - R4-2015080 TP to TR 38.717-03-02: CA_n5-n66-n77 - R4-2015081 TP to TR 38.717-03-02: CA_n2-n66-n77 - R4-2016759 TP to add CA_n25A-n41A-n77A, CA_n25A-n41(2A)-n77A, CA_n25A-n41C-n77A - R4-2016760 TP to add CA_n25A-n66A-n77A - R4-2016761 TP to add CA_n25A-n71A-n77A - R4-2016762 TP to add CA_n41A-n66A-n77A, CA_n41(2A)-n66A-n77A, CA_n41C-n66A-n77A - R4-2016763 TP to add CA_n41A-n71A-n77A, CA_n41(2A)-n71A-n77A, CA_n41C-n71A-n77A - R4-2016764 TP to add CA_n66A-n71A-n77A | 0.2.0 |