3GPP TSG-RAN Meeting #97-e	R4-2016550

# Online Meeting, November 2 – 13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7004 | rev |  | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test parameters for FDD and TDD intra-frequency RSRP for Cat-M1 UE in CEModeA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-11-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add cell timing information to RSRP intra-frequecy tests for UE Cat M1 in CE ModeA for consistency with other similar tests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specify cell 2 timing offset with respect to cell 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test configuration for RSRP intra-frequency tests for UE Cat-M1 in CE ModeA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.1.52, A.9.1.52A, A.9.1.53, A.9.1.53A, A.9.1.54 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Rel-15 shadow of R4-2017066 (CR 7002) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

### A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.52.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.52.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.52.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.52.2-1: FD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.20 FDD | - | R.20 FDD | - | R.20 FDD | - |
| PDSCH allocation |  |  | Follows R.20 FDD | - | Follows R.20 FDD | - | Follows R.20 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.16 FDD |  | R.16 FDD |  | R.16 FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B Note 6 |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. Note 6: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |  |

#### A.9.1.52.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

#### A.9.1.52A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for FD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.52A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.52A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.52A.2-1: FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.28 FDD | - | R.28 FDD | - | R.28 FDD | - |
| PDSCH allocation |  |  | Follows R.28 FDD | - | Follows R.28 FDD | - | Follows R.28 FDD | - |
| MPDCCH Reference measurement channel |  |  | R.24 FDD |  | R.24 FDD |  | R.24 FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -103 |  | -83 |  | -109.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -97 | -102 | -77 | -82 | -106.5 | -110.5 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.28 |  | -50.28 |  | -78.94 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.52A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.53.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.53.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.53.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.53.2-1: HD-FDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.10 HD-FDD | - | R.10 HD-FDD | - | R.10 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - | Follows R.10 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.6 HD-FDD |  | R.6 HD-FDD |  | R.6 HD-FDD |  |
| OCNG Patterns |  |  | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD | OP.21 FDD | OP.6 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -114 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -113 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -112.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.5 | -6 | 2.5 | -6 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands FDD-M1_B |  |  |  |  |  | -112.5 | -116.5 |
|  | Bands FDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands FDD-M1_D |  |  |  |  |  | -111.5 | -115.5 |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -111 | -115 |
|  | Bands FDD-M1_G |  |  |  |  |  | -110 | -114 |
|  | Bands FDD-M1_H |  |  |  |  |  | -109.5 | -113.5 |
| IoNote3 | Bands FDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands FDD-M1_B |  |  |  |  |  | -81.93 |  |
|  | Bands FDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands FDD-M1_D |  |  |  |  |  | -80.93 |  |
|  | Bands FDD-M1_E, FDD-M1_F Note 4 |  |  |  |  |  | -80.43 |  |
|  | Bands FDD-M1_G |  |  |  |  |  | -79.43 |  |
|  | Bands FDD-M1_H |  |  |  |  |  | -78.93 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.53.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

#### A.9.1.53A.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for HD-FDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.53A.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.53A.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.53A.2-1: HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 5 |  | 5 |  | 5 |  |
| Measurement bandwidth |  |  | 10—15 |  | 10—15 |  | 10—15 |  |
| PDSCH Reference measurement channel |  |  | R.18 HD-FDD | - | R.18 HD-FDD | - | R.18 HD-FDD | - |
| PDSCH allocation |  |  | Follows R.18 HD-FDD | - | Follows R.18 HD-FDD | - | Follows R.18 HD-FDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 HD-FDD |  | R.14 HD-FDD |  | R.14 HD-FDD |  |
| OCNG Patterns |  |  | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD | OP.22 FDD | OP.19 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD-M1_N | dBm/15 kHz | -103 |  | -83 |  | -109.5 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Note3 |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | 0.46 | -5.76 |
| RSRPNote3 | Bands FDD-M1_N | dBm/15 kHz | -97 | -102 | -77 | -82 | -106.5 | -110.5 |
| IoNote3 | Bands FDD-M1_N | dBm/4.5 MHz | -70.28 |  | -50.27 |  | -78.94 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | ms | - | 3 | - | 3 | - | 3 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.53A.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

### A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA

#### A.9.1.54.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.21.1 and 9.1.21.2 for TDD intra frequency RSRP measurements for Cat-M1 UE in CEModeA.

#### A.9.1.54.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.54.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.54.2-1: TDD RSRP Intra frequency test parameters for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel |  |  | R.16 TDD | - | R.16 TDD | - | R.16 TDD | - |
| PDSCH allocation |  |  | Follows R.16 TDD | - | Follows R.16 TDD | - | Follows R.16 TDD | - |
| MPDCCH Reference measurement channel |  |  | R.14 TDD |  | R.14 TDD |  | R.14 TDD |  |
| OCNG Patterns |  |  | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD | OP.11 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD-M1_A | dBm/15 kHz | -106 |  | -86 |  | -116 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -115 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -114 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD-M1_A | dBm/15 kHz | -100 | -105 | -80 | -85 | -113 | -117 |
|  | Bands TDD-M1_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD-M1_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD-M1_A | dBm/9 MHz | -70.27 |  | -50.27 |  | -82.43 |  |
|  | Bands TDD-M1_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD-M1_E |  |  |  |  |  | -80.43 |  |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x1 |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 |  | s | - | 3 | - | 3 | - | 3 |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.54.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.21.1 and 9.1.21.2.

# < End of change >