3GPP TSG-RAN WG4 Meeting #97-e                                R4-2016726

Online, 2nd Nov-13th Nov, 2020

Source:	KDDI

Title:	TP for TR 37.717-11-21: DC_18A_n3A-n41A

Agenda item:			10.7.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_18A_n3A-n41A according to the request in [1].

# 2. Reference

- RP-201477, New WID on Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	DC_18_n3-n41

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n3A-n41A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Based on the co-existence studies of DC_18A-n3A and DC_18A-n41, impact on own Rx of 3rd band is as follows.

- 2nd and 3rd order IMD generated by dual uplink of Band 18 + Band n3 may also fall into own Rx of band n41.

- 2nd order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n3.

### 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n3A-n41A are shown in the following tables.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0.3 |
|  | n3 | 0.5 |
|  | n41 | 0.3 |

Table 6.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0 |
|  | n3 | 0 |
|  | n41 | 0 |

### 6.x.5	MSD

Table 6.x.5-1 shows the required MSD:

Table 6.x.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 5 | 25 | 2540 | 5 | 29.41 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2655 | 5 | 25 | 2655 | 5 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 28.2 |
| Note1: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>