3GPP TSG-RAN WG4 Meeting #96-e                                                             R4-2016755

Online, 2nd-13th Nov, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n3A-n28A-n41A

Agenda item:			10.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n3A-n28A-n41A according to the request in [1].

# 2. Reference

- RP-201541, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.x	CA_n3-n28-n41

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: CA band combination of band n3+n28+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported bandwidths per CA band combination of band n3+n28+n41

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | CA_n3A-n28A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

#### 5.1.x.3	UE co-existence studies

Co-existence studies of CA_n3-n28-n41 with 2UL have been covered in the constituent fall-back modes, it can get that

- IMD2 and IMD3 of band n3 UL and band n28 UL falling to band n41 DL

#### 5.1.x.4	REFSENS requirements

Table 5.1.x.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value of DC_3A-41A_n28A.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |

<End of Text Proposal>