3GPP TSG-WG4 Meeting #97-e	R4-2016811

# Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0368 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General corrections for V2X sections in 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some NR V2X section numbers have been denoted with suffix C. It was agreed that all NR V2X sections numbers will be denoted with suffix E. Also,in some instances the cross-referencing between NR V2X sections in 38.101-3 and 38.101-1 is not correct and needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All NR V2X sections have been denoted with section numbers with suffix E. the cross-referencing between NR V2X sections in 38.101-3 and 38.101-1 has been fixed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X sections will not have consistent section numbering. Also, the cross referencing between NR V2X sections in 38.101-3 and 38.101-1 would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E, 5.2E.1, 5.2E.2, 6.2E.1.3, 6.2E.2, 6.2E.3, 6.2E.4, 6.4E, 6.4E.1, 6.4E.2, 6.5E, 6.5E.1, 6.5E.2, 6.5E.3, 6.5E.4, 7.3E, 7.3E.1, 7.3E.2, 7.4E, 7.5E, 7.6E, 7.7E, 7.8E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 5.2B	Operating bands for DC

### 5.2B.1	General

The operating bands are specified in clause 5.5B for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured.

### 5.2B.2	Void

### 5.2B.3	Void

### 5.2B.4	Void

### 5.2B.5	Void

### 5.2B.6	Void

### 5.2B.7	Void

## 5.2E	Operating bands for V2X

### 5.2E.1	Intra-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink or sidelink on the operating bands combinations listed in Table 5.2E.1-1.

Table 5.2E.1-1: Intra-band V2X operating bands

| V2X con-current operating band | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_47_n471 | 47 | PC5 |
|  | n47 | PC5 |
| NOTE 1: Only single switched SL is supported. |  |  |

### 5.2E.2	Inter-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1: Inter-band con-current V2X operating bands

| E-UTRA-NR V2X Band Combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_20_n38 | 20 | Uu |
|  | n38 | PC5 |
| V2X_ n71_47 | 47 | PC5 |
|  | n71 | Uu |
| V2X_n71_(n) 471 | 47 | PC5 |
|  | n47 | PC5 |
|  | n71 | Uu |
| NOTE 1: Only single switched SL in ITS band is supported. |  |  |

## 5.3	UE Channel bandwidth

## 5.3A	UE Channel bandwidth for CA

### 5.3A.1	Inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class as specified in clause 5.3A.5 of TS 38.101-1 [2] and clause 5.3A.4 of TS 38.101-2 [3] independently.

## <End of Changes>

## <Start of Changes>

#### 6.2E.1.3	UE maximum output power for Inter-band V2X

For the inter-band con-current NR V2X operation, the maximum output power is specified in Table 6.2E.1.3-1. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.3-1: Con-current V2X UE Power Class

| V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_20A_n38A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_n71A_47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 6.2E.2	UE maximum output power reduction for V2X

#### 6.2E.2.1	UE maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, maximum output power reduction specified in clause 6.2.3G [4] and in clause 6.2E.2 [2] apply, respectively.

#### 6.2E.2.2	UE maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in subclause 6.2.3 of TS 36.101 [4] apply for E-UTRA Uu operation in licensed band, and the MPR requirements in subclause 6.2E.2 of TS 38.101-1 [2] apply for NR sidelink operation in Band n47.

### 6.2E.3	UE additional maximum output power reduction for V2X

#### 6.2E.3.1	UE additional maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, additional maximum output power reduction specified in clause 6.2.4G [4] and in clause 6.2E.3 [2] apply, respectively.

#### 6.2E.3.2	UE additional maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in subclause 6.2.3 of TS 36.101 [4] apply for E-UTRA Uu operation in licensed band, and the A-MPR requirements in in subclause 6.2E.3 of TS 38.101-1 [2] apply for NR sidelink operation in Band n47.

### 6.2E.4	Configured output power for V2X

#### 6.2E.4.1	UE configured output power for Intra-band V2X

For intra-band V2X operating UE, each UE configured output power specified in clause 6.2.5G [4] and in clause 6.2E.4 [2] apply, respectively.

#### 6.2E.4.2	UE configured output power for Inter-band V2X

When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,Uu and PCMAX,c,V2X for the configured E-UTRA or NR uplink carrier and the configured NR V2X SL or E-UTRA V2X SL carrier, respectively, and its total configured maximum output power PCMAX,c. The  DTIB,V2X of PCMAX,c,Uu is specified in Table 6.2E.4.2-1.

The configured maximum output power PCMAX c,Uu(p) in subframe p for the configured E-UTRA or NR uplink carrier shall be set within the bounds:

PCMAX_L,c, Uu (p) ≤  PCMAX,c, Uu (p) ≤  PCMAX_H,c, Uu (p)

where PCMAX_L,c,Uu and PCMAX_H,c, Uu are the limits for a serving cell c as specified in subclause 6.2.5 TS 36.101 [4] or 6.2.4 TS 38.101-1 [2].

The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR or E-UTRA V2X SL carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in subclause 6.2E.4 of TS 38.101-1 [2] or 6.2.5G or TS 36.101 [5].

The total UE configured maximum output power PCMAX (p,q) in a subframe p of E-UTRA uplink carrier and a slot q of NR V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,Uu (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c, Uu (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,Uu are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,Uu (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA uplink and NR V2X carriers is

PUMAX = 10 log10 [pUMAX,c,Uu + pUMAX,c,V2X],

where pUMAX,c,Uu  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier or NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X SL carrier or E-UTRA V2X SLcarrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2C.4-1. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

Table 6.2E.4.2-1: ΔTIB,V2X for inter-band con-current V2X operation (two bands)

| V2X con-current operating band Configuration | Operating Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_20A_n38A | 20 | [0.21] |
| Note 1: The ΔTIB,V2X is applied on top of ΔTIB,c of DC_20A_n38A when use harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |

## 6.3	Output power dynamics

## <End of Changes>

## <Start of Changes>

#### 6.4B.2.5	Transmit modulation quality for inter-band EN-DC including both FR1 and FR2

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.2 and 6.5.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.4.2 of TS 38.101-1 [2] and clause 6.4.2, 6.4A.2 and 6.4D.2 of TS 38.101-2 [3] apply.

## 6.4E	Transmit signal quality for V2X operation in FR1

### 6.4E.1	Frequency error for V2X

For intra-band V2X operating UE, the requirement shall apply on each component carrier as defined in clause 6.5.1G in TS 36.101 [4] and in clause 6.4E.1 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.5.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4E.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

#### 6.4E.2	Transmit modulation quality for V2X

#### 6.4E.2.1	Transmit modulation quality for Intra-band V2X

##### 6.4E.2.2.1	Error Vector Magnitude

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.1 in TS 36.101 [4] and in clause 6.4E.2.2 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.5.2 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4E.2.2 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

##### 6.4E.2.2.2	Carrier leakage

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.2 in TS 36.101 [4] and in clause 6.4E.2.3 in TS 38.101-1 [2], respectively.

##### 6.4E.2.2.3	In-band emissions

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.3 in TS 36.101 [4] and in clause 6.4E.2.4 in TS 38.101-1 [2], respectively.

#### 6.4E.2.2	Transmit modulation quality for Inter-band V2X

For inter-band V2X with transmission assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [4] and in clause 6.4.2 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.2A in TS 36.101 [4] apply for those component carriers.

## 6.5	Void

## 6.5A	Output RF spectrum emissions for CA

## <End of Changes>

## <Start of Changes>

#### 6.5B.5.5	Inter-band EN-DC including both FR1 and FR2

Transmit intermodulation requirement specified in clauses 6.7.1 and 6.7.1A of TS 36.101 [4] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

## 6.5E	Output RF spectrum emissions for V2X operation in FR1

### 6.5E.1	Occupied bandwidth

#### 6.5E.1.1	Intra-band V2X

For intra-band V2X, the occupied bandwidth specified in clause 6.6.1G in TS 36.101 [4] and specified in clause 6.5E.1 in TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5E.1.2	inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.6.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.5E.2	Out-of-band emissions

#### 6.5E.2.1	Intra-band V2X

For intra-band V2X, out-of-band emissions specified in clause 6.6.2G in TS 36.101 [4] and specified in clause 6.5E.2 in TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5E.2.2	Inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the general SEM/additional SEM requirements and ACLR specified in subclause 6.6.2 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the general SEM/additional SEM and ACLR requirements specified in subclause 6.5E.2 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.5E.3	Spurious emissions

#### 6.5E.3.1	Intra-band V2X

##### 6.5E.3.1.1	General spurious emissions

For intra-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

##### 6.5E.3.1.2	Spurious emission band UE co-existence

For intra-band V2X, the spurious emissions band UE co-existence requirements specified in clause 6.6.3.2 of TS 36.101 [4] and clause 6.5.3.2 of TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5E.3.2	Inter-band V2X con-current operation

##### 6.5E.3.2.1	General spurious emissions

For inter-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

##### 6.5E.3.2.2	Spurious emission band UE co-existence

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.2.2-1 apply for the corresponding inter-band con-current operation with transmission assigned to both E-UTRA uplink in licensed band and sidelink in NR Band n47.

Table 6.5E.3.2.2-1: Requirements for inter-band con-current V2X operation

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_20_n38 | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| V2X_n71_47_ | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_XX is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

### 6.5E.4	Transmit intermodulation

#### 6.5E.4.1	Intra-band V2X

For intra-band V2X, transmit intermodulation requirements specified in clause 6.7.1G of TS 36.101 [4] and clause 6.5E.4 of TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5E.4.2	Inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.7.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.4 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

## 6.6B	Beam correspondence for DC

6.6B.1	Void

## <End of Changes>

## <Start of Changes>

##### 7.3B.3.5.5	ΔRIB,c for EN-DC six bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.5-1, ΔRIB,c for constituent FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

## 7.3E	Reference sensitivity for V2X operation in FR1

### 7.3E.1	General

For V2X operation, REFSENS requirements defined in TS 38.101-1 [2] and TS 36.101 [4] apply to all downlink bands of V2X configurations listed in clause 5.5E, unless sensitivity degradation exception is allowed in this clause of this specification, clause 7.3E in TS 38.101-1 [2] or clause 7.3.1G in TS 36.101 [4].

### 7.3E.2	Reference sensitivity for V2X

#### 7.3E.2.1	Intra-band contiguous V2X

For intra-band contiguous V2X listed in Table 5.5E.2-1, the each REFSENS requirements specified in clause 7.3.1G of TS 36.101 [4] and clause 7.3E.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

#### 7.3E.2.2	Intra-band non-contiguous V2X

For intra-band non-contiguous V2X listed in Table 5.5E.3-1, the each REFSENS requirements specified in clause 7.3.1G of TS 36.101 [4] and clause 7.3E.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

#### 7.3E.2.3	Inter-band V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with E-UTRA uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2. Also the E-UTRA downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.

Table 7.3E.2.3-1 is proposed the reference sensitivity requirements for inter-band con-current V2X UE reception without any self-interference problem.

Table 7.3E.2.3-1: Reference sensitivity for V2X QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Band | E-UTRA or NR V2X band (Uu) | E-UTRA or NR Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) |  | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | 20 | 20 | 15 | -97 | -94 | -91.2 |  | -90 |  |  | FDD |
|  |  | n38 | 15 |  | 96.5 |  |  | -93.2 | -91.4 | -90.1 | HD |
|  |  |  | 30 |  | -96.1 |  |  | -93.4 | -91.7 | -90.2 |  |
|  |  |  | 60 |  | -96.9 |  |  | -93.1 | -91.9 | -90.4 |  |
| n71 | 47 | 47 | 15 |  | -90.4 |  |  | -87.5 |  |  | FDD |  |
|  |  | n71 | 15 | -97.2 | -94.0 |  | -91.6 | -86.0 |  |  | HD |  |
|  |  |  | 30 |  | -94.3 |  | -91.9 | -87.4 |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |

Table 7.3E.2.3-2 is specified the additional Rx insertion loss according to harmonic trap filter to reduce the harmonic problem based on specific self desense analysis according to specific NR V2X inter-band con-current operation.

Table 7.3E.2.3-2: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | V2X operating Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_20_n38 | 20 | [0.21] |
| Note 1: The ΔRIB,V2X is applied on top of ΔRIB,c of DC_20_n38 when use harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |

The reference sensitivity is defined to be met with Uu uplink assigned to one band (that differs from the V2X operating band) and all E-UTRA downlink carriers active. The Uu uplink resource blocks as defined in Table 7.3E.2.3-3 and Table 7.3E.2.3-4 shall be located as close as possible to V2X operating band but confined within the transmission bandwidth configuration for the channel.

Table 7.3E.2.3-3: Uplink configuration for reference sensitivity of V2X UE (PC5)

| Inter-band V2X con-current band configuration |  | E-UTRA or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | Uu band (Uu) | UL band | Channel Bandwidth (MHz) | SCS (kHz) | NRB | Duplex Mode |
| n38 | 20 | 20 | 10 | 15 | 50 | FDD |
| 47 | n71 | n71 | 10 | 15 | 52 | FDD |
|  |  |  |  | 30 | 24 |  |

Table 7.3E.2.3-4: SL Tx configuration for reference sensitivity of V2X UE (Uu)

| Inter-band V2X con-current band configuration |  | E-UTRA or NR UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X band (PC5) | Uu band (Uu) | V2X band (PC5) | Channel Bandwidth (MHz) | SCS (kHz) | NRB | Duplex Mode |
| n38 | 20 | n38 | 10 | 15 | 50 | HD |
|  |  |  |  | 30 | 24 |  |
|  |  |  |  | 60 | 10 |  |
| 47 | n71 | 47 | 10 | 15 | 50 | HD |

#### 7.3E.2.3.1	Reference sensitivity exception due to UL harmonic problem

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the inter-band con-current V2X UE. Reference sensitivity exceptions (MSD) for the victim band (high) are specified in Table 7.3E.2.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3E.2.3.1-2.

Table 7.3E.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for inter-band con-current operation

| V2X inter-band con-current band combinations | Operating Bands / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X_20_n38 | UL band | SL operation | 10 MHz (dB) | 20 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) |
|  | 20 | n38 | [12.0] | [9.0] | [7.1] | [6.0] |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the sidelink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 3: The MSD level applied to all supported SCSs in victim band. |  |  |  |  |  |  |

Table 7.3E.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for inter-band con-current V2X in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| UL band | SL operation | 10 MHz (LCRB) | 20 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) |
| 20 | n38 | [25] | [50] | [50] | [50] |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies |  |  |  |  |  |

## 7.4	Void

## 7.4A	Maximum input level for CA

For inter-band NR CA between FR1 and FR2, the maximum input level specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## <End of Changes>

## <Start of Changes>

### 7.4B.5	Inter-band EN-DC including both FR1 and FR2

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.4, 7.4A and 7.4B of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## 7.4E	Maximum input level for V2X operation in FR1

For intra-band V2X UE, the maximum input requirements specified in clause 7.4.1G of TS 36.101 [4] and clause 7.4E.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.4E of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.4.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.5	Void

## 7.5A	Adjacent channel selectivity for CA

For inter-band NR CA between FR1 and FR2, the adjacent channel selectivity specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## <End of Changes>

## <Start of Changes>

### 7.5B.5	Inter-band EN-DC including both FR1 and FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.5, 7.5A and 7.5Bof TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## 7.5E	Adjacent channel selectivity for V2X operation in FR1

For intra-band V2X operation, the adjacent channel selectivity specified in clause 7.5.1G in TS 36.101 [4] and specified in clause 7.5E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.5E of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.5.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.6	Void

## <End of Changes>

## <Start of Changes>

#### 7.6B.4.5	Inter-band EN-DC including both FR1 and FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

## 7.6E	Blocking characteristics for V2X in FR1

For intra-band V2X operation, the blocking charateristics specified in clause 7.6.1.1G in TS 36.101 [4] and specified in clause 7.6E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band con-current NR V2X operation, the in-band blocking and out of band blocking requirement specified in clause 7.6E in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.6 in TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

No narrow band blocking requirement applied for NR V2X carrier.

## 7.7	Void

## <End of Changes>

## <Start of Changes>

### 7.7B.5	Inter-band EN-DC including both FR1 and FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (three bands) in clause 5.5B.6.2 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

## 7.7E	Spurious response for V2X in FR1

For intra-band V2X operation, the spurious response specified in clause 7.7.1G in TS 36.101 [4] and specified in clause 7.7E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.7E of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.7.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.8	Void

## 7.8A	Intermodulation characteristics for CA

For inter-band NR CA between FR1 and FR2, the intermodulation characteristics specified in TS 38.101-1 [2] apply for FR1.

## <End of Changes>

## <Start of Changes>

#### 7.8B.2.5	Inter-band EN-DC including both FR1 and FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

## 7.8E	Intermodulation characteristics for V2X operation in FR1

For intra-band V2X operation, the intermodulation characteristics specified in clause 7.8.1G in TS 36.101 [4] and specified in clause 7.8E in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band NR V2X con-current operation, the wideband inter-modulation requirement specified in clause 7.8E in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.8.1 in TS 36.101 [4] shall apply on E-UTRA downlink reception in licensed band while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

## 7.9	Void

## 7.9A	Spurious emissions for CA

For inter-band NR CA between FR1 and FR2, the spurious emission specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## <End of Changes>