3GPP TSG-RAN4 Meeting #97-e	R4-2017056

# Electronic Meeting, 2nd – 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1266 | rev | 1 | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-RAT SFTD measurement test cases R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To correct wrong Io calculations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Io calculations are corrected. Typos are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spce is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Following changes are made acdording to the comments received during 1st round: - "Proposed change affects" field on the coversheet is updated; - Io for Config 3 and 6 are updated to -58.50 dB. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.8.4	Measurement procedure

### A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX

A.8.4.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.8.4.1.1.1-1 below. Test parameters and cell-specific parameters for the NR cell are provided in Tables A.8.4.1.1.1-2 and A.8.4.1.1.1-3 below, respectively. Cell-specific parameters for the E-UTRA cell are provided in Table A.3.7.2.1-1 in clause A.3.7.2.1.

Table A.8.4.1.1.1-1: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.4.1.1.1-2: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SMTC-SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,3,4 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 5,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

Table A.8.4.1.1.1-3: Cell specific test parameters for Cell 2 in inter-RAT SFTD measurement delay test

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  | Config 1,4 | SMTC.2 |
|  |  | Config 2,3,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.50 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

A.8.4.1.1.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at TRRC_procedure_delay + Tmeasure_SFTD1 after the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX

A.8.4.1.2.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.8.4.1.2.1-1 below. Test parameters are provided in Tables A.8.4.1.2.1-2 below. Cell-specific parameters for the E-UTRA and NR cells are provided in Table A.3.7.2.1-1 in clause A.3.7.2.1, and Table A.8.4.1.1.1-3 in clause A.8.4.1.1.1, respectively.

Table A.8.4.1.2.1-1: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test in DRX

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.4.1.2.1-2: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SMTC-SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | DRX.4 |  | DRX configuration as specified in clause A.3.3.4 |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,3,4 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 5,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

A.8.4.1.2.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at the earliest DRX activity time following upon  TRRC_procedure_delay + Tmeasure_SFTD1 from the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

<End of modified section 1>