3GPP TSG-RAN WG4 Meeting #97-e	R4-2014005

Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	New gap patterns for PRS measurements

Agenda item:	7.7.2.4

Document for:	Discussion

# 1	Introduction

In RAN4 95-e, the new gap patterns for PRS measurements have been discussed. Companies’ views were summarized in the discussion summary [1], the content copied below:

Issue 1-2-3: If existing CSSF is used then rules for new MG sharing between RRM and PRS measurements? Candidate options: - Option 1: - Equal split of gaps between PRS and all RRM measurements - Option 2: - NR positioning measurements are counted together with inter-frequency and inter-RAT measurement objects and - gap sharing of NR positioning should be based on the same principle of LTE-PRS, i.e., scarce PRS (corresponding to long periodicities) should be prioritized over other candidates for measurement in the same gap instance. - Option 3: - If PRS periodicity >= 80 ms, then PRS measurement is prioritized over other candidates for measurement in the same gap instance; - Otherwise, Equal split of gaps between PRS and all RRM measurements

- This paper discusses the open issues listed above in the summary and provide our view.

2	New gap patterns for PRS measurements

In this paper we will go through the three options one by one. Option 1 gives the simplest solution while ignoring some factors which might cause impact on RRM measurements. PRS measurements can be scarce thus there were concerns raised during RAN4 96-e that equal splitting between RRM measurements and PRS measurements are not efficient enough. We share such concerns and thus further propose a new option, which is the measurement gap is split between RRM measurements and PRS measurements by a certain percentage X%.

The measurement gap is split between RRM measurements and PRS measurements by a certain percentage X%.

Balancing RRM measurements and PRS measurements, we propose X = 70. We’re willing to discuss this value further during the meeting.

The measurement gap is split between RRM measurements and PRS measurements by a certain percentage 70%. The value of X can be further discussed.

The issue with Option 3 is that it tried to mix Option 1 and 2 but it’s not a complete solution. With Option 2, it’s a bit complicated with specification modification and maintenance. In our view splitting the MG with a certain percentage is efficient enough. Further enhancement can be made in R-17 phase.

4	Conclusion

Proposal 1: The measurement gap is split between RRM measurements and PRS measurements by a certain percentage X%.

Proposal 2: The measurement gap is split between RRM measurements and PRS measurements by a certain percentage 70%. The value of X can be further discussed.

References

- R4-2012048