3GPP TSG-RAN WG4 Meeting #97-e                                                          R4-2014033

Electronic Meeting, 2 - 13 November 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.21-11: Addition of DC configuration for DC_2-8_n2

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_2A-8A_n2A as defined in WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_2-8_n2

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-8A_n2A | DC_2A_n2A1 DC_8A_n2A |
| NOTE1: Only single switched UL is supported |  |

### 5.x.2	Co-existence studies

Co-existence analysis from DC_8_n2 show that IMD4 of the dual UL of DC_8_n2 may fall into the DL of band 2.

### 5.x.3	∆TIB and ∆RIB values

For DC_2-8_n2, the TIB,c and RIB,c values are reused from the DC_8_n2, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-8-n2 | 2 | 0.3 |
|  | 8 | 0.3 |
|  | n2 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-8-n2 | 2 | 0 |
|  | 8 | 0 |
|  | n2 | 0 |

### 5.x.4	REFSENS requirements

Based on the co-existence studies MSD due to IMD4 is required. The MSD values can be reused from DC_8-n2.

Table 6.1.x.6-1: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_2-8_n2 | 2 | 1860 | 5 | 25 | 1940 | 4 | FDD | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A |  | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A |  | N/A |

##### ---End of changes---

# Reference

[1]	RP-201728, “WID revision Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, RAN#89-e