3GPP TSG-RAN WG4 Meeting #97-e                                                         R4-2014136

Online, 2nd -13th Nov, 2020

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_5-46_n66

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_5-46_n66 according to the request in [1].

# 2. Reference

- RP-201728, New WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_5-46_n66

### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |

### 5.x.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fallback modes. It can be got:

- The IMD4 and IMD5 of UL configuration on DC_5_n66 will impact of the Band 46 DL

### 5.x.3	∆TIB and ∆RIB values

For DC_5-46_n66, the TIB,c and RIB,c values are given in the tables below that reused the values for DC_46-66_n5.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-46_n66 | 5 | 0.3 |
|  | 46 | 0 |
|  | n66 | 0.3 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-46_n66 | 5 | 0 |
|  | 46 | 0 |
|  | n66 | 0 |

### 5.x.4	REFSENS requirements

Table 5.x.4-1 shows the required MSD levels for the DC configuration.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-46A_n66A | 5 | 847 | 5 | 25 | 892 | N/A | N/A |
|  | 46 | 5163 | 10 | 50 | 5163 | 9.01 | IMD4 ¦2*fB5+2*fn66¦ |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

<End of Text Proposal>