3GPP TSG-RAN WG4 Meeting # 97-e                                                            R4-2014400

Electronic Meeting, 2 - 13 Nov., 2020

Source: 	CATT

Title: 	Simulation results for NR PUSCH UL timing adjustment demodulation requirement

Agenda item:	7.15.3.2.3

Document for:	Discussion

- Introduction

In RAN4#95-e meeting, introducing additional SCS/CBW combinations for ULTA in WF [1] was approved as follows:

- Additional SCS/CBW combinations - Add simulation assumptions for 5MHz CBW/15KHz SCS and 10Mhz CBW/30KHz SCS to requirements for agreed UL timing adjustment scenarios and use applicability rule to only test supported SCS/CBW combinations.

In RAN4#96-e meeting,  the 12 contiguously allocated RBs for each UE for 5MHz CBW/15KHz SCS and 10Mhz CBW/30KHz SCS in WF[2] was approved as follows:

- Minimum CBW FDRA and SRS configuration - Use the following configurations: - FDRA - 5MHz CBW/15KHz SCS: 12 contiguously allocated RBs for each UE - Start RB index - Moving UE: 0 - Stationary UE: 12 - 10MHz CBW/30KHz SCS: 12 contiguously allocated RBs for each UE - Start RB index - Moving UE: 0 - Stationary UE: 12

And, introducing scenario X requirements was approved as follows:

- Addition of scenario “X” - RAN4 agree to introduce scenario X requirements under rel-16 HST WI, adding it in non-HST sections/tables to avoid misleading. - A BS, which declares to support HST and passes scenario Y or scenario Z, can assume implicit test passing of scenario X. - Scenario X parameters:

This paper will provide our evaluation results for NR PUSCH UL timing for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS for scenario Y and Z, and also provide our evaluation results for NR PUSCH UL timing for all CBW/SCS combinations for scenario X for simulation alignment.

- Evaluation results

The simulation assumptions [1] are shown in Table A-1 in Annex.

The evaluation results for  NR PUSCH UL timing adjustment are shown in Table 2-1 and Table 2-2.

Table 2-1: Evaluation results for NR PUSCH UL timing adjustment for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS for scenario Y and Z

| Scenario | CBW | SCS | l0 | MCS | A | Δω | PUSCH mapping type | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| Scenario Y | 5MHz | 15KHz | 2 | 16 | 10us | 0.13s-1 | Type A | 6.2 | 8.7 |
|  | 5MHz | 15KHz | 0 | 16 | 10us | 0.13s-1 | Type B | 6.17 | 8.67 |
|  | 10MHz | 30KHz | 2 | 16 | 5us | 0.26s-1 | Type A | 6.2 | 8.7 |
|  | 10MHz | 30KHz | 0 | 16 | 5us | 0.26s-1 | Type B | 6.18 | 8.68 |
| Scenario Z | 5MHz | 15KHz | 2 | 16 | 10us | 0.18s-1 | Type A | 6.28 | 8.78 |
|  | 5MHz | 15KHz | 0 | 16 | 10us | 0.18s-1 | Type B | 6.25 | 8.75 |
|  | 10MHz | 30KHz | 2 | 16 | 5us | 0.36s-1 | Type A | 6.27 | 8.77 |
|  | 10MHz | 30KHz | 0 | 16 | 5us | 0.36s-1 | Type B | 6.25 | 8.75 |

Table 2-2: Evaluation results for NR PUSCH UL timing adjustment for all CBW/SCS combinatons for scenario X

| Scenario | CBW | SCS | l0 | MCS | A | Δω | PUSCH mapping type | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| Scenario X | 10MHz | 15KHz | 2 | 16 | 10us | 0.04s-1 | Type A | 8.07 | 10.57 |
|  | 10MHz | 15KHz | 0 | 16 | 10us | 0.04s-1 | Type B | 8.06 | 10.56 |
|  | 40MHz | 30KHz | 2 | 16 | 5us | 0.08s-1 | Type A | 10.7 | 13.2 |
|  | 40MHz | 30KHz | 0 | 16 | 5us | 0.08s-1 | Type B | 10.52 | 13.02 |
| Scenario X | 5MHz | 15KHz | 2 | 16 | 10us | 0.04s-1 | Type A | 8.61 | 11.11 |
|  | 5MHz | 15KHz | 0 | 16 | 10us | 0.04s-1 | Type B | 8.63 | 11.13 |
|  | 10MHz | 30KHz | 2 | 16 | 5us | 0.08s-1 | Type A | 10.42 | 12.92 |
|  | 10MHz | 30KHz | 0 | 16 | 5us | 0.08s-1 | Type B | 10.29 | 12.79 |

- Conclusion

This paper provides our evaluation results for NR PUSCH UL timing for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS for scenario Y and Z, and also provides our evaluation results for NR PUSCH UL timing for all CBW/SCS combinations for scenario X for simulation alignment.

- References

- R4-2008821, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#95-e.

- R4-2012676, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- Annex

Table A-1: Simulation assumptions for NR PUSCH UL timing adjustment

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and type B |
| MCS index | MCS 16 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | Type A: l0 = 2 Type B: l0 = 0 |
| SCS and BW | 15kHz:10MHz, 5MHz; 30kHz: 40MHz, 10MHz |
| RB assignment | 15kHz SCS/10MHz CBW: 25 continuously allocated RBs for each UE 30kHz SCS/40MHz CBW: 50 continuously allocated RBs for each UE 15kHz SCS/5MHz CBW: 12 continuously allocated RBs for each UE 30kHz SCS/10MHz CBW: 12 continuously allocated RBs for each UE |
| Starting PRB index | 15kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 25 30kHz SCS/40MHz CBW: Moving UE 0, Stationary UE 50 15kHz SCS/5MHz CBW: Moving UE 0, Stationary UE 12 30kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 12 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario X: Stationary UE: AWGN, Moving UE: TDLC300-400 15kHz SCS: A=10us, △ω=0.04s-1 30kHz SCS: A=5us, △ω=0.08s-1 Scenario Y: Stationary UE: AWGN, Moving UE: AWGN 15kHz SCS: A=10us, △ω=0.13s-1 30kHz SCS: A=5us, △ω=0.26s-1 Scenario Z: Stationary UE: AWGN, Moving UE: AWGN 15kHz SCS: A=10us, △ω=0.18s-1 30kHz SCS: A=5us, △ω=0.36s-1 |
| SRS transmission(optional) | Disabled |
| Testing metric | SNR @70% of maximum throughput |