3GPP TSG- RAN WG4 Meeting #97-e	R4-2014408

# Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1169 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.133, Remove duplication definition for measurement requirements for power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The applicability of relaxed measurement requirements for EMR is defined clearly in TS38.304. The conditions of “T331 timer is not running…” in current specification is not accurate and duplicated. 1 hour measurement interval has been defined in TS38.304, and no tested will be defined in RAN4. The measurements for UE fulfillslow mobility and not-at-cell edge criteria are duplicated and may lead to misalignment with RAN2 specification. For measurement requirements for higher priority carrier for inter frequency and inter-RAT when UE fulfills not-at-cell edge criterion are normal requirements, they need not be defined in clause 4.2.2.10.3 and 4.2.2.11.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add declaration applicable defined in TS38.304, and remove the conditions of “T331 timer is not running…”. Remove the requirement definition in clause 4.2.2.9.4, 4.2.2.10.4, and 4.2.2.11.4. Remove measurement requirements for higher priority carrier for inter frequency and inter-RAT when UE fulfills not-at-cell edge criterion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some requirements and conditions are defined duplicately with RAN2 specification and may lead to misalignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.9, 4.2.2.10, 4.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.2.9 	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

4.2.2.9.1 	Introduction

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, and UE has fulfilled only the lowMobilityEvalutation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.1-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.1-1.

Table 4.2.2.9.1-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |  |  |

4.2.2.9.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criteria and combineRelaxedMeasCondition [2] not configured or is configured but set to FALSE, and UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.9.3-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.9.3-1.

Table 4.2.2.9.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

4.2.2.9.4	Void

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

4.2.2.10.1	Introduction

This clause contains the requirements for measurements on applicable inter-frequency NR cells as defined in clause 5.2.4.9.0 in [1] when the UE is configured with any of following relaxed measurement criteria and meets the EMR requirements as defined in [1]:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4. 9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4. 9.1 and 5.2.4.X.2 in [1] respectively.

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.2-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.10.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

4.2.2.10.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.10.3-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.10.3-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.10.3-1.

Table 4.2.2.10.3-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 | cycles) |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

4.2.2.10.4 	Void

4.2.2.11 	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

4.2.2.11.1 	Introduction

This clause contains the requirements for measurements on applicable inter-RAT E-UTRAN cells as defined in clause 5.2.4.9.0 in [1] when the UE is configured with any of following relaxed measurement critera and meets the EMR requirements as defined in [1]:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

4.2.2.11.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with lowMobilityEvalutation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, and

-	UE has fulfilled only the lowMobilityEvalutation [2] criterion.

When Srxlev ≤ SnonIntraSearchP and Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.2-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.11.2-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

4.2.2.11.3 	Measurements for UE fulfilling with not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured or configured but set to FALSE, and

-	UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The requirements defined in clause 4.2.2.3 apply for this clause except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.11.3-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.11.3-1.

Table 4.2.2.11.3-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvalutation [2] criterion. |  |  |  |

4.2.2.11.4 	Void