3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014423

Electronic Meeting, 2-13 Nov., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0503 | rev | - | Current version: | 16.5.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1, Introduce new band combination of V2X_n39A-n47A and V2X_n40A-n47A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LTE_V2X_PC5_combos |  |  |  |  |  | Date: |  |  | 2020-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The con-current operation of V2X_n39A-n47A and V2X_n40A-n47A should be introduced based on request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add operating bands for V2X_n39A-n47A and V2X_n40A-n47A in Table 5.2E.2-1 in section 5.2E.2 - Add channel bandwidths for V2X_n39A-n47A and V2X_n40A-n47A in table Table 5.2E.2-1 in section 5.3E.2 - Add maximum output power for V2X_n39A-n47A and V2X_n40A-n47A in Table 6.2E.1.2-1 in section 6.2E.1.2 - Add ΔTIB,c for V2X_n39A-n47A and V2X_n40A-n47A in Table 6.2E.4.3-1 in section 6.2E.4.3 - Add spurious emissions for UE co-existence in Table 6.5E.3.1.1-1 in clause 6.5E.3.1.1 - Add REFSENS and ΔRIB,c for V2X_n39A-n47A and V2X_n40A-n47A in Table 7.3E.3-1 and Table 7.3E.3-2 in section 7.3E.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The con-current operation of V2X_n39A-n47A and V2X_n40A-n47A would not be defined in 38.101-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2E.2, 5.3E.2, 6.2E.1.2, 6.2E.4.3, 6.5E.3.1.1, 7.3E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.2E.2	V2X operating bands for concurrent operation

NR V2X operation is designed to operate concurrent with NR uplink/downlink on the operating bands combinations listed in Table 5.2E.2-1.

Table 5.2E.2-1 Inter-band con-current V2X operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n39-n47 | n39 | Uu |
|  | n47 | PC5 |
| V2X_n40-n47 | n40 | Uu |
|  | n47 | PC5 |
| V2X_n71-n47 | n71 | Uu |
|  | n47 | PC5 |

<End of Change 1>

<Start of Change 2>

5.3E.2	Channel bandwidth for V2X concurrent operation

For NR V2X inter-band con-current operation in FR1, the NR V2X channel bandwidths for each operating band is specified in Table 5.3E.2-1.

Table 5.3E.2-1: Inter-band con-current V2X configurations

| V2X con-current operating band Configuration |  | NR Bands |  | SCS kHz |  | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 40 MHz |  |  | 50 MHz |  | 60 MHz | 80 MHz |  | Maximum bandwidth [MHz] |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n39A-n47A |  | n39 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  | 80 |  | 0 |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n47 |  | 15 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| V2X_n40A-n47A |  | n40 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  | 120 |  | 0 |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | n47 |  | 15 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| V2X_n71A-n47A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |

<End of Change 2>

<Start of Change 3>

6.2E.1.2	UE maximum output power for V2X con-current operation

For the inter-band con-current NR V2X operation, the maximum output power is specified in Table 6.2E.1.1-1. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.2-1: Con-current NR V2X UE Power Class for uplink inter-band combination (two bands)

| NR V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n39A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n40A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n71A-n47A |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

<End of Change 3>

<Start of Change 4>

6.2E.4.3	ΔTIB,c for V2X

For the UE which supports inter-band V2X configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2E.4.3-1: ΔTIB,c due to V2X con-current operation (two bands)

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| V2X_n39A-n47A | n39 | TBD |
|  | n47 | TBD |
| V2X_n40A-n47A | n40 | TBD |
|  | n47 | TBD |

<End of Change 4>

<Start of Change 5>

6.5E.3.3	Spurious emissions for UE co-existence for V2X con-current operation

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.1.1-1 apply for the corresponding inter-band con-current operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

Table 6.5E.3.1.1-1: Requirements for inter-band con-current V2X operation

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n39A-n47A | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | NR Band n47 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n40A-n47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 42, 44, 45, 68, 72 NR Band n47, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| V2X_n71A-n47A | E-UTRA Band 5, 26, 53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | NR Band n5, n47, n71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

<End of Change 5>

<Start of Change 6>

7.3E.3	Reference sensitivity power level for V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with NR uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes 8.2 with parameters specified in Table 7.3E.3-1. Also the NR downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.

For the inter-band con-current NR V2X operation, and the UE also supports a NR downlink inter-band con-current configuration in Table 7.3E.3-2, the minimum requirement for reference sensitivity shall be increased by the amount given in ΔRIB,c in Table 7.3E.3-2 for the corresponding NR V2X inter-band combinations.

Table 7.3E.3-1: Reference sensitivity for V2X Communication QPSK PREFSENS

| Inter-band V2X reception |  |  |  |  | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | NR band |  | NR Band |  |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  |  | 15 MHz (dBm) | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  |  | 40 MHz (dBm) |  | 50 MHz (dBm) |  | 60 MHz (dBm) |  | 80 MHz (dBm) |  | Duplex Mode |
| n47 | n39 |  | n39 |  |  | 15 |  | -100.0 |  | -96.8 |  |  | -95.0 | -93.8 |  | -92.7 |  | -91.9 |  |  | -90.6 |  |  |  |  |  |  |  | TDD |
|  |  |  |  |  |  | 30 |  |  |  | -97.1 |  |  | -95.1 | -94.0 |  | -92.8 |  | -92.0 |  |  | -90.7 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | -97.5 |  |  | -95.4 | -94.2 |  | -93.0 |  | -92.1 |  |  | -90.9 |  |  |  |  |  |  |  |  |
|  |  |  | n47 |  |  | 15 |  |  |  | -92.8 |  |  |  | -89.7 |  |  |  | -87.9 |  |  | -86.6 |  |  |  |  |  |  |  | HD |
|  |  |  |  |  |  | 30 |  |  |  | -93.1 |  |  |  | -89.9 |  |  |  | -88.0 |  |  | -86.7 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | -93.5 |  |  |  | -90.1 |  |  |  | -88.1 |  |  | -86.9 |  |  |  |  |  |  |  |  |
| n47 | n40 |  | n40 |  |  | 15 |  | -100.0 |  | -96.8 |  |  | -95.0 | -93.8 |  | -92.7 |  | -91.9 |  |  | -90.6 |  | -89.6 |  |  |  |  |  | TDD |
|  |  |  |  |  |  | 30 |  |  |  | -97.1 |  |  | -95.1 | -94.0 |  | -92.8 |  | -92.0 |  |  | -90.7 |  | -89.7 |  | -88.9 |  | -87.6 |  |  |
|  |  |  |  |  |  | 60 |  |  |  | -97.5 |  |  | -95.4 | -94.2 |  | -93.0 |  | -92.1 |  |  | -90.9 |  | -89.8 |  | -89.1 |  | -87.6 |  |  |
|  |  |  | n47 |  |  | 15 |  |  |  | -92.8 |  |  |  | -89.7 |  |  |  | -87.9 |  |  | -86.6 |  |  |  |  |  |  |  | HD |
|  |  |  |  |  |  | 30 |  |  |  | -93.1 |  |  |  | -89.9 |  |  |  | -88.0 |  |  | -86.7 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | -93.5 |  |  |  | -90.1 |  |  |  | -88.1 |  |  | -86.9 |  |  |  |  |  |  |  |  |
| n47 | n71 | n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  | FDD |
|  |  |  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n47 | 15 |  | -92.5 |  | -89.2 |  | -87.4 | -86.1 |  |  |  | HD |
|  |  |  | 30 |  | -92.1 |  | -89.4 |  | -87.7 | -86.2 |  |  |  |  |
|  |  |  | 60 |  | -92.9 |  | -89.1 |  | -87.9 | -86.4 |  |  |  |  |
| NOTE 1: Reference measurement channel is defined in A.7.2. NOTE 2: The signal power is specified per antenna port. NOTE 3: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3E.3-2: ΔRIB,V2X (two bands)

| V2X inter-band con-current band Combination | NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n39-n47 | n39 | TBD |
| V2X_n40-n47 | n39 | TBD |
| V2X_n71-n47 | n71 | 0.0 |

The reference sensitivity is defined to be met with NR uplink assigned to one band (that differs from the V2X operating band) and all NR downlink carriers active. The NR uplink resource blocks shall be located as close as possible to NR V2X operating band but confined within the transmission bandwidth configuration for the channel. The uplink configuration for the NR operating band is specified in Table 7.3E.3-3 and 7.3E.3-4. The REFSENS of Uu downlink and PC5 sidelink will be tested at the same time.

Table 7.3E.3-3: Uplink configuration for REFSENS of NR V2X Bands (PC5)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X UL band (Uu) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n39 | n39 | 15 | 10 | 50 | TDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n40 | n40 | 15 | 10 | 50 | TDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n71 | n71 | 15 | 10 | 52 | FDD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 11 |  |

Table 7.3E.3-4: Sidelink TX configuration for REFSENS of NR V2X Bands (Uu)

| Inter-band NR V2X con-current band configuration |  | NR UL band / SCS/ Channel BW / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X band (PC5) | NR V2X band (Uu) | NR V2X band (PC5) | SCS (kHz) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| n47 | n39 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n40 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| n47 | n71 | n47 | 15 | 10 | 50 | HD |
|  |  |  | 30 | 10 | 24 |  |
|  |  |  | 60 | 10 | 10 |  |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. |  |  |  |  |  |  |

<< Unchanged sections are omitted >>

<End of Change 6>