3GPP TSG-RAN4 Meeting #97-e	                                       R4-2014438

Electronic meeting, Nov. 2-13, 2020

Title:	Simulation results for CSI-RSRQ measurement accuracy

Source:	CATT

Agenda Item:	7.14.2.1.2

Document for:	Discussion

- Introduction

In RAN4#92 meeting [1], it was agreed to conduct simulation studies to evaluate measurement bandwidth of CSI-RS for RRM measurement requirement. And the simulation assumption for CSI-RS based measurement is agreed in [2].

In RAN4 #96e meeting [3], it was agreed that the CSI-RS based L3 measurement requirements in R16 are defined for CSI-RS configuration {D=3 with PRBs ≥ 48}.

Based on the latest conclusion, in this contribution, we provide the simulation results for CSI-RSRQ measurement.

- Discussion

In this section, we provide the simulation results of delta RSRQ under AWGN and EPA5 channel for 15 kHz and 30 kHz and provide the simulation results of delta RSRQ under TDL-A and TDL-C channel for 60kHz. Table 1-6 provide the delta RSRQ based on different samples, different transmission power which is indicated by SINR for 15kHz, 30kHz and 60kHz, respectively. The simulations are based on the CSI-RS configuration {D=3, PRB = 48}. The CDF figures of simulation results of delta RSRQ can be found in Annex.

15 kHz case:

Table 1: Delta RSRQ, SINR=-5.97 dB, td= CP/2, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.32 | 0.33 | 1.67 | 1.65 | 1.34 | 1.65 |
|  | 5 | -1.08 | 0.33 | 1.48 | 1.41 | 1.15 | 1.41 |
|  | 10 | -0.49 | 0.33 | 1.26 | 0.82 | 0.93 | 0.93 |
| EPA5 | 3 | -1.67 | 0.31 | 1.97 | 1.98 | 1.66 | 1.98 |
|  | 5 | -1.39 | 0.31 | 1.57 | 1.70 | 1.26 | 1.70 |
|  | 10 | -0.75 | 0.26 | 1.34 | 1.01 | 1.08 | 1.08 |

Table 2: Delta RSRQ, SINR=-3.97 dB, td= CP/2, 15 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.61 | 0.33 | 1.1 | 0.94 | 0.77 | 0.94 |
|  | 5 | -0.34 | 0.33 | 0.93 | 0.67 | 0.60 | 0.67 |
|  | 10 | -0.12 | 0.33 | 0.82 | 0.45 | 0.49 | 0.49 |
| EPA5 | 3 | -1.03 | 0.24 | 1.22 | 1.27 | 0.98 | 1.27 |
|  | 5 | -0.89 | 0.24 | 0.97 | 1.13 | 0.73 | 1.13 |
|  | 10 | -0.48 | 0.07 | 0.81 | 0.55 | 0.74 | 0.74 |

30 kHz case:

Table 3: Delta RSRQ, SINR=-5.97 dB, td= CP/2, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -1.17 | 0.33 | 1.87 | 1.50 | 1.54 | 1.54 |
|  | 5 | -0.73 | 0.33 | 1.44 | 1.06 | 1.11 | 1.11 |
|  | 10 | -0.37 | 0.33 | 1.09 | 0.70 | 0.76 | 0.76 |
| EPA5 | 3 | -1.75 | 0.47 | 2.66 | 2.22 | 2.19 | 2.22 |
|  | 5 | -1.37 | 0.47 | 1.89 | 1.84 | 1.42 | 1.84 |
|  | 10 | -0.59 | 0.47 | 1.74 | 1.06 | 1.27 | 1.27 |

Table 4: Delta RSRQ, SINR=-3.97 dB, td= CP/2, 30 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 3 | -0.55 | 0.32 | 1.17 | 0.87 | 0.85 | 0.87 |
|  | 5 | -0.35 | 0.29 | 0.86 | 0.64 | 0.57 | 0.64 |
|  | 10 | -0.07 | 0.29 | 0.74 | 0.36 | 0.45 | 0.45 |
| EPA5 | 3 | -1.24 | 0.17 | 1.51 | 1.41 | 1.34 | 1.41 |
|  | 5 | -0.96 | 0.17 | 1.29 | 1.13 | 1.12 | 1.13 |
|  | 10 | -0.43 | 0.17 | 1.02 | 0.60 | 0.85 | 0.85 |

60 kHz case:

Table 5: Delta RSRQ, SINR=-5.97 dB, td= CP/2, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.80 | 1.15 | 3.58 | 2.95 | 2.43 | 2.95 |
|  | 5 | -0.77 | 1.15 | 3.05 | 1.92 | 1.90 | 1.92 |
|  | 10 | -0.34 | 0.98 | 2.22 | 1.32 | 1.24 | 1.32 |
| TDL-C | 3 | -1.36 | 0.44 | 2.32 | 1.80 | 1.88 | 1.88 |
|  | 5 | -1.05 | 0.44 | 2.14 | 1.49 | 1.70 | 1.70 |
|  | 10 | -0.41 | 0.56 | 1.45 | 0.97 | 0.89 | 0.97 |

Table 6: Delta RSRQ, SINR=-3.97 dB, td= CP/2, 60 KHz, D=3, 48PRB

| Channel | Sample num | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDL-A | 3 | -1.6 | 0.41 | 2.31 | 2.01 | 1.90 | 2.01 |
|  | 5 | -1.23 | 0.41 | 1.90 | 1.64 | 1.49 | 1.64 |
|  | 10 | -0.75 | 0.41 | 1.30 | 1.16 | 0.89 | 1.16 |
| TDL-C | 3 | -1.06 | 0.25 | 1.53 | 1.31 | 1.28 | 1.31 |
|  | 5 | -0.84 | 0.25 | 1.25 | 1.09 | 1.00 | 1.09 |
|  | 10 | -0.52 | 0.25 | 0.60 | 0.77 | 0.35 | 0.77 |

According to the above simulation results, for the case SINR=-5.97dB, when the sample number is larger than 5, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB. For the case SINR=-3.97dB, when the sample number is larger than 3, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB.

Observation 1: For the case SINR=-5.97dB, when the sample number is larger than 5, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB.

Observation 2: For the case SINR=-3.97dB, when the sample number is larger than 3, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB.

- Conclusion

In this contribution, we provide the simulation results for CSI-RSRQ measurement, and provide the observations as follows:

Observation 1: For the case SINR=-5.97dB, when the sample number is larger than 5, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB.

Observation 2: For the case SINR=-3.97dB, when the sample number is larger than 3, for most of cases, the measurement error which is indicated by delta RSRQ can be in ±2dB.

- Reference

[1] R4-1910701,	“RAN4 #92 Meeting report”,		RAN4

[2] R4-1910949,	“Link-level simulation assumptions for CSI-RS based measurements”,		CATT

[3] RAN4_96e_RRM_chairman_report_12_EOM

- Annex

15 KHz case:

Figure 1. AWGN 15KHz CSI-RS Periodicity=40ms SINR=-6dB  Figure 2. AWGN 15KHz CSI-RS Periodicity=40ms SINR=-4dB

Figure 3. EPA5 15KHz CSI-RS Periodicity=40ms SINR=-6dB  Figure 4. EPA5 15KHz CSI-RS Periodicity=40ms SINR=-4dB

30 KHz case:

Figure 5. AWGN 30KHz CSI-RS Periodicity=40ms SINR=-6dB  Figure 6. AWGN 30KHz CSI-RS Periodicity=40ms SINR=-4dB

Figure 7. EPA5 30KHz CSI-RS Periodicity=40ms SINR=-6dB  Figure 8. EPA5 30KHz CSI-RS Periodicity=40ms SINR=-4dB

60 KHz case:

Figure 9. TDL-A 60KHz CSI-RS Periodicity=40ms SINR=-6dB  Figure 10. TDL-A 60KHz CSI-RS Periodicity=40ms SINR=-4dB

Figure 11. TDL-C 60KHz CSI-RS Periodicity=40ms SINR=-6dB Figure 12. TDL-C 60KHz CSI-RS Periodicity=40ms SINR=-4dB