3GPP TSG-RAN4 Meeting #97-e	                                                                 R4-2014439

Electronic meeting, Nov. 2-13, 2020

Title:	Discussion on performance requirement for CSI-RSRP

Source:	CATT

Agenda Item:	7.14.2.1.1

Document for:	Discussion

1 Introduction

In RAN4 #96-e meeting [1], the core part of CSI-RS based measurement was completed. According to the WI process, we will discuss accuracy requirements in this meeting.

In this paper, we have some discussions on the performance requirement of CSI-RSRP measurement and give our proposals. The discussion is based on the side condition, accuracy requirement and report mapping of CSI-RSRP measurement. The corresponding CR can be found in [2].

2 Discussion

2.1 Side condition and accuracy requirements

In RAN4 #92bis meeting, the simulation assumption for CSI-RS based L3 measurement is approved [3]. In RAN4 #96e meeting [1], it was agreed the CSI-RS based L3 measurement requirements in R16 are defined for CSI-RS configuration {D=3, PRB≥48}. Based on the simulation assumption and the latest conclusion, we provide the simulation results for CSI-RSRP measurement in [4].

According to the simulation results, for the case of SINR=-5.97dB in FR1, when the sample number is larger than 5, for most of cases, the measurement accuracy including absolute accuracy and relative accuracy can be in ±2dB.

For the case of SINR=-5.97dB in FR2, when the sample number is larger than 5, for most of cases, the absolute accuracy can be in ±3.5dB while the relative accuracy can be in ±2.5dB.

Proposal 1: Side conditions for CSI-RSRP measurement are defined -6dB for intra-frequency measurement in FR1 and FR2.

Proposal 2: CSI-RSRP measurement requirements are defined based on 5 samples for intra-frequency measurement.

As for the conditions under which the accuracy requirements of CSI-RS based L3 measurement apply, since the receive sensitivity is same for CSI-RS based measurement and SSB based measurement, the conditions of SSB based measurement can be reused for CSI-RS based L3 measurement in our understanding.

Proposal 3: The conditions of SSB based measurement can be reused for CSI-RS based L3 measurement

When defining performance requirement of CSI-RS based L3 measurement, the implementation margin approximately 2dB is needed. Base on the discussion above, the performance requirement of CSI-RSRP can be defined as shown in [2].

2.2 Report mapping

Since the proposed side condition of CSI-RSRP measurement is same as that of SS-RSRP, and the accuracy requirement can also compared to the accuracy of SS-RSRP, the reporting range and resolution of SS-RSRP measurement can be reused for the CSI-RSRP measurement for L3 reporting.

As for differential reporting of CSI-RSRP for L3 reporting, if UE choose to report the largest power of CSI-RS to be measured, the reporting range shall be from -30dBm to 0dBm. And the existing 2dB resolution can be used. The corresponding CR can be found in [2].

Proposal 4: The reporting range and resolution of SS-RSRP measurement can be reused for CSI-RSRP measurement for L3 reporting.

3 Summary

In this paper, the performance requirements of CSI-RSRP measurement are discussed, and following proposals are given：

Proposal 1: Side conditions for CSI-RSRP measurement are defined -6dB for intra-frequency measurement in FR1 and FR2.

Proposal 2: CSI-RSRP measurement requirements are defined based on 5 samples for intra-frequency measurement.

Proposal 3: The conditions of SSB based measurement can be reused for CSI-RS based L3 measurement.

Proposal 4: The reporting range and resolution of SS-RSRP measurement can be reused for CSI-RSRP measurement for L3 reporting.

4 References

- RAN4_96e_RRM_chairman_report_12_EOM

- R4-2014441, CR on performance requirement for CSI-RSRP, CATT

- R4-1910949, Link-level simulation assumptions for CSI-RS based measurements, CATT

- R4-2014437, Simulation results for CSI-RSRP measurement accuracy, CATT