3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014473

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	11.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals of UE Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Approval

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7] and RAN4#96-e [8].

This contribution provides the proposals of UE parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [9] and [10].

# 2.	Discussion

The downlink coexistence simulation results provided in [9] have shown that:

1) The 5%-tile downlink throughput loss of the victim UE in the simulated urban macro scenario is higher than 5% using 45dB BS ACLR and 33dB UE ACS at 7GHz, while the downlink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with downlink ACIR offsets of -2dB at 10GHz (i.e. 2dB less stringent ACIR).

2) The downlink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with a downlink ACIR offset of -1dB and -0.5dB, respectively, at 7GHz and 10GHz (i.e. 1dB and 0.5dB less stringent ACIR).

3) The downlink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with a downlink ACIR offset of -8dB and -12dB, respectively, at 7GHz and 10GHz (i.e. 8dB and 12dB less stringent ACIR).

Considering the most stringent case among the three simulated scenarios, there is no room for DL ACIR relaxation at 7GHz comparing to the currently specified 45dB BS ACLR and 33dB UE ACS below 6GHz, while 0.5dB DL ACIR relaxation could be considered at 10GHz. Therefore, it is proposed to keep the currently specified 33dB UE ACS for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

The uplink coexistence simulation results provided in [10] have shown that:

1) The uplink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with an uplink ACLR offset of -3dB and -4dB, respectively, at 7GHz and 10GHz (i.e. 3dB and 4dB less stringent ACLR). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -40dBm and -39dBm, respectively, at 7GHz and 10GHz.

2) The 5%-tile uplink throughput loss of the victim UE in the simulated indoor hotspot scenario is higher than 5% using 30dB UE ACLR and 45dB BS ACS at 7GHz, while the uplink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with uplink ACLR offsets of -0.5dB at 10GHz (i.e. 0.5dB less stringent ACLR). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -29dBm and -28dBm, respectively, at 7GHz and 10GHz.

3) The uplink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with uplink ACLR offsets of -9dB at 10GHz (i.e. 9dB less stringent ACLR), while the 5%-tile uplink throughput is not achieved in the simulated dense urban scenario at 10GHz (as shown in Figure 16 above where the 5%-tile UE UL SINR is less than the minimum required SINR of -10dB). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -45dBm and -44dBm, respectively, at 7GHz and 10GHz.

Considering the most stringent case among the three simulated scenarios, there is no room for UL ACLR relaxation at 7GHz comparing to the currently specified 30dB UE ACLR and 45dB BS ACS below 6GHz, while 0.5dB UL ACLR relaxation could be considered at 10GHz. Therefore, it is proposed to keep the currently specified 30dB UE ACLR for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

# 3.	Conclusion

This contribution provides the proposals of UE parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz according to the downlink and uplink coexistence simulation results provided in [9] and [10].

Proposals for approval:

1) To keep the currently specified 33dB UE ACS below 6GHz for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

2) To keep the currently specified 30dB UE ACLR below 6GHz for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz.

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2014476, “Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[10]	R4-2014477, “Uplink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.