3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014477

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	11.1.3.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Uplink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Discussion

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7].

This contribution provides the uplink coexistence simulation results according to the agreed assumptions in [7] and [8].

# 2.	Discussion

All simulation assumptions for the urban macro and indoor uplink scenario were agreed in [7], except the AAS BS antenna characteristics which point to the WF agreed in [9], these assumptions were used to obtain the simulation results in this contribution.

2.1	Urban Macro Scenario

The urban macro simulation results of the victim and interfering UE transmit power at 7GHz and 10GHz carrier frequency are provided in Figures 1 and 2 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that over 20% and 30% of the UE are transmitting at maximum power, respectively, at 7GHz and 10GHz.

Figure 1: Urban Macro UE transmit power at 7GHz

Figure 2: Urban Macro UE transmit power at 10GHz

The urban macro simulation results of the victim UE UL SINR at 7GHz and 10GHz carrier frequency are provided in Figures 3 and 4 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that the target UL SINR of 15dB is nearly achieved by most of the victim UE at 7GHz and 10GHz.

Figure 3: Urban Macro UE UL SINR at 7GHz

Figure 4: Urban Macro UE UL SINR at 10GHz

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) at 7GHz and 10GHz carrier frequency are provided in Figures 5 and 6 below. The 99.99%-tile received blocking signal power levels are around -40dBm and -39dBm, respectively, at 7GHz and 10GHz.

Figure 5: Urban Macro received blocking signal power at 7GHz

Figure 6: Urban Macro received blocking signal power at 10GHz

The urban macro simulation results of the uplink throughput loss of the victim UE with different ACLR offsets (compared to the 30dB UE ACLR) at 7GHz and 10GHz carrier frequency are provided in Table 1 below. It can be seen from the table that the uplink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with an uplink ACLR offset of -3dB and -4dB, respectively, at 7GHz and 10GHz (i.e. 3dB and 4dB less stringent ACLR).

| ACLR offset X [dB] | 0 | -1 | -2 | -3 | -4 | -5 | -6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 1.08% | 1.24% | 1.43% | 1.65% | 1.89% | 2.15% | 2.45% |
| 5%-tile throughput loss (7GHz) | 1.01% | 1.68% | 3.14% | 3.98% | 5.76% | 8.08% | 8.84% |
| Average throughput loss (10GHz) | 0.90% | 1.04% | 1.21% | 1.39% | 1.60% | 1.84% | 2.11% |
| 5%-tile throughput loss (10GHz) | 1.98% | 2.27% | 2.28% | 3.90% | 4.00% | 6.41% | 6.41% |

Table 1: Urban Macro UL throughput loss of victim UE at 7GHz and 10GHz

2.2	Indoor Hotspot Scenario

The indoor hotspot simulation results of the victim and interfering UE transmit power at 7GHz and 10GHz carrier frequency are provided in Figures 7 and 8 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that most of the UE are not transmitting at maximum power at 7GHz nor 10GHz, because of the smaller propagation loss in this scenario.

Figure 7: Indoor Hotspot UE transmit power at 7GHz

Figure 8: Indoor Hotspot UE transmit power at 10GHz

The indoor hotspot simulation results of the victim UE UL SINR at 7GHz and 10GHz carrier frequency are provided in Figures 9 and 10 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that the target UL SINR of 15dB is not achieved by most of the victim UE at 7GHz nor 10GHz even without the interfering UE, because the co-channel interference is the dominant factor in this scenario.

Figure 9: Indoor Hotspot UE UL SINR at 7GHz

Figure 10: Indoor Hotspot UE UL SINR at 10GHz

The indoor hotspot simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) at 7GHz and 10GHz carrier frequency are provided in Figures 11 and 12 below. The 99.99%-tile received blocking signal power levels are around -29dBm and -28dBm, respectively, at 7GHz and 10GHz.

Figure 11: Indoor Hotspot received blocking signal power at 7GHz

Figure 12: Indoor Hotspot received blocking signal power at 10GHz

The indoor hotspot simulation results of the uplink throughput loss of the victim UE with different ACIR offsets (compared to the 30dB UE ACLR) at 7GHz and 10GHz carrier frequency are provided in Table 2 below. It can be seen from the table that the 5%-tile uplink throughput loss of the victim UE in the simulated indoor hotspot scenario is higher than 5% using 30dB UE ACLR and 45dB BS ACS at 7GHz, while the uplink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with uplink ACLR offsets of -0.5dB at 10GHz (i.e. 0.5dB less stringent ACLR).

| ACLR offset X [dB] | 0 | -1 | -2 | -3 | -4 | -5 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 2.80% | 3.19% | 3.65% | 4.14% | 4.69% | 5.32% |
| 5%-tile throughput loss (7GHz) | 5.89% | 6.60% | 8.11% | 8.44% | 10.02% | 11.19% |
| Average throughput loss (10GHz) | 2.80% | 3.19% | 3.65% | 4.13% | 4.68% | 5.31% |
| 5%-tile throughput loss (10GHz) | 4.62% | 5.36% | 7.28% | 7.60% | 9.13% | 9.71% |

Table 2: Indoor Hotspot UL throughput loss of victim UE at 7GHz and 10GHz

2.3	Dense Urban Scenario

The dense urban simulation results of the victim and interfering UE transmit power at 7GHz and 10GHz carrier frequency are provided in Figures 13 and 14 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that over 35% and 45% of the UE are transmitting at maximum power, respectively, at 7GHz and 10GHz.

Figure 13: Dense Urban UE transmit power at 7GHz

Figure 14: Dense Urban UE transmit power at 10GHz

The urban macro simulation results of the victim UE UL SINR at 7GHz and 10GHz carrier frequency are provided in Figures 15 and 16 below. Here the uplink ACIR is obtained using 30dB UE ACLR and 45dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figures that the target UL SINR of 15dB is nearly achieved by half of the victim UE at 7GHz and 10GHz.

Figure 15: Dense Urban UE UL SINR at 7GHz

Figure 16: Dense Urban UE UL SINR at 10GHz

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) at 7GHz and 10GHz carrier frequency are provided in Figures 17 and 18 below. The 99.99%-tile received blocking signal power levels are around -45dBm and -44dBm, respectively, at 7GHz and 10GHz.

Figure 17: Dense Urban received blocking signal power at 7GHz

Figure 18: Dense Urban received blocking signal power at 10GHz

The urban macro simulation results of the uplink throughput loss of the victim UE with different ACLR offsets (compared to the 30dB UE ACLR) at 7GHz and 10GHz carrier frequency are provided in Table 3 below. It can be seen from the table that the uplink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with uplink ACLR offsets of -9dB at 10GHz (i.e. 9dB less stringent ACLR), while the 5%-tile uplink throughput is not achieved in the simulated dense urban scenario at 10GHz (as shown in Figure 16 above where the 5%-tile UE UL SINR is less than the minimum required SINR of -10dB).

| ACLR offset X [dB] | 0 | -2 | -4 | -6 | -8 | -10 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 0.39% | 0.54% | 0.75% | 1.04% | 1.42% | 1.92% |
| 5%-tile throughput loss (7GHz) | 0.36% | 0.83% | 1.06% | 2.52% | 4.31% | 5.37% |
| Average throughput loss (10GHz) | 0.31% | 0.43% | 0.61% | 0.85% | 1.16% | 1.59% |
| 5%-tile throughput loss (10GHz) | N/A | N/A | N/A | N/A | N/A | N/A |

Table 3: Dense Urban UL throughput loss of victim UE at 7GHz and 10GHz

# 3.	Conclusion

This contribution has provided the urban macro uplink coexistence simulation results according to the agreed assumptions.

The simulation results have shown that:

1) The uplink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with an uplink ACLR offset of -3dB and -4dB, respectively, at 7GHz and 10GHz (i.e. 3dB and 4dB less stringent ACLR). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -40dBm and -39dBm, respectively, at 7GHz and 10GHz.

2) The 5%-tile uplink throughput loss of the victim UE in the simulated indoor hotspot scenario is higher than 5% using 30dB UE ACLR and 45dB BS ACS at 7GHz, while the uplink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with uplink ACLR offsets of -0.5dB at 10GHz (i.e. 0.5dB less stringent ACLR). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -29dBm and -28dBm, respectively, at 7GHz and 10GHz.

3) The uplink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with uplink ACLR offsets of -9dB at 10GHz (i.e. 9dB less stringent ACLR), while the 5%-tile uplink throughput is not achieved in the simulated dense urban scenario at 10GHz (as shown in Figure 16 above where the 5%-tile UE UL SINR is less than the minimum required SINR of -10dB). The 99.99%-tile received blocking signal power levels at the TAB connector (i.e. including the antenna array gain) are around -45dBm and -44dBm, respectively, at 7GHz and 10GHz.

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2008926, “WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Huawei, Nokia.

[10]	R4-2011195, “TP to TR 38.921: Dense urban system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[11]	R4-2011932, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, Ericsson.

[12]	R4-2011829, “TP to TR 38.921: BS IMT technology related parameters”, Huawei, HiSilicon, Ericsson, Nokia, ZTE.