3GPP TSG-RAN WG4 Meeting #97-e	R4-2014588

# Electronic Meeting, Nov 2nd – Nov 13th, 2020

Agenda item:	12.3.2.1.5

Source: 	Intel Corporation

Title: 	UE requirements for CA configurations within the same frequency group based on CBM

Document for:	Discussion

# Introduction

Common Beam Management (CBM) architecture can be used in inter-band DL CA for certain CA configurations. Its limitations need to be well understood before corresponding RF requirements are defined. In this contribution, the CBM limitations are analyzed and RF requirements are proposed.

# Discussion

CBM UE uses one common beam to support inter-band CA. It is our understanding that CBM UE cannot work well in non-collocated gNB deployments where incoming DL signals are assumed from two different directions. In general, a common beam may not be able to optimize two different AoA/AoD directions simultaneously.

Observation 1: CBM is more applicable to collocated gNB deployment scenario.

CBM suffers beam squint when covered frequency spectrum getting larger. Phase shifters for beam steering are frequency dependent. Phase shift values for a given direction are optimized at certain frequency to achieve the highest antenna array gain while such phase shift values are not optimized at other frequencies due to phase mismatches between applied phase shift values and required phase shift values at those frequencies for given direction (there could be an exception to beam peak direction, aka, boresight. Explained below). When frequency span from lower channel edge of lowest CC in lower band to upper channel edge of highest CC in upper band in the CA configuration is large, then antenna gains of CBM across the frequency span will experience beam squint effect.

The exception mentioned above is peak direction, the boresight direction. At peak direction the required phase shift values are all zeros (in relative sense across antenna element) for all frequency components assumed with perfect phase calibration.

Observation 2: There is no beam squint degradation at beam peak under perfect phase calibration.

Since REFSENS is defined at Rx beam peak direction, there should be no performance degradation in REFSENS due to beam squint under ideal conditions of no residual phase errors. But by considering practical phase calibrations and quantized phase shifters, the residual phase errors are inevitable, REFSENS will also suffer from beam squint. The same analysis applies to peak EIRP due to reciprocity.

The following table shows some preliminary simulation results of degradation for REFSENS at carrier frequency offset ∆f with phase shifter optimized at carrier frequency Fc under the consideration of the impact from residual phase errors at Fc.

Table 1. REFSENS degradation vs Frequency offset due to phase calibration errors

|  | Frequency offset (∆f) / REFSENS Degradation (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0~0.7GHz | 0.7~2.0GHz | 2.0~4.0GHz | 4.0~6.0GHz | 6.0~8.0GHz |
| Fc = 24GHz | 0 | 0.08 | 0.21 | 0.36 | 0.51 |
| Fc = 37GHz | 0 | 0.05 | 0.14 | 0.22 | 0.32 |

Based on Table 1, we have the following observation and proposal.

Observation 3: beam squint effect happens at beam peak direction if residual phase errors are not zero.

Proposal: RAN4 should study the impact of beam squint effect on beam peak (REFSENS and peak EIRP) based on practical phase calibrations for CBM together with spherical coverage.

For EIS spherical coverage, further analysis is needed. In general, when required EIS spherical coverage are smaller, the beam squint impact is smaller because AoA/AoD directions are close to boresight where at ideal conditions UEs don’t suffer degradation from beam squint.

# Conclusion

We present our analysis on beam squint effect on beam peak direction under non-zero relative phase errors. We have following observations and proposal:

Observation 1: CBM is more applicable to collocated gNB deployment scenario.

Observation 2: There is no beam squint degradation at beam peak under perfect phase calibration.

Observation 3: beam squint effect happens at beam peak direction if residual phase errors are not zero.

Proposal: RAN4 should study the impact of beam squint effect on beam peak (REFSENS and peak EIRP) based on practical phase calibrations for CBM together with spherical coverage.

# References