3GPP TSG-RAN WG4 Meeting #97-e	R4-2014780

# Electronic Meeting, 2 Nov. – 13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | DraftCR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NR V2X PSFCH demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The V2X PSFCH demodulation requirements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new requirements for PSFCH demodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

# 11.X PSFCH demodulation requirements

## 11.X.1 ACK missed detection requirements

### 11.X.1.1 General

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent. The test parameters in table 11.X.1.1-1 are configured.

Table 11.X.1.1-1: Test Parameters

| Parameter | unit | Test 1 |
| --- | --- | --- |
| Communication resource pool configuration |  | [As specified in Table A.X-X (Configuration #X-V2X)] |
| at antenna port | dBm/15kHz | -98 |
| Allocated resource blocks | RB | 1 |
| The number of PSFCH symbols | symbol | 2 |
| Number of information bits | bit | 1 |
| Synchronization source |  | GNSS |
| Timing offset | Ts | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| sl-PSFCH-Period-r16 | slot | [1] |
| sl-NumMuxCS-Pair-r16 |  | [1] |
| sl-MinTimeGapPSFCH-r16 | slot | [2] |
| sl-PSFCH-HopID-r16 |  | [0/disable] |
| sl-PSFCH-CandidateResourceType-r16 |  | [‘startSubCH’] |

### 11.X.1.2 Minimum requirements

The ACK missed detection probability shall not exceed [1%] at the SNR given in table 11.X.1.2.1.

Table 11.X.1.2-1: Minimum requirements

| Test num. | Bandwidth | Subcarrier spacing | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ACK missed detection probability (%) | SNR (dB) |
| 1 | 20 MHz | 30kHz | [TDLA30-180] | [1] | TBD |

## 11.X.2 NACK to ACK requirements

### 11.X.2.1 General

The NACK to ACK detection probability is the probability that an ACK bit is falsely detected when an NACK bit was sent on the particular bit position. The test parameters in table 11.X.1.1-1 are configured.

### 11.X.2.2 Minimum requirements

The NACK to ACK probability shall not exceed [0.1%] at the SNR given in table 11.X.2.2.2.

Table 11.X.2.2-2: Minimum requirements

| Test num. | Bandwidth | Subcarrier spacing | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NACK to ACK probability (%) | SNR (dB) |
| 1 | 20 MHz | 30kHz | [TDLA30-180] | [0.1] | TBD |

## 11.X.3 DTX to ACK requirements

### 11.X.3.1 General

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent:

ProbPSFCH DTX→Ack bits= #(false ACK bits)#PSFCH DTX*#(ACK/NACK bits)

where:

-	#(false ACK bits) denotes the number of detected ACK bits.

-	#(ACK/NACK bits) denotes the number of encoded bits per slot

-	#(PSFCH DTX) denotes the number of DTX occasions

The test parameters in table 11.X.1.1-1 are configured.

### 11.X.3.2 Minimum requirements

The DTX to ACK probability shall not exceed [1%] at the SNR given in table 11.X.2.2.3.

Table 11.X.2.2-3: Minimum requirements

| Test num. | Bandwidth | Subcarrier spacing | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | DTX to ACK probability (%) | SNR (dB) |
| 1 | 20 MHz | 30kHz | [TDLA30-180] | [0.1] | TBD |

<< End of Change >>