3GPP RAN WG4 Meeting #97e		R4-2014892

Electronic meeting, 2nd – 13th November 2020

Agenda item:	13.2.1.1

Source:	Apple Inc.

Title:	Further considerations on the numerology and channel bandwidth sizes for 60GHz frequency range

WI/SI:	FS_NR_52_to_71GHz

Release:	Rel-17

Document for:	Discussion

# 1	Introduction

As a first step for enabling even higher frequency ranges for the NR radio system, 3GPP RAN was studying operation in frequency bands above 52.6GHz and up to 114.25GHz [1], for which the corresponding TR 38.807 [2] captured global spectrum availability, regulatory requirements, and potential use cases. After RAN#86 meeting, a new SI was agreed that aims at studying more specific technical aspects on how the NR technology can be adopted to the 60GHz frequency range [3]. Referring to the corresponding SI description, the scope has been narrowed down to the operation between 52.6 GHz and 71 GHz. Furthermore, it has been also agreed that existing waveforms will be re-used thus tasking RAN WG1 and WG4 to study "of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments".

During RAN4#96 meeting, a number of contributions concerning 60GHz numerology were submitted, general summary of which was also captured in [6], but no conclusion was made. In fact, it was noted that quite many numerology aspects depend on SCS, discussions for which in turn depend on further considerations for the phase noise model. In this discussion paper we present our further views on potential numerology for the 60GHz frequency range focusing on number of RBs, achievable spectral utilisation, and channel bandwidth.

# 2	60GHz numerology and channel bandwidth

Table 2.2-1 below makes a summary of potential numerology options based on existing and new sub-carrier spacing. For the sake of clarity, Table 2-1 presents parameters of a single component carrier, which in turn can be used by carrier aggregation to cover larger channel sizes that can cover a single MGWS channel of 2.16GHz [4] [5]. Different numerology options are grouped around the same sub-carrier spacing, comments for which are presented below. It is worth mentioning that number of RBs for higher SCS sizes are exemplary ones and are determined based on the target channel utilization of approximately 95%.

-	120kHz SCS. This option can be viewed as the baseline or the legacy option as it just leverages existing single carrier parameters available in FR2. By aggregating 5 component carrier, the whole channel bandwidth of 2.16GHz can be covered.

-	240kHz SCS. For this case we present several options with different channel bandwidth sizes ranging from 400MHz to 800MHz. Option 240_a is a direct scaling of 120_a resulting in a channel bandwidth of 800MHz. Its only downside is that two component carriers will occupy only 1600MHz, while three component carriers will be larger than 2.16GHz (one can of consider an option of aggregating 800+800+400MHz carriers). Option 240_b is an interesting option in a sense that three component carriers of 700MHz size will result in a good utilization of 2.16GHz channel. With regards to Option 240_d, even though it might look less attractive because 5 component carriers have to aggregated (as in 120_a), it will require only 2048 FFT.

-	480kHz SCS. Option 480_a is an outcome of further scaling of 120_a and 240_a allowing a single component carrier of 1600MHz. As in case of 240_a, a combination of e.g. 1600+400 carriers will be needed to cover, if needed, the whole channel. In that sense options 480_b or 480_c looks more appealing as one can aggregate 2 or 3 component carriers achieving good utilisation of the 2.16GHz channel.

-	960kHz SCS. Finally, by adopting 960kHz SCS one can adopt a single large component carrier that will fit the whole 2.16GHz channel. It also possible to consider 1GHz channel size, if needed, as further exemplified by option 960_b.

Table 2-1: Component carrier parameters

| Option | SCS (kHz) | N_rb | FFT | Transmission bandwidth (MHz) | Channel bandwidth (MHz) | Utilization (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 120_a | 120 | 264 | 3168 | 380,16 | 400 | 95,04 |
| 240_a | 240 | 264 | 3168 | 760,32 | 800 | 95,04 |
| 240_b | 240 | 230 | 2760 | 662,4 | 700 | 94,63 |
| 240_c | 240 | 164 | 1968 | 472,32 | 500 | 94,46 |
| 240_d | 240 | 132 | 1584 | 380,16 | 400 | 95,04 |
| 480_a | 480 | 264 | 3168 | 1520,64 | 1600 | 95,04 |
| 480_b | 480 | 160 | 1920 | 921,6 | 1000 | 92,16 |
| 480_c | 480 | 115 | 1380 | 662,4 | 700 | 94,63 |
| 960_a | 960 | 174 | 2088 | 2004,48 | 2100 | 95,45 |
| 960_b | 960 | 82 | 984 | 944,64 | 1000 | 94,46 |
| 960_c | 960 | 58 | 696 | 668,16 | 700 | 95,45 |

Table 2-2: 2.16GHz channel utilization for single carrier and carrier aggregation scenarios

| Option | Single carrier bandwidth (MHz) | Number of aggregated component carriers |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 |
| 120_a | 400 | 17,6% | 35,2% | 52,8% | 70,4% | 88% |
| 240_a | 800 | 35,2% | 70,4% |  |  |  |
| 240_b | 700 | 30,67% | 61,33% | 92% |  |  |
| 240_c | 500 | 21,87% | 43,73% | 65,6% | 87,47% |  |
| 240_d | 400 | 17,6% | 35,2% | 52,8% | 70,4% | 88% |
| 480_a | 1600 | 70,4% |  |  |  |  |
| 480_b | 1000 | 42,67% | 85,33% |  |  |  |
| 480_c | 700 | 30,67% | 61,33% | 92% |  |  |
| 960_a | 2100 | 92,8% |  |  |  |  |
| 960_b | 1000 | 43,73% | 87,47% |  |  |  |
| 960_c | 700 | 30,93% | 61,87% | 92,8% |  |  |

Referring back to Table 2-1 and 2-2, it is possible to make the following preliminary observations:

-	Existing FR2 numerology based on 120kHz SCS combined with carrier aggregation can be used to utilize the whole 2.16GHz channel;

-	From an individual UE perspective, it can be configured even with one or two component carriers, while the whole system can operate at the whole 2.16GHz channel configuring and scheduling UE accordingly;

-	From the numerology perspective, the only downside of 120kHz SCS is a potential scheduling overhead if a particular UE is configured with e.g. 5 component carriers;

-	Higher numerology can obviously benefit from lower scheduling overhead, but do not necessarily always reduce the number of required FFT points (see Table 2-3 below for more information);

-	As already discussed in RAN WG1, sub-carrier spacing larger than 120kHz SCS can mitigate phase noise needed for 64QAM operation;

-	Increasing further sub-carrier spacing will impact TTI size and, as a result, will impact further RF and BB timing;

-	The best channel utilization can be achieved when there is a single 2100MHz channel or three aggregated 700MHz channels. Nevertheless, two aggregated 1000MHz channels as well as aggregated 400-500MHz channels also provide a good utilization of around 88%.

Finally, Table 2.3-3 below presents a small summary of required number of FFT points for different channel sizes. As can be seen, increasing sub-carrier spacing size do not always result in the decreased number of FFT points, i.e. individual baseband processing complexity will be as high as for 120kHz SCS. As an example, with 240kHz SCS only 400MHz channel will require 2048 FFT, while remaining larger channel size will have to use 4096. Same can be observed for 480kHz SCS: only 700MHz require 2048 FFT, while larger channel sizes require 4096.

Table 2-3: Summary of required FFT

| SCS (kHz) | Component carrier channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 400 | 500 | 700 | 800 | 1000 | 1600 | 2100 |
| 120 | 4096 |  |  |  |  |  |  |
| 240 | 2048 | 4096 | 4096 | 4096 |  |  |  |
| 480 |  |  | 2048 |  | 4096 | 4096 |  |
| 960 |  |  | 1024 |  | 2048 |  | 4096 |

As can be seen from our findings, one of the key parameters that will govern remaining decisions on 60GHz numerology is which sub-carrier spacing value(s) we can consider. That in turn depends on further studies for tolerable phase noise and phase noise models.

Proposal 1:	Since the minimum/maximum channel bandwidth depends heavily on the selected SCS, the latter should be tackled first.

Proposal 2:	The candidate SCS values for the 60GHz frequency range should be discussed further based on the RAN4 and RAN1 conclusion for the phase noise models.

Proposal 3:	Accounting for different use cases and scenarios, carrier aggregation should be supported to cover larger spectrum available in the 60GHz frequency range.

# 3	Conclusions

In this discussion paper we have provided our initial view on potential numerology options for the 60GHz band. As further detailed in the paper, purely from the numerology perspective 120..480kHz sub-carrier spacing values can be sufficient for the 60GHz frequency range. However, as phase noise increases with higher frequency values, more studied are needed in RAN WG1 and WG4 to understand whether lower SCS values can be used. At the same time, higher sub-carrier spacing value(s) will have more noticeable impact to the existing design and protocols inevitably increasing time-to-market of 60GHz.

Proposal 1:	Since the minimum/maximum channel bandwidth depends heavily on the selected SCS, the latter should be tackled first.

Proposal 2:	The candidate SCS values for the 60GHz frequency range should be discussed further based on the RAN4 and RAN1 conclusion for the phase noise models.

Proposal 3:	Accounting for different use cases and scenarios, carrier aggregation should be supported to cover larger spectrum available in the 60GHz frequency range.

# 4	References

RP-182861, "Study on NR design above 52.6GHz (spectrum & regulatory)", Intel

TR 38.807, "Study on requirements for NR beyond 52.6 GHz"

RP-193259, "New SID: Study on supporting NR from 52.6GHz to 71 GHz", Intel

ITU-R M.2003-2, "Multiple Gigabit Wireless Systems in frequencies around 60 GHz"

ETSI EN 302 567, "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum"

R4-2011838, "WF on numerologies for FS_NR_52_to_71GHz", Intel