3GPP RAN WG4 Meeting #97-e		R4-2014911

Online, November 2nd – 13th, 2020

Agenda item:	10.22.5

Source:	Apple Inc.

Title:	UE RF requirements tables with channel BW dependency

WI/SI:	NR_FR1_35MHz_45MHz_BW-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

Unlike in E-UTRA where the channel bandwidth has been restricted to the set of 6 bandwidths since Rel-8 specifications [1], NR is devised with the flexibility to allow new channel bandwidths to be introduced in later releases. And up till the latest technical specifications [2], there has been 13 channel bandwidths introduced. In RAN #88-e meeting, a new WID was approved to further introduce two more channel bandwidths at 35 MHz and 45 MHz [3]. Considering that the existing UE RF requirements tables with channel bandwidth dependency is already fairly crowded such as for general NR spectrum emission mask as highlighted in the “WF on UE RF requirement for 35MHz and 45MHz channel bandwidths” approved in last RAN4 meeting [4], adding more new channel bandwidths would continue worsening the editability and readability for certain UE RF requirements based on the existing table formats. In this contribution, we propose to simplify several UE RF requirements tables where the requirements with channel bandwidth dependency can be specified with equation-based rerpesentation without explicitly writing down the number for each channel bandwidth to minimize the table size growth when new channel bandwidths are further introduced in future.

# 2	Discussion

## 2.1	General spectrum emission mask

Table 2.1-1 shows the current NR FR1 general spectrum emission mask (SEM) requirement table, recaptured from the latest technical specifications [2]. It can be seen that the table width has already exceeded the text boarders. Adding more channel bandwidths such as 35 MHz and 45 MHz based on the existing table format would be a challenge to fit all the requirements into one table. Fortunately, the general SEM requirement has been defined with a simple rule over channel bandwidth except for 5MHz channel. The requirement inside the DfOOB range is basically divided into 4 regions,

Region 1: ± (0 – 1) MHz

- For 5 MHz ≤ BWChannel < 50 MHz, requirement is -13 dBm per 1% of channel BW

- For 50 MHz ≤ BWChannel ≤ 100 MHz, requirement is -24 dBm/30 kHz

Region 2: ± (1 – 5) MHz

- For all channel BWs, requirement is -10 dBm/1 MHz

Region 3: ± (5 – BWChannel) MHz

- For all channel BWs, requirement is -13 dBm/1 MHz

Region 4: ± (BWChannel – BWChannel + 5) MHz

- For all channel BWs, requirement is -25 dBm/1 MHz

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 |  | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

Table 2.1-1 General NR spectrum emission mask recaptured from Table 6.5.2.2-1 in TS 38.101-1

By following the general SEM requirement rule over channel bandwidth, the requirement table can be simplified to as shown in Table 2.1-2 where new channel bandwidths can be easily incorporated into the requirement table without noticeably expanding the table size.

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10, 15, 20, 25, 30, 40 (MHz) | 50, 60, 70, 80, 90, 100 (MHz) | Measurement bandwidth |
| ± 0-1 | -13 | -13 |  | 1 % channel bandwidth |
| ± 0-1 |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -13 |  |  |  |
| ± 6-10 | -25 |  |  |  |
| ± 5-BWChannel |  | -13 |  |  |
| ± BWChannel- BWChannel+5 |  | -25 |  |  |

Table 2.1-2 Simplified table format for general NR spectrum emission mask

Proposal 1: General NR spectrum emission mask requirement table is modified to as shown in Table 2.1-2

## 2.2	ACS requirements

As ACS requirements are either logarithmically scaled with channel bandwidth for bands < 2700 MHz or a constant for bands ≥ 3300 MHz, the requirements tables can be simplified as illustrated below.

Table 2.2-1 shows the current NR FR1 ACS for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.5-1.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | 33 | 33 | 30 | 27 | 26 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| ACS | dB | 25.5 | 24 | 23 | 22.5 | 21 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 20.5 | 20 |  |  |  |

Table 2.2-1 NR ACS for bands < 2700 MHz recaptured from TS 38.101-1 Table 7.5-1

The table can be simplified using the equation-based representation for channel bandwidth ≥ 20 MHz, as shown in Table 2.2-2.

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |

Table 2.2-2 Simplified table format for NR ACS for bands < 2700 MHz

Table 2.2-3 shows the current NR FR1 ACS for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.5-2.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 33 | 33 |  |  |  |

Table 2.2-3 NR ACS for bands ≥ 2700 MHz recaptured from TS 38.101-1 Table 7.5-2

The table can be simplified to as shown in Table 2.2-4.

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 |

Table 2.2-4 Simplified table format for NR ACS for bands ≥ 3300 MHz

Table 2.2-5 shows the current NR FR1 ACS case 1 test parameters for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.5-3 (without notes). The table can be simplified using equation-based representation as shown in Table 2.2-6.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 dB | REFSENS + 38.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 38 dB | REFSENS + 36.5 dB | REFSENS + 35.5 dB | REFSENS + 35 dB | REFSENS + 33.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 33 dB | REFSENS + 32.5 dB |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |

Table 2.2-5 NR ACS case 1 test parameters for bands < 2700 MHz from TS 38.101-1 Table 7.5-3

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |

Table 2.2-6 Simplified table format for NR ACS case 1 test parameters for bands < 2700 MHz

Table 2.2-7 shows the current NR FR1 ACS case 2 test parameters for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.5-4 (without notes). The table can be simplified using equation-based representation as shown in Table 2.2-8.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -49.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | -49 | -47 | -46.5 | -46 | -44.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | -44 | -43.5 |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |

Table 2.2-7 NR ACS case 2 test parameters for bands < 2700 MHz from TS 38.101-1 Table 7.5-4

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | -56.5 | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -25 |  |  |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |

Table 2.2-8 Simplified table format for NR ACS case 2 test parameters for bands < 2700 MHz

Table 2.2-9 shows the current NR FR1 ACS case 1 test parameters for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.5-5 (without notes). The table can be simplified using equation-based representation as shown in Table 2.2-10.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 | 30 / -30 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 45.5 dB |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| Finterferer (offset) | MHz | 40 / -40 | 50 / -50 | 60 / -60 | 70 / -70 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| Finterferer (offset) | MHz | 100 / -90 | 100 / -100 |  |  |  |

Table 2.2-9 NR ACS case 1 test parameters for bands ≥ 3300 MHz from TS 38.101-1 Table 7.5-5

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |

Table 2.2-10 Simplified table format for NR ACS case 1 test parameters for bands ≥ 3300 MHz

Table 2.2-11 shows the current NR FR1 ACS case 2 test parameters for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.5-5 (without notes). The table can be simplified using equation-based representation as shown in Table 2.2-12.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| Finterferer (offset) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 25 / -25 | 30 / -30 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 | -25 |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| Finterferer (offset) | MHz | 40 / -40 | 50 / -50 | 60 / -60 | 70 / -70 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| Finterferer (offset) | MHz | 90 / -90 | 100 / -100 |  |  |  |

Table 2.2-11 NR ACS case 2 test parameters for bands ≥ 3300 MHz from TS 38.101-1 Table 7.5-6

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |

Table 2.2-12 Simplified table format for NR ACS case 2 test parameters for bands ≥ 3300 MHz

Proposal 2: NR FR1 ACS requirements tables are modified to as shown in Table 2.2-2, Table 2.2-4, Table 2.2-6, Table 2.2-8, Table 2.2-10, and Table 2.2-12.

## 2.3	Blocking characteristics

### 2.3.1	In-band blocking requirements

Table 2.3.1-1 shows the current NR FR1 in-band blocking test parameters for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.6.2-1 (without notes). The table can be simplified using equation-based representation as shown in Table 2.3.1-2.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |

Table 2.3.1-1: NR in-band blocking test parameters for bands < 2700 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | dBm | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |

Table 2.3.1-2: Simplified table format for NR in-band blocking test parameters for bands < 2700 MHz

Table 2.3.1-3 shows the current NR FR1 in-band blocking test parameters for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.6.2-3 (without notes). The table can be simplified using equation-based representation as shown in Table 2.3.1-4.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 25 | 30 |
| FIoffset, case 1 | MHz | 15 | 22.5 | 30 | 37.5 | 45 |
| FIoffset, case 2 | MHz | 25 | 37.5 | 50 | 62.5 | 75 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 40 | 50 | 60 | 70 | 80 |
| FIoffset, case 1 | MHz | 60 | 75 | 90 | 105 | 120 |
| FIoffset, case 2 | MHz | 100 | 125 | 150 | 175 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 90 | 100 |  |  |  |
| FIoffset, case 1 | MHz | 135 | 150 |  |  |  |
| FIoffset, case 2 | MHz | 225 | 250 |  |  |  |

Table 2.3.1-3: NR in-band blocking test parameters for bands ≥ 3300 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB |
| BWinterferer | dBm | BWChannel |
| FIoffset, case 1 | MHz | (3/2) BWChannel |
| FIoffset, case 2 | MHz | (5/2) BWChannel |

Table 2.3.1-4: Simplified table format for NR in-band blocking test parameters for bands ≥ 3300 MHz

Proposal 3: NR FR1 in-band blocking requirements tables are modified to as shown in Table 2.3.1-2 and Table 2.3.1-4.

### 2.3.2	Out-of-band blocking requirements

Table 2.3.2-1 shows the current NR FR1 out-of-band blocking test parameters for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.6.3-1 (without notes). The table can be simplified using equation-based representation as shown in Table 2.3.2-2.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |

Table 2.3.2-1: NR out-of-band blocking test parameters for bands < 2700 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |

Table 2.3.1-2: Simplified table format for NR out-of-band blocking parameters for bands < 2700 MHz

Table 2.3.2-3 shows the current NR FR1 out-of-band blocking test parameters for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.6.3-3 (without notes). The table can be simplified as shown in Table 2.3.2-4.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 |  |  |  |

Table 2.3.2-3: NR out-of-band blocking test parameters for bands ≥ 3300 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |

Table 2.3.1-4: Simplified table format for NR out-of-band blocking parameters for bands ≥ 3300 MHz

Proposal 4: NR FR1 out-of-band blocking requirements tables are modified to as shown in Table 2.3.2-2 and Table 2.3.2-4.

### 2.3.3	Narrow band blocking

Table 2.3.3-1 shows the current NR FR1 narrow band blocking requirements with image captured from the latest technical specifications [2] Table 7.6.4-1. The image has been shrunk to fit into the width of portrait layout. The table can be simplified using equation-based representation as shown in Table 2.3.2-2.

Table 2.3.3-1: NR narrow band blocking requirements from TS 38.101-1 Table 7.6.4-1

| Band | RX parameter | Units | Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74, n75, n76n | Pw | dBm | REFSENS + channel BW specific value below |  |  |  |  |
|  |  |  | 16 | 13 | 14 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset, SCS = 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  | NA |
|  | Fuw (offset, SCS = 30 kHz) | MHz | NA |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |

Table 2.3.3-2: Simplified table format for NR narrow band blocking requirements

Proposal 5: NR FR1 narrow band blocking requirements table is modified to as shown in Table 2.3.3-2.

## 2.4	Spurious response

Table 2.4-1 shows the current NR FR1 spurious response test parameters for bands < 2700 MHz, recaptured from the latest technical specifications [2] Table 7.7-1 (without notes). The table can be simplified using equation-based representation as shown in Table 2.4-2.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 11 | 12 | 13 | 14 | 15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 15.5 | 16 |  |  |  |

Table 2.4-1: NR spurious response test parameters for bands < 2700 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |

Table 2.4-2: Simplified table format for NR spurious response test parameters for bands < 2700 MHz

Table 2.4-3 shows the current NR FR1 spurious response test parameters for bands ≥ 3300 MHz, recaptured from the latest technical specifications [2] Table 7.7-1a (without notes). The table can be simplified as shown in Table 2.4-4.

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 6 | 7 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |
|  | dB | 9 | 9 |  |  |  |

Table 2.4-3: NR spurious response test parameters for bands ≥ 3300 MHz from TS 38.101-1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |

Table 2.4-4: Simplified table format for NR spurious response test parameters for bands ≥ 3300 MHz

Proposal 6: NR FR1 spurious response requirements tables are modified to as shown in Table 2.4-2 and Table 2.4-4.

# 3	Conclusion

In this contribution, we propose to simplify several UE RF requirements tables to accommodate the anticipated new channel bandwidths at 35 MHz and 45 MHz and other potential new channel bandwidths in future to maintain the requirements table’s editability and readability. The requirements include,

- Spectrum emission mask – Proposal 1

- ACS – Proposal 2

- In-band blocking – Proposal 3

- Out-of-band blocking – Proposal 4

- Narrow band blocking – Proposal 5

- Spurious response – Proposal 6

# 4	References

3GPP TS 36.101 V8.1.0 (2008-03)

3GPP TS 38.101-1 V16.5.0 (2020-09)

RP-201321, “New WID on introduction of channel bandwidths 35MHz and 45MHz for NR”, Huawei, HiSilicon, Etisalat, 3GPP TSG-RAN #88-e, Online, June 29th – July 3rd, 2020

R4-2011793 “WF on UE RF requirements for 35MHz and 45MHz channel bandwidths”, Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #96-e, Online, August 17th – 28th, 2020