3GPP TSG-RAN WG4 Meeting #97-e 	R4-2015090

Electronic Meeting, 2-13 Nov., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 HST BS demodulation PUSCH requirements and simulation results

Agenda item:	7.15.3.2.1

Document for:	Discussion

Introduction

All previously known open issues on the topic of NR Rel-16 HST BS demodulation PUSCH requirements have been resolved at the RAN4#96e meeting. Results are captured in the email discussion summary [1], WF [2], and the meeting minutes (some agreements from the last go-to-webinar).
For ease of reading and referencing, the agreements from the last GtW are captured hereunder:

- Specification drafting of multi-path fading requirements - Introduce multi-path fading channel requirements with high Doppler value in a separate table under section “8.2.4 Requirements for PUSCH for high speed train” - This requirement only applicable for wide-area, medium-range BS which supporting HST

Since the last meeting, we have identified the following topics that still need discussion:

- Simulation results misalignment (span too large) in [3].

- Multi-path carrier frequency.

- Applicability rules not implemented in spec.

In this contribution we will express our views on the above issues.
Additionally, we include the results of our remaining simulation campaigns (multi-path fading channel under high Doppler) directly in this Tdoc.

Discussion on open issues

Here we discuss open issues, left over from the last meeting.

Simulation results misalignment

We noticed the following uncertain result in the summary excel of 350kph PUSCH [3]:

Figure 1: Simulation alignment issue.

One company’s impairment result is better than the corresponding ideal one for 30kHz/10MHz, 2334Hz, 1T8R, MCS16. This causes a -102 error (span too large).

In case the identified cases in the simulation summary excel are not updated in this meeting, remove them from the requirement calculation and move ahead with replacing TBDs in the CRs.

Multi-path carrier frequency

In discussions about PUSCH multi-path fading channel models with high Doppler values, it was neglected to discuss the centre/carrier frequencies to be used for simulations with the respective fading scenarios.

We have used the following values for our evaluations:

Table 1: Used carrier frequencies

| Propagation condition | fc (GHz) | SCS | CBW |
| --- | --- | --- | --- |
| TDLC300-600 FO=0Hz | 2.1 | 15KHz | 5MHz |
| TDLC300-1200 FO=0Hz | 3.6 | 30KHz | 10MHz |

If required for simulation alignment, RAN4 to consider fc=2.1GHz for TDLC300-600 FO=0Hz (15kHz), and fc=3.6GHz for TDLC300-1200 FO=0Hz (30kHz).

Applicability rules not implemented in spec

While verifying the CR implementation status of the latest specifications, we have noticed that some previously agreed PUSCH applicability rules have not been included.

In particular the agreements from RAN4#95e [R4-2008821] seem to be missing from TS 38.141-1 and TS 38.141-2:

- PUSCH implicit test passing applicability rule - Capture the following applicability rule in test specifications: - “Unless otherwise stated, a BS that declares to support 500km/h (see D.1XX in table 4.6-1) and passes the tests for 500km/h, can also consider the tests for 350km/h as passed.” - PUSCH 1T1R applicability rule - Applicability rule: In high speed train requirements, unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), if the BS supports 1RX, the tests with low MIMO correlation level shall apply only for either one connector or the second lowest number of supported connectors, in addition to the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. If the BS doesn’t support 1RX, the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. - Remark on the applicability rule: The highest number of connectors can simultaneously be second lowest number.

The implicit test passing and 1T1R applicability rules from RAN4#95e have yet to be included in TS 38.141-1 and TS 38.141-2.

The companies responsible for PUSCH HST CRs to TS 38.141-1 and TS 38.141-2, are requested to include the implicit test passing and 1T1R applicability rules in this meeting.

Simulation results

In RAN4#96-e the propagation conditions for “multi-path fading channel under high Doppler” have been clarified [2]:

- Multi-path fading scenarios under consideration - The following models are under consideration: - TDLC300-600 FO=0Hz TDLC300-1200 FO=0Hz - Specification of multi-path fading channel under high Doppler - Introduce PUSCH requirements for HST multi-path fading channel under high Doppler, with configuration - TDLC300-600 FO=0Hz (15kHz), TDLC300-1200 FO=0Hz (30kHz) - MCS: 2 - Waveform: CP-OFDM - DM-RS: 1+1+1 - Antenna: 1T2R - Bandwidth: 5MHz for 15kHz SCS, 10MHz for 30kHz SCS

Following this we update the previously delivered simulation results.

Parameters

Unless otherwise stated in the following (or the above multi-path clarifications), the simulation setup follows the HST PUSCH baseline as captured in the WF [2]:

Table 1: PUSCH HST baseline configuration

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | v = 350km/h |  | v = 500km/h |  |  |
| Transform precoding | Disabled FFS: Enabled |  | Disabled FFS: Enabled |  |  |
| Number of Tx | 1 |  | 1 |  |  |
| Number of Rx | Tunnel: 2, 1 Open space: 2, 8 |  | Tunnel: 2, 1 Open space: 2, 8 |  |  |
| Number of layers | 1 |  | 1 |  |  |
| SCS | 15kHz, 30kHz |  | 15kHz, 30kHz |  |  |
| Reference signal | DMRS type 1 with 1+1+1 |  | DMRS type 1 with 1+1+1 |  |  |
| l0 | l0 = 2 or 3 |  | l0 = 2 or 3 |  |  |
| symbols length | 14 |  | 14 |  |  |
| start symbol index | 0 |  | 0 |  |  |
| Time domain resource allocation type | type A |  | type A |  |  |
| Frequency domain resource | Full applicable test bandwidth |  | Full applicable test bandwidth |  |  |
| MCS index | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 MCS2 and [16] for 1T1R |  | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 MCS2 and [16] for 1T1R |  |  |
| Maximum Doppler shift | 15kHz SCS: 1340Hz | 30kHz SCS: 2334Hz | 15kHz SCS: 1740Hz |  | 30kHz SCS: 3334Hz |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |  | 15kHz SCS: 1.8GHz (Band n3) 30kHz SCS: 3.6GHz |  |  |
| Propagation condition | HST single-tap channel model: Tunnel: Ds=300m, Dmin=2m Open space: Ds=700m, Dmin=150m FFS: Multi-path fading channel under high Doppler value 15kHz: [600]Hz; 30kHz: [1200]Hz |  |  |  |  |
| SCS and BW | 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz |  |  |  |  |
| Number of HARQ transmissions | 4 |  |  | 4 |  |
| Testing metric | SNR @70% of maximum throughput |  |  | SNR @70% of maximum throughput |  |

Multipath with high Doppler

Here are our simulation results for the multi-path fading channel model with high Doppler values:

Table 4: PUSCH, CP-OFDM, Multi-path, Type A TDRA: simulation setup and results summary

| Propagation condition | fc (GHz) | SCS | CBW | Tx/Rx | PUSCH mapping type | DMRS | l0 | MCS | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| TDLC300-600 FO=0Hz | 2.1 | 15KHz | 5MHz | 1T2R | Type A | 1+1+1 | 2 or 3 | 2 | -4.61 | -2.11 |
| TDLC300-1200 FO=0Hz | 3.6 | 30KHz | 10MHz | 1T2R | Type A | 1+1+1 | 2 or 3 | 2 | -4.63 | -2.13 |

Conclusion

In this contribution we have provided our views on various open PUSCH HST issues. In particular, simulation results misalignment, multi-path carrier frequency, and missing agreed applicability rules in the specifications. Additionally, we have delivered the results of our simulation campaign on multi-path fading channel models with high Doppler values.
We have made the following observations and proposals:

Simulation results misalignment

- One company’s impairment result is better than the corresponding ideal one for 30kHz/10MHz, 2334Hz, 1T8R, MCS16. This causes a -102 error (span too large).

- In case the identified cases in the simulation summary excel are not updated in this meeting, remove them from the requirement calculation and move ahead with replacing TBDs in the CRs.

Multi-path carrier frequency

If required for simulation alignment, RAN4 to consider fc=2.1GHz for TDLC300-600 FO=0Hz (15kHz), and fc=3.6GHz for TDLC300-1200 FO=0Hz (30kHz).

Applicability rules not implemented in spec

The implicit test passing and 1T1R applicability rules from RAN4#95e have yet to be included in TS 38.141-1 and TS 38.141-2.

The companies responsible for PUSCH HST CRs to TS 38.141-1 and TS 38.141-2, are requested to include the implicit test passing and 1T1R applicability rules in this meeting.

- References

- R4-2012739, Email discussion summary for [96e][323] NR_HST_Demod_BS, Moderator (Nokia, Nokia Shanghai Bell), RAN4#96-e.

- R4-2012676, WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- R4-2012749, Summary of ideal and impairment results for NR HST demodulation requirements, CATT, RAN4#96-e.