3GPP TSG-RAN WG4 Meeting #97-e 	R4-2015093

Electronic Meeting, 2-13 Nov., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 HST BS demodulation UL TA requirements and simulation results

Agenda item:	7.15.3.2.3

Document for:	Discussion

Introduction

Most previously known open issues on the topic of NR Rel-16 HST BS demodulation UL TA requirements have been resolved at the RAN4#96e meeting. Results are captured in the email discussion summary [1], and WF [2].

The following topics are still open in the WF:

- SCS/CBW combinations

- Applicability rules concerning SCS/CBW

We did not identify any further open issues since the last meeting.

In this contribution we will express our views on the above issues.
Additionally, we include the results of our remaining simulation campaigns (scenario X and SCS/CBW combinations) directly in this contribution.

Discussion on open issues

Here we discuss open issues, left over from the last meeting.

SCS/CBW combinations

One of the very few unanswered questions from the last meeting, are the SCS/CBW combinations to be specified [2]:

- SCS/CBW combinations - Option 1: Have requirements for - 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz. - Option 2: Have requirements for - 15kHz: 5MHz; 30kHz: 10MHz. - Option 3: Have requirements for all Rel-15 non-HST PUSCH bandwidths. - Other options not precluded.

Introducing new mandatory tests for all CBS/SCS combinations introduced in Rel-15 NR_newRAT seems to be an excessive increase in testing load and cost.
For scenarios Y and Z, we used the combinations 15/5, 15/10, 30/10, 30/40.
Implicit passing for scenario X was also agreed (see next section), so the combinations covered in scenario Y and Z should be re-used.
Hence we support to have scenario X requirements for 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

Introducing new mandatory tests with scenario X for all SCS/CBW combinations introduced in Rel-15 NR_newRAT adds a non-negligible testing load. For scenarios Y and Z, we used the combinations 15/5, 15/10, 30/10, 30/40. Implicit passing for scenario X was also agreed.

RAN4 to consider having scenario X requirements for 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

Applicability rules for SCS/CBW combinations and implicit test passing.

The second unanswered question from the last meeting, are the exact applicability rules concerning SCS/CBW combinations [2]:

- Applicability rules concerning SCS/CBW - Option 1: Re-use non-HST PUSCH applicability rules. - Option 2: Re-use HST PUSCH applicability rules. - Other options not precluded.

We remind that implicit test passing was also agreed [2]:

- RAN4 agree to introduce scenario X requirements under rel-16 HST WI, adding it in non-HST sections/tables to avoid misleading. - A BS, which declares to support HST and passes scenario Y or scenario Z, can assume implicit test passing of scenario X.

Following the previous agreements to have implicit test passing, to have the requirements in non-HST sections, and considering the options from the WF, we propose the following applicability rule text in 38.141-1 section 8.2.5.1 (similar for 38.141-2):

“Unless otherwise stated, a BS that declares to support PUSCH HST (see D.109 in table 4.6-1) and passes the tests for scenario Y or scenario Z, can also consider the tests for scenario X as passed.”

We don’t make a mention of the SCS/CBW combinations in this text, as the specification (TS 38.141-1) already states:

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.1.

I.e., all non-HST PUSCH applicability rules are re-used.

RAN4 to apply the non-HST PUSCH applicability rules to SCS/CBW combinations, i.e., not to modify the current specification text in this regard.

RAN4 to capture the implicit test passing agreement for scenario X, as follows:
“Unless otherwise stated, a BS that declares to support PUSCH HST (see D.1XX in table 4.6-1) and passes the tests for scenario Y or scenario Z, can also consider the tests for scenario X as passed.”

Simulation results

In RAN4#96-e the propagation conditions for “multi-path fading channel under high Doppler” have been clarified [2]:

- Multi-path fading scenarios under consideration - The following models are under consideration: - TDLC300-600 FO=0Hz TDLC300-1200 FO=0Hz - Specification of multi-path fading channel under high Doppler - Introduce PUSCH requirements for HST multi-path fading channel under high Doppler, with configuration - TDLC300-600 FO=0Hz (15kHz), TDLC300-1200 FO=0Hz (30kHz) - MCS: 2 - Waveform: CP-OFDM - DM-RS: 1+1+1 - Antenna: 1T2R - Bandwidth: 5MHz for 15kHz SCS, 10MHz for 30kHz SCS

Following this we update the previously delivered simulation results.

Parameters

Unless otherwise stated in the following, the simulation setup follows the HST UL TA baseline as last outlined in [4] and amended with agreements in [5]:

Table 1: HST UL TA baseline configuration

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Channel bandwidth |  | 15 kHz SCS: 10 MHz 30 kHz SCS: 40 MHz |
| MCS |  | 16 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 10MHz CBW: 25 RB for each UE 40MHz CBW: 50 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 25 for 10 MHz CBW, and 50 for 40 MHz CBW |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | For FDD: slot #1 in radio frames For TDD: - last symbol in slot #3 in radio frames for 15KHz - last symbol in slot #7 in radio frames for 30KHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS = 11, BSRS =0, for 40 RB 30 kHz SCS: CSRS = 21, BSRS =0, for 80 RB |
| Note 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

The specific scenarios (X, Y, Z) are then defined as follows:

Table 2: UL TA HST baseline scenarios

| Parameter | Scenario X | Scenario Y | Scenario Z |
| --- | --- | --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 | Stationary UE: AWGN Moving UE: AWGN | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 120 km/h | 350 km/h | 500 km/h |
| CP length | Normal |  |  |
| A | 15 kHz: 10 s 30 kHz: 5 s | 15kHz: 10 µs 30kHz: 5 µs | 15 kHz: 10 s 30 kHz: 5 s |
| Δω | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 | 15kHz: 0.13 s-1 30kHz: 0.26 s-1 | 15 kHz: 0.18 s-1 30 kHz: 0.36 s-1 |

Scenario X

Here are our simulation results for the (new) scenario X cases:

Table 3: PUSCH UL TA, 500kph, CP-OFDM, 1T2R, Scenario X: simulation results summary

| PUSCH transmission | SRS transmission | CBW | SCS | DMRS | l0 | MCS | A | Δω | PUSCH mapping type | SNR (dB) @ 70% TPUT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impaired |
| FDD | Disabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.04s-1 | Type A | 7.71 | 10.21 |
|  |  | 10MHz | 15KHz | 1+1+1 | 0 | 16 | 10us | 0.04s-1 | Type B | 7.72 | 10.22 |
|  | Disabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.08s-1 | Type A | 7.68 | 10.18 |
|  |  | 40MHz | 30KHz | 1+1+1 | 0 | 16 | 5us | 0.08s-1 | Type B | 7.68 | 10.18 |
| FDD | Disabled | 5MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.04s-1 | Type A | 7.31 | 9.81 |
|  |  | 5MHz | 15KHz | 1+1+1 | 0 | 16 | 10us | 0.04s-1 | Type B | 7.32 | 9.82 |
|  | Disabled | 10MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.08s-1 | Type A | 7.42 | 9.92 |
|  |  | 10MHz | 30KHz | 1+1+1 | 0 | 16 | 5us | 0.08s-1 | Type B | 7.42 | 9.92 |

Scenario Y

Here are our simulation results for the (new CBW) scenario Y cases:

Table 3: PUSCH UL TA, 500kph, CP-OFDM, 1T2R, Scenario Y: simulation results summary

| PUSCH transmission | SRS transmission | CBW | SCS | DMRS | l0 | MCS | A | Δω | PUSCH mapping type | SNR (dB) @ 70% TPUT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impaired |
| FDD | Disabled | 5MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type A | 5.56 | 8.06 |
|  |  | 5MHz | 15KHz | 1+1+1 | 0 | 16 | 10us | 0.13s-1 | Type B | 5.56 | 8.06 |
|  | Disabled | 10MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type A | 5.56 | 8.06 |
|  |  | 10MHz | 30KHz | 1+1+1 | 0 | 16 | 5us | 0.26s-1 | Type B | 5.56 | 8.06 |

Scenario Z

Here are our simulation results for the (new CBW) scenario Z cases:

Table 3: PUSCH UL TA, 500kph, CP-OFDM, 1T2R, Scenario Z: simulation results summary

| PUSCH transmission | SRS transmission | CBW | SCS | DMRS | l0 | MCS | A | Δω | PUSCH mapping type | SNR (dB) @ 70% TPUT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal | Impaired |
| FDD | Disabled | 5MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.18s-1 | Type A | 5.57 | 8.07 |
|  |  | 5MHz | 15KHz | 1+1+1 | 0 | 16 | 10us | 0.18s-1 | Type B | 5.57 | 8.07 |
|  | Disabled | 10MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.36s-1 | Type A | 5.57 | 8.07 |
|  |  | 10MHz | 30KHz | 1+1+1 | 0 | 16 | 5us | 0.36s-1 | Type B | 5.57 | 8.07 |

Conclusion

In this contribution we have provided our views on various open UL TA HST issues. In particular, SCS/CBW combinations, and applicability rules for SCS/CBW combinations and implicit test passing.
Additionally, we have delivered the results of our simulation campaign on multi-path fading channel models with high Doppler values.
We have made the following observations and proposals:

SCS/CBW combinations

- Introducing new mandatory tests with scenario X for all SCS/CBW combinations introduced in Rel-15 NR_newRAT adds a non-negligible testing load. For scenarios Y and Z, we used the combinations 15/5, 15/10, 30/10, 30/40. Implicit passing for scenario X was also agreed.

- RAN4 to consider having scenario X requirements for 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz.

Applicability rules for SCS/CBW combinations and implicit test passing.

RAN4 to apply the non-HST PUSCH applicability rules to SCS/CBW combinations, i.e., not to modify the current specification text in this regard.

RAN4 to capture the implicit test passing agreement for scenario X, as follows:
“Unless otherwise stated, a BS that declares to support PUSCH HST (see D.1XX in table 4.6-1) and passes the tests for scenario Y or scenario Z, can also consider the tests for scenario X as passed.”

- References

- R4-2012739, Email discussion summary for [96e][323] NR_HST_Demod_BS, Moderator (Nokia, Nokia Shanghai Bell), RAN4#96-e.

- R4-2012676, WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- R4-2012749, Summary of ideal and impairment results for NR HST demodulation requirements, CATT, RAN4#96-e.

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon, RAN4#93.

- R4-2002405, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#94-e.