3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2015109

# Electronic Meeting, 2-13 Nov, 2020

Source:		Ericsson

Title:			Discussion on IAB EMC Radiated Emissions

Agenda item:		7.4.6.2

Document for:	Approval

# Introduction

IAB (Integrated Access and Backhaul) is essentially about NR self-backhauling, i.e. using NR, the new 5G radio-access technology, not only for the access link, but also as a wireless backhaul technology (instead of fiber) to enable flexible and very dense deployment of cells without the need for pulling fiber to new sites. In the following sections we provide our view on IAB EMC radiated emissions.

# Discussion

## EMC Radiated Emissions

EMC emissions testing covers both conducted and radiated emissions.  EMC conducted emission testing is performed to assess the electromagnetic energy levels versus frequency conducted by product cables (power lines, network cables, etc). EMC radiated emissions test involves measuring the electromagnetic field strength of the emissions that are unintentionally generated by a product. Emissions are inherent to the switching voltages and currents within any digital circuit. In this case, the radiated emission requirements as well as the test methods that apply are defined by IEC and CISPR subcommittee in the Standard CISPR 32 [1]. Testing and limits of emissions intentionally generated by the EUT are covered by ITU-R recommendations SM.329 [2] and SM.1539 [3].

## EMC Radiated Emissions Applicability in 3GPP

It has been agreed in TR 38.809 [4] that for IAB type 1-O and BS type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.174 [5]. At the same time, TS 38.174 [5] has agreed on the definition of the spurious emissions limits applicable for IAB type 1-H. With this consideration, the emission requirement for the IAB node should follow the same principle implemented in TS 38.113: This is reusing the limits defined in the RF area to set the limits applicable for EMC radiated emissions.

Observation 1: 3GPP has agreed that for IAB type 1-O and type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.174 [5].

Observation 2: 3GPP has agreed on the definition of spurious emission for IAB node. Reusing the limits defined in the IAB RF spec in the IAB EMC specification is a reasonable approach. Same principle applied for NR BS EMC specification.

# Conclusion

In this contribution considerations on the IAB EMC Radiated Emission have been presented. The following observations have been discussed:

Observation 1: 3GPP has agreed that for IAB type 1-O and type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.174 [5].

Observation 2: 3GPP has agreed on the definition of spurious emission for IAB node. Reusing the limits defined in the IAB RF spec in the IAB EMC specification is a reasonable approach. Same principle applied for NR BS EMC specification.

Based on these elements we propose:

Proposal 1: To define the IAB EMC Radiated Emissions requirements for IAB type 1-H reusing the ones already set in the IAB RF specifications.

Proposal 2: : To agree on the companion CR to TS 38.175 [6] on IAB EMC emission requirements.

# References

CISPR 32. Electromagnetic compatibility of multimedia equipment - Emission requirements.

Recommendation ITU-R SM.329-12: "Unwanted emissions in the spurious domain

Recommendation ITU-R SM.1539-1: "Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329"

3GPP TR 38.809. “NR; Background for Integrated access and backhaul radio transmission and reception.”

TS 38.174 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Integrated access and backhaul radio transmission and reception.

R4-2015110. CR to TS 38.175 on IAB EMC Emission. Ericsson