3GPP TSG-RAN WG4 Meeting #97e	R4-2015346

Electronic Meeting, 2-13 Nov., 2020

Title:	Discussion on inter-band CA HPUE SAR

Source:	OPPO

Agenda item:	10.18.2

Document for:	Approval

- Introduction

WI [1] was approved, in which inter-band CA SAR issue is one of the topic. Initial discussions happen in RAN4#96e, however, no conclusion was reached.

This paper continue discuss this aspect.

- Discussion

In [2], it pointed out that for EN-DC combinations, E-UTRA is MCG and is prioritized so the duty cycle solution is reporting SCG’s duty cycle according to MCG’s duty cycle. And for inter-band 2UL CA, PCC/SCC configuration is UE specific rather than fixed prioritization. Therefore, a solution for inter-band UL CA is proposed in [2] is that each carrier report UL duty cycle capability according to the other carrier’s configuration.

Following the approach, UE reports a group of combined capabilities for Band X + Band Y, as below:

- UE reports (X1%, Y1%), (X2%, Y2%), (X3%, Y3%)…, (Xn%, Yn%)…, in which X1<X2<X3…<Xn…; - When Band X is the PCC - The scheduled duty cycle for Band X is between Xn-1% and Xn% - Then capability (Xn%, Yn%) is taken. - Only when the scheduled duty cycle on Band Y is below Yn% then the PC2 is applied. - When Band Y is the PCC - The scheduled duty cycle for Band Y is between Yn-1% and Yn% - Then capability (Xn-1%, Yn-1%) is taken. - Only when the scheduled duty cycle on Band X is below Xn-1% then the PC2 is applied.

From above example it seems the solution is doable.

Observation 1:   Reporting of combined Band X +Band Y duty cycle capability is a possible way for inter-band UL CA HPUE SAR issues.

Proposal 1:        It is proposed to consider reporting a group of combined maxUplinkdutycycle capabilities for inter-band UL CA HPUE SAR issue.

- Conclusion

Observation 1:   Reporting of combined Band X +Band Y duty cycle capability is a possible way for inter-band UL CA HPUE SAR issues.

Proposal 1:        It is proposed to consider reporting a group of combined maxUplinkdutycycle capabilities for inter-band UL CA HPUE SAR issue.

Reference

[1] R4-2011789, WF on UE PC2 for NR inter-band CA and SUL configurations, China Telecom

[2] R4-2009796, Discussion on SAR solution for interband PC2 2UL CA, CATT