3GPP TSG-RAN4 Meeting #97-e	R4-2015370

# Electronic meeting, Nov. 2-13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on conditions for NR RSTD measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conditions for NR RSTD measurement need to be defined when specifying the performance requirements for RSTD measurement in 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the conditions for NR RSTD measurement in 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditions for NR RSTD measurement are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.10, B.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

B.2.10	Conditions for NR RSTD intra-frequency measurements

This clause defines the following conditions for NR RSTD intra-frequency measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.10-1 for FR1 NR cells.

The conditions are defined in Table B.2.10-2 for FR2 NR cells.

Table B.2.10-1: Conditions for NR RSTD intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  | dB |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -134 | -131 | -13 |
|  | NR_FDD_FR1_B | -133.5 | -130.5 |  |
|  | NR_TDD_FR1_C | -133 | -130 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -132.5 | -129.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -132 | -129 |  |
|  | NR_FDD_FR1_F | -131.5 | -128.5 |  |
|  | NR_FDD_FR1_G | -131 | -128 |  |
|  | NR_FDD_FR1_H | -130.5 | -127.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.10-2: Conditions for NR RSTD intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -135.3+Y1 | -120.8 | -119.1 | -134.8+Y4 | (Value for SCSPRS = 120 kHz) +3dB | ≥-13 |
|  |  | n258 | -135.3+Y1 | -120.8 | -119.1 | -134.8+Y4 |  |  |
|  |  | n259 |  |  | -115.5 |  |  |  |
|  |  | n260 | -132.3+Y1 |  | -116.5 | -132.8+Y4 |  |  |
|  |  | n261 | -135.3+Y1 | -120.8 | -119.1 | -134.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -127.3+Z1 | -109.8 | -108.2 | -125.8+Z4 | (Value for SCSPRS = 120 kHz) +3dB | ≥-13 |
|  |  | n258 | -127.3+Z1 | -109.8 | -108.2 | -125.8+Z4 |  |  |
|  |  | n259 |  |  | -102.7 |  |  |  |
|  |  | n260 | -124.3+Z1 |  | -103.9 | -120.8+Z4 |  |  |
|  |  | n261 | -127.3+Z1 | -109.8 | -108.2 | -125.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.10-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

B.2.11	Conditions for NR RSTD inter-frequency measurements

This clause defines the following conditions for NR RSTD inter-frequency measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.11-1 for FR1 NR cells.

The conditions are defined in Table B.2.11-2 for FR2 NR cells.

Table B.2.11-1: Conditions for RSTD inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  | dB |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -132 | -129 | -13 |
|  | NR_FDD_FR1_B | -131.5 | -128.5 |  |
|  | NR_TDD_FR1_C | -131 | -128 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -131.5 | -127.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -130 | -127 |  |
|  | NR_FDD_FR1_F | -129.5 | -126.5 |  |
|  | NR_FDD_FR1_G | -129 | -126 |  |
|  | NR_FDD_FR1_H | -128.5 | -125.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.11-2: Conditions for RSTD inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum PRP1,2 Note 2, Note 3 |  |  |  |  | PRS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSPRS |  |  |  |  | dB |
|  |  |  | SCSPRS = 120 kHz |  |  |  | SCSPRS = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSPRS = 120 kHz) +3dB | ≥-13 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSPRS = 120 kHz) +3dB | ≥-13 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum PRS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.11-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1, and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

<End of Change 1>