3GPP TSG-RAN WG4 Meeting # 97-e                                  R4-2015403

Electronic Meeting, 2nd – 13th November, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-11-11: DC_12A_n71A

Agenda Item:	10.3.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #89e meeting. This contribution provides a TP for TR 37.717-11-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-201553, “Revised WID: Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL)”, CHTTL

- Text Proposal

<TP for TR 37.717-11-11>

6.1.x	DC_12_n71

6.1.x.1	Configuration for DC

Table 6.1.x.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_12_n71 | DC_12_n71X | Yes |
| NOTE X: Only single switched UL is supported. |  |  |

#### 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_12_n71 X | 23 | +2/-3 |
| NOTE X: Only single switched UL is supported. |  |  |

#### 6.1.x.3	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed.

#### 6.1.x.4	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed.

#### 6.1.x.5	∆TIB and ∆RIB values

For DC_12_n71, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.x.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 0.5 |
|  | n71 | 0.5 |

Table 6.1.x.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 0.3 |
|  | n71 | 0.3 |

<End of TP >