3GPP TSG-RAN WG4 Meeting #97-e	R4-2015469

# Online, 2th– 13th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on UE Transmission Timing Accuracy Tests for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE transmission timing accuracy requirements has been specified for NR V2X, and the corresponding tests shall be defined in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduction of UE transmission timing accuracy tests for V2X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE transmission timing accuracy tests will be missing for NR V2X. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9, A.9.1, A.9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.9	V2X Tests

A.9.1	V2X Tests in FR1

A.9.1.1	V2X UE Transmit Timing Tests

A.9.1.1.1 Test for GNSS as Synchronization Reference Source

A.9.1.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE timing requirements as specified in clause 12.2.2, when the GNSS is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

Table A.9.1.1.1.1-1 and A.9.1.1.1.1-2 define test parameters for UE transmit timing accuracy tests for V2X. There is one GNSS based synchronization source during the test. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.6.1.

Table A.9.1.1.1.1-1: Supported test configurations for FR1 SL

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SL SCS 30 kHz, BW 20 MHz |
| 2 | NR TDD, SL SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.9.1.1.1.1-2: V2X Sidelink Test Parameters for UE Transmit Timing Tests for GNSS as Timing Reference

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | Band n47 or n38 |
| BWchannel | Config 1 |  | 20: NRB,c = 50 |  |
|  | Config 2 |  | 40: NRB,c = 100 |  |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | None |  |
| V2X sidelink communication preconfiguration |  |  | As specified in Table A.3.19.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel | Config 1 |  | [CC.1A TDD] |  |
|  | Config 2 |  | [CC.1A TDD] |  |
| PSSCH Reference Measurement Channel | Config 1 |  | [CD.1A TDD] |  |
|  | Config 2 |  | [CD.1A TDD] |  |
| S-SSB configuration |  |  | [S-SSB.1 FR1] |  |
| Propagation condition |  |  | AWGN |  |

A.9.1.1.1.2	Test requirements

For parameters specified in Tables A.9.1.1.1-1 and A.9.1.1.1-2, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.2. The timing accuracy is verified by using PSSCH transmissions.

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source

A.9.1.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 12.2.5, when SyncRef UE is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

Table A.9.1.1.2.1-1 and A.9.1.1.2.1-2 define test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active SyncRef UE in this test without either serving cell and or GNSS signals. Before the test starts, the UE has been synchronized to the SyncRef UE. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.9.1.1.2.1-1: Supported test configurations for FR1 SL

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SL SCS 30 kHz, BW 20 MHz |
| 2 | NR TDD, SL SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.9.1.1.2.1-2: General Test Parameters for V2X UE Transmit Timing Test for SyncRef UE as Timing Reference

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| RF Channel Number |  |  |  |  | 1 | Band n47 or n38 |
| BWchannel |  | Config 1 |  | MHz | 20: NRB,c = 50 |  |
|  |  | Config 2 |  |  | 40: NRB,c = 100 |  |
| Active cell |  |  |  |  | None |  |
| Active SyncRef UE |  |  |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| V2X sidelink communication preconfiguration |  |  |  |  | As specified in Table A.3.19.2-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  |  | Config 1 |  | [CC.1A TDD] |  |
|  |  |  | Config 2 |  | [CC.1A TDD] |  |
| PSSCH Reference Measurement Channel |  |  | Config 1 |  | [CD.1A TDD] |  |
|  |  |  | Config 2 |  | [CD.1A TDD] |  |
| S-SSB configuration |  |  |  |  | [S-SSB.1 FR1] |  |
| Note2 |  |  |  | dBm/15 kHz | -98 |  |
| Note1,2 | Config 1, 2 |  |  | dBm/SCS | -95 |  |
| SyncRef UE 1 | sl-SSB-TimeAllocation |  |  |  | sl-SSB-TimeAllocation1 |  |
|  | slssid |  |  |  | 30 |  |
|  | inCoverage |  |  |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  |  |  | ON |  |
|  | V2X sidelink communication configuration |  |  |  | As specified in Table A.3.19.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
|  |  |  |  |  | 3 |  |
|  | PSBCH-RSRP Note1, Note 2 |  |  | dBm/SCS | -92 |  |
| Propagation condition |  |  |  |  | AWGN |  |
| Note 1: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. |  |  |  |  |  |  |

A.9.1.1.2.2	Test requirements

For parameters specified in Tables A.9.1.1.2.1-1 and A.9.1.1.2.1-2, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.5. The timing accuracy is verified by using PSSCH transmissions.

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source

A.9.1.1.3.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 12.2.3, when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication.

Table A.9.1.1.3.1-1 A.9.1.1.3.1-2 and A.9.1.1.3.1-3 define test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active cell (PCell) in this test. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.9.1.1.3.1-1: Supported test configurations for FR1 PCell and SL

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz NR SL: TDD, SL SCS 30 kHz, BW 20 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz NR SL: TDD, SL SCS 30 kHz, BW 20 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz NR SL: TDD, SL SCS 30 kHz, BW 20 MHz |
| 4 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz NR SL: TDD, SL SCS 30 kHz, BW 40 MHz |
| 5 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz NR SL: TDD, SL SCS 30 kHz, BW 40 MHz |
| 6 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz NR SL: TDD, SL SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.9.1.1.3.1-2: V2X Sidelink Test Parameters for V2X UE Transmit Timing Accuracy Test for gNB as Timing Reference

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | Band n47 or n38 |
| BWchannel | Config 1,2,3 |  | 20: NRB,c = 50 |  |
|  | Config 4,5,6 |  | 40: NRB,c = 100 |  |
| Active cell |  |  | Cell 1 |  |
| Active SyncRef UE |  |  | None |  |
| V2X sidelink communication configuration |  |  | As specified in Table A.3.19.2-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel | Config 1,2,3 |  | [CC.1A TDD] |  |
|  | Config 4,5,6 |  | [CC.1A TDD] |  |
| PSSCH Reference Measurement Channel | Config 1,2,3 |  | [CD.1A TDD] |  |
|  | Config 4,5,6 |  | [CD.1A TDD] |  |
| S-SSB configuration |  |  | [S-SSB.1 FR1] |  |

Table A.9.1.1.3.1-3: Cell Test Parameters for V2X UE Transmit Timing Accuracy Test for gNB as Timing Reference

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1,4 |  | FDD |
|  | Config 2,3,5,6 |  | TDD |
| TDD configuration | Config 1,4 |  | Not Applicable |
|  | Config 2,5 |  | TDDConf.1.1 |
|  | Config 3,6 |  | TDDConf.1.2 |
| BWchannel | Config 1,2,4,5 | MHz | 10: NRB,c = 52 |
|  | Config 3,6 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |
|  | Config 2,5 |  | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |
|  | Config 2,5 |  | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD |
|  | Config 2,5 |  | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note2 | Config 1,2,3,4,5,6 | dBm/15 kHz | -98 |
| Note2 | Config 1,2,4,5 | dBm/SCS | -98 |
|  | Config 3,6 |  | -95 |
|  |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2,4,5 | dBm/SCS | -95 |
|  | Config 3,6 |  | -92 |
| Io Note 3 | Config 1,2,4,5 | dBm/9.36 MHz | -65.2 |
|  | Config 3,6 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

A.9.1.1.3.2	Test requirements

For parameters specified in Tables A.9.1.1.3.1-1 A.9.1.1.3.1-2 and A.9.1.1.3.1-3, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.3. The timing accuracy is verified by using PSSCH transmissions.

<End of Change 1>