3GPP TSG-RAN WG4 Meeting # 97-e                                  		R4-2015537

Electronic Meeting, 2nd – 13th November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for 38.101-1 to add BCS1 for SUL_n78A-n84A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R17-Core |  |  |  |  |  | Date: |  |  | 2020-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - To add BCS1 for SUL_n78A-n84A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To add BCS1 for SUL_n78A-n84A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specification can’t support BCS1 for SUL_n78A-n84A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change1>>

5.5C	Configurations for SUL

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration |  | NR Band |  | Subcarrier spacing (kHz) |  | 5 MHz |  | 10 MHz |  | 15 MHz |  |  | 20 MHz |  | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 70 MHz |  |  | 80 MHz |  | 90 MHz | 100 MHz |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n80A |  | n41 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n80 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n41A-n81A |  | n41 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n81 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n41A-n95A |  | n41 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n95 |  | 15 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n77A-n80A |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n80 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n77A-n84A |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n84 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n80A |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n80 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n81A |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n81 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n82A |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  | Yes | Yes |  |  |
|  |  | n82 |  | 15 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n83A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  | n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n84A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 1 |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| SUL_n78A-n86A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  | n86 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n80A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| SUL_n79A-n81A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  | n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n84A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n95A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  | n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5C-2: Supported channel bandwidths per SUL band combination with downlink CA

| SUL band combination with downlink CA | SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78(2A)_SUL_n78A-n86A | SUL_n78A-n86A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n86 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |

<<End of Change1>>