3GPP TSG-RAN WG4 Meeting #97-e	R4-2015610

# Electronic Meeting, 2nd - 13th Nov, 2020

Title: 	Discussion and simulation results on the UL timing adjustment

Source: 	Huawei, HiSilicon

Agenda item:	7.15.3.2.3

Document for:	Discussion

Background

During RAN4#96-e meeting, way forward [1] for NR Rel-16 HST BS demodulation requirements was approved. In this contribution, we share our view and provide our updated simulation results on the UL timing adjustment demodulation requirements.

Discussion

SCS/CBW combinations for Scenario X

- SCS/CBW combinations - Option 1: Have requirements for - 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz. - Option 2: Have requirements for - 15kHz: 5MHz; 30kHz: 10MHz. - Option 3: Have requirements for all Rel-15 non-HST PUSCH bandwidths. - Other options not precluded. - Applicability rules concerning SCS/CBW - Option 1: Re-use non-HST PUSCH applicability rules. - Option 2: Re-use HST PUSCH applicability rules. - Other options not precluded.

The only one issue remains is that the SCS/CBW combinations for Scenario X. In our view, we think that cases that only 5MHz CBW for 15 kHz SCS and 5MHz CBW for 15 kHz are sufficient. Firstly, the purpose for testing UL TA is to verify whether proper BS implementation can be performed for TO estimation and TA command transmission. For different bandwidth, the TA procedure is same and higher accuracy of TO estimation can be achieved with higher bandwidth. If a BS support higher bandwidth, it is surely that the performance can be better comparing the worst case that the smallest bandwidth is assumed. Also, with MCS 16, the working SNR is high enough to achieve rather good TO estimation performance even with the smallest bandwidth. Moreover, only requirements for 5MHz for 15 kHz SCS, 10MHz for 30 kHz SCS are defined in HST PUSCH.

Above all, it is no need to define requirements for other SCS/CBW combinations. Therefore, we prefer Option 2, i.e. have requirements only for 15kHz: 5MHz; 30kHz: 10MHz.

Proposal 1: For SCS/CBW combinations for Scenario X, define requirements only for 15kHz: 5MHz; 30kHz: 10MHz.

Simulations

As per updated simulation assumption in WF [1], we provide the following updated simulation results.

Table 3-1: Ideal Simulation results for UL TA

| Case Number | Scenario | CHBW/SCS | PUSCH mapping type | SNR@70% Max TP |
| --- | --- | --- | --- | --- |
| 1 | X | 5MHz/15kHz | A | 10.38 |
| 2 | X | 5MHz/15kHz | B | 11.29 |
| 3 | X | 10MHz/30kHz | A | 11.72 |
| 4 | X | 10MHz/30kHz | B | 12.28 |

Conclusion

In this contribution, we discuss and share our simulation results on UL TA requirements for NR HST. Our observations and proposals are:

Proposal 1: For SCS/CBW combinations for Scenario X, define requirements only for 15kHz: 5MHz; 30kHz: 10MHz.

Reference

R4-2012676, WF on Rel-16 NR HST BS demodulation requirements, RAN4#96-e, Nokia