3GPP TSG-RAN WG4 Meeting #97-e	R4-2015787

Electronic Meeting, 2 – 13 November, 2020

Title: 	Discussion on CSI-SINR accuracy and report mapping

Source: 	Huawei, HiSilicon

Agenda item:	7.14.2.1.3

Document for:	Discussion

Introduction

RRM core requirements for CSI-RS L3 measurement have been concluded in RAN4#96-e. According to the work plan, RAN4 should start to define accuracy requirements and report mapping for CSI-RS measurement.

In this paper we will provide our views on CSI-SINR accuracy and report mapping.

Discussion

Side condition

In our companion paper, we discussed the side condition for CSI-RSRP accuracy requirements. As the SINR measurement performance is impacted by the same factors as RSRP, the proposed side condition for CSI-RSRP can be reused for CSI-SINR.

As the exact accuracy would come from link level simulations from companies, and most of the simulation results submitted by companies are without timing error, RAN4 needs to collect CSI-SINR simulation results with timing error.

Proposal 1: The side condition of CSI-SINR measurement accuracy is defined as

- Es/Iot condition same as SSB L3 measurement

- BW of 48-RB and Density of D=3

- 5 measurement samples (it is captured as core requirements, and is used to derive the accuracy)

- 3us timing error (other values can be discussed for FDD)

Proposal 2: RAN4 to collect simulation results with timing error from interested companies.

Accuracy

In Table 1, we provide our simulation results based on the side conditions in Proposal 1. For comparison, the simulation results with zero timing error are also shown.

Table 1: Simulation results for CSI-RSRQ

| SCS | Channel | Timing error = 0 |  |  | Timing error = 3us |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | 5% | 50% | 95% |
| 15kHz | AWGN | -0.74 | -0.06 | 0.56 | -0.71 | -0.06 | 0.54 |
|  | EPA5 | -0.78 | -0.05 | 0.63 | -0.84 | -0.05 | 0.64 |
|  | ETU30 | -0.31 | -0.54 | 0.15 | -1.26 | -0.48 | 0.19 |
|  | ETU70 | -1.31 | -0.48 | 0.18 | -1.29 | -0.50 | 0.15 |
|  | TDL-A30 (3km/h) | -0.78 | -0.06 | 0.56 | -0.78 | -0.07 | 0.55 |
|  | TDL-C30 (30km/h) | -0.76 | -0.04 | 0.60 | -0.81 | -0.05 | 0.58 |
| 30kHz | AWGN | -0.70 | -0.03 | 0.54 | -1.02 | -0.29 | 0.32 |
|  | EPA5 | -0.82 | 0.03 | 0.58 | -1.08 | -0.29 | 0.36 |
|  | TDL-A30 (3km/h) | -0.79 | -0.04 | 0.55 | -1.08 | -0.30 | 0.30 |
|  | TDL-C30 (30km/h) | -0.74 | -0.06 | 0.54 | -1.10 | -0.35 | 0.34 |
| 60kHz (FR1) | AWGN | -0.73 | -0.06 | 0.58 | -2.07 | -1.16 | -0.38 |
|  | EPA5 | -0.74 | -0.06 | 0.56 | -2.08 | -1.15 | -0.34 |
|  | TDL-A30 (3km/h) | -0.82 | 0.03 | 0.57 | -2.11 | -1.16 | -0.32 |
|  | TDL-C30 (30km/h) | -0.73 | -0.04 | 0.57 | -0.09 | -1.16 | -0.36 |
| 60kHz (FR2) | AWGN | -0.74 | -0.04 | 0.58 | -2.05 | -0.15 | -0.38 |
|  | TDL-A30 (3km/h) | -0.81 | -0.07 | 0.55 | -2.13 | -1.14 | -0.39 |
|  | TDL-C30 (30km/h) | -0.76 | -0.07 | 0.58 | -2.06 | -1.16 | -0.37 |
| 120kHz | AWGN | -0.77 | -0.04 | 0.55 | -4.60 | -3.18 | -2.05 |
|  | TDL-A30 (3km/h) | -0.66 | -0.06 | 0.59 | -4.55 | -3.11 | -2.02 |
|  | TDL-C30 (30km/h) | -0.71 | -0.04 | 0.56 | -4.58 | -3.15 | -2.01 |

The simulation results are very similar to those for CSI-RSRP in our companion paper [1]. This is aligned with the expectation since the inaccuracy is mainly introduced by the estimation error in the RSRP (signal) part. Particularly, the same trend can be observed where the performance is quite dependent on the SCS of CSI-RS due to timing error. Therefore, it is suggested to define CSI-SINR accuracy to be SCS specific.

Proposal 3: CSI-SINR accuracy requirements are defined to be SCS specific.

Report mapping

In 38.133, the reporting range of SS-SINR for L3 reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. As the definition of CSI-SINR is quite similar as SS-SINR, it is straightforward to reuse the same report mapping.

Proposal 4: The range of CSI-SINR report is from -23 dB to 40 dB with 0.5 dB resolution.

Conclusions

In this paper we provided our views on CSI-RSRP accuracy and report mapping.

Proposal 1: The side condition of CSI-SINR measurement accuracy is defined as

- Es/Iot condition same as SSB L3 measurement

- BW of 48-RB and Density of D=3

- 5 measurement samples (it is captured as core requirements, and is used to derive the accuracy)

- 3us timing error (other values can be discussed for FDD)

Proposal 2: RAN4 to collect simulation results with timing error from interested companies.

Proposal 3: CSI-SINR accuracy requirements are defined to be SCS specific.

Proposal 4: The range of CSI-SINR report is from -23 dB to 40 dB with 0.5 dB resolution.

Reference

- R4-2015783, Discussion on CSI-RSRP accuracy and report mapping, Huawei, HiSilicon