3GPP TSG-RAN WG4 Meeting #97-e	R4-2015816

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	6.2.3.2

Source:	Ericsson

Title:	Test cases of MSG3 channel quality report on non-anchor carrier

Document for:	Discussion

# 1	Introduction

RAN4#96e agreed with the way forward on the test cases of channel quality reports on non-anchor carrier [1][2]. In this contribution, we provide the test case design according to the way forward.

- DL channel quality reporting in non-anchor carrier - Using 4-bit table in AWGN channel

# 2	Test cases

3GPP has introduced NB-IoT MSG3-based DL channel quality report on anchor carrier in Rel-14, and RAN4 accordingly defined the corresponding test cases with 4-bit reporting table in TS36.133 A.9.14.

In Rel-16 NB-IoT, the MSG3-based channel quality report is extended to non-anchor carrier. Since the core requirements of MSG3-based channel quality report on non-anchor carrier is basically same as Rel-14 channel quality reporting on anchor carrier, it is straightforward to reuse Rel-14 test cases for Rel-16 MSG3-based channel quality reporting test on non-anchor carrier.

Proposal 1: Reuse the Rel-14 MSG3-based channel quality report test on anchor for Rel-16 MSG3-based channel quality report test on non-anchor.

According to the TS36.331, the UE needs to measure the DL channel quality on the configured non-anchor carrier during RAR reception period and reports the corresponding NPDCCH repetition level in MSG3, if NB-IoT UE supports MSG3-based DL channel quality report and NW configures the random access response (RAR) is received on a non-anchor carrier as below.

TS36.331 V16.2.1 5.3.3.3/5.3.3.3a/5.3.7.4 1> if the UE is a NB-IoT UE: 2> if the UE supports DL channel quality reporting in MSG3 and cqi-Reporting is present in SystemInformationBlockType2-NB: 3> set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received as specified in TS 36.133 [16]; NOTE: The downlink channel quality measurements use measurement period T1 or T2, as defined in TS 36.133 [16].

Since the non-anchor carrier for RAR reception is configured by NPDDCH carrier index (npdcch-CarrierIndex-r14) in SIB22 (SystemInformationBlockType22-NB), we propose to configure npdcch-CarrierIndex-r14 for Rel-16 MSG3-based channel quality report test on non-anchor carrier as shown in Table 1.

Table 1	Example of test parameters for downlink channel quality reporting accuracy test on non-anchor carrier.

| Parameter | Unit | Value |
| --- | --- | --- |
| NB-IoT operational mode |  | Standalone |
| CP Length |  | Normal |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPUSCH repetition level |  | 1 |
| NPDCCH carrier index (ndpcch-CarrierIndex-r14) |  | 1 (Note 1) |
| Note 1: Index of non-anchor carrier configured in SystemInformationBlockType22-NB (See TS 36.331 [2]) |  |  |

Proposal 2: Configure NPDCCH carrier index (ndpcch-CarrierIndex-r14) for Rel-16 MSG3-based channel quality report test on non-anchor.

# 3		Summary

Proposal 1: Reuse the Rel-14 MSG3-based channel quality report test on anchor for Rel-16 MSG3-based channel quality report test on non-anchor.

Proposal 2: Configure NPDCCH carrier index (ndpcch-CarrierIndex-r14) for Rel-16 MSG3-based channel quality report test on non-anchor.

The detailed test cases are drafted in [3].

# References

- R4-2012194, “WF on Rel-16 NB-IoT RRM performance requirements”, Huawei, HiSilicon.

- R4-2012195, “List for RRM test cases for Rel-16 NB-IoT”, Huawei, HiSilicon.

- R4-2015817, “Draft CR: MSG3 based channel quality reporting on non-anchor carrier”, Ericsson.