3GPP TSG-RAN WG4 Meeting #97-e	R4-2015827

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.3.3.1

Source:	Ericsson

Title:	Simulation results of L1-SINR measurement accuracy

Document for:	Discussion

# 1	Introduction

In RAN4#96-e, interested companies provided the simulation results of L1-SINR measurement accuracy, but the discussion was postponed [1]. We provide our L1-SINR simulation results to discuss L1-SINR the measurement accuracy requirements.

# 2	Discussion

## 2.1	Simulation results

Table 1 summarizes our simulation results of L1-SINR accuracy where we show the 5%-ile and 95%-ile of L1-SINR measurements with the target SINR of -3dB. MAX-DIFF is given by max(abs(5%-ile – ideal SINR), (95%-ile – ideal SINR)), where the ideal SINR is -3dB. The detailed simulation parameters are shown in Appendix. Table 1 compares the measurement accuracy with different number of CMR/IMR samples. According to the core requirements, the number of samples for CMR and IMR are same and it is M=1 or 3 according to the configuration such as resource periodicity (periodic/aperiodic/semi-persistent) and/or measurement restriction configuration.

It is observed from the simulation results, measurement accuracy with M=1 is worse (Larger MAX-DIFF) than that with M=3. Considering the minimum requirements, we propose to derive L1-SINR measurement accuracy requirements based on M=1.

Proposal 1: Derive L1-SINR measurement accuracy requirements based on the simulation results with M=1.

Table 1	Simulation results of L1-SINR accuracy

|  |  |  | SCS=15kHz |  |  | SCS=30kHz |  |  | SCS=120kHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMR | IMR | Samples (M) | 5% (dB) | 95% (dB) | MAX-DIFF (dB) | 5% (dB) | 95% (dB) | MAX-DIFF (dB) | 5% (dB) | 95% (dB) | MAX-DIFF (dB) |
| CSI-RS | N/A | 1 | -4.9 | -2.0 | 1.9 | -4.5 | -1.6 | 1.5 | -4.5 | -1.8 | 1.5 |
|  |  | 3 | -4.1 | -2.6 | 1.1 | -3.8 | -2.3 | 0.8 | -3.8 | -2.3 | 0.8 |
| SSB | NZP-IMR | 1 | -5.1 | -1.9 | 2.1 | -4.7 | -1.7 | 1.7 | -4.7 | -1.7 | 1.7 |
|  |  | 3 | -4.2 | -2.5 | 1.2 | -3.9 | -2.2 | 0.9 | -3.9 | -2.2 | 0.9 |
| SSB | ZP-IMR | 1 | -5.0 | -2.0 | 2.0 | -4.7 | -1.7 | 1.7 | -4.6 | -1.8 | 1.6 |
|  |  | 3 | -4.2 | -2.5 | 1.2 | -3.9 | -2.2 | 0.9 | -4.0 | -2.2 | 1.0 |
| CSI-RS | NZP-IMR | 1 | -4.5 | -2.4 | 1.5 | -4.1 | -2.1 | 1.1 | -4.1 | -2.1 | 1.1 |
|  |  | 3 | -3.9 | -2.8 | 0.9 | -3.6 | -2.5 | 0.6 | -3.7 | -2.6 | 0.7 |
| CSI-RS | ZP-IMR | 1 | -4.8 | -2.0 | 1.8 | -4.5 | -1.8 | 1.5 | -4.5 | -1.8 | 1.5 |
|  |  | 3 | -4.1 | -2.6 | 1.1 | -3.8 | -2.3 | 0.8 | -3.8 | -2.3 | 0.8 |

## 2.1	Accuracy requirements

The existing L1-RSRP measurement accuracy requirements are set based on the following assumption [2].

FR1 Relative accuracy requirements - Considering fading condition (+1.0dB) and reporting step size of 1dB for absolute reporting and 2dB for differential reporting Normal condition; Extreme condition. SSB based relative L1-RSRP; [+/-3.0] dB; [+/-4.0] dB. CSI-RS based relative L1-RSRP; [+/-3.0] dB; [+/-4.0] dB. FR1 Absolute accuracy requirements - Normal condition with Io=-70dBm/BW is derived by adding 1.0dB (fading) + 2.5dB (RF margin) to the average of alignment results (1.5dB). => 1.5+2.5+1.0 = [+/-5.0]dB - Extreme condition with Io=-70dBm/BW is derived by adding 4.5dB to the absolute L1-RSRP accuracy in normal condition. - Normal condition with Io=-50dBm/BW is derived by adding 3.5dB to the absolute L1-RSRP accuracy in normal condition with Io=-70dBm/BW. - Extreme condition with Io=-50dBm/BW is derived by adding 2dB to the absolute L1-RSRP accuracy in extreme condition with Io=-70dBm/BW. FR2 Relative accuracy requirements - Considering fading condition (+1.0dB), RF margin for FR2 (+4.0dB) Normal condition; Extreme condition. SSB based relative L1-RSRP; [+/-6.5] dB; [+/-9.5] dB. CSI-RS based relative L1-RSRP; [+/-6.5] dB; [+/-9.5] dB. FR2 Absolute accuracy requirements - Normal condition with Io=-70dBm/BW is same as the relative accuracy of normal condition because FR2 cannot compensate the RF impairments, i.e., [+/-6.5]dB - Extreme condition with Io=-70dBm/BW is derived by adding 3dB to the absolute L1-RSRP accuracy in normal condition. - Normal condition with Io=-50dBm/BW is derived by adding 2dB to the absolute L1-RSRP accuracy in normal condition with Io=-70dBm/BW. - Extreme condition with Io=-50dBm/BW is derived by adding 2dB to the absolute L1-RSRP accuracy in extreme condition with Io=-70dBm/BW.

From the UE measurement functionality point of view, we don’t see any difference between L1-SINR and L1-RSRP measurements. We therefore propose to reuse the same methodology as L1-RSRP to derive L1-SINR measurement accuracy requirements.

Proposal 2: After taking an average of companies simulation results, adopt the same methodology as L1-RSRP measurement accuracy to derive L1-SINR measurement accuracy requirements.

# 3	Summary

Proposal 1: Derive L1-SINR measurement accuracy requirements based on the simulation results with M=1.

Proposal 2: After taking an average of companies simulation results, adopt the same methodology as L1-RSRP measurement accuracy to derive L1-SINR measurement accuracy requirements.

# References

- R4-2012218, “Email discussion summary for [96e][218] NR_eMIMO_RRM”, Moderator (Samsung).

- R4-1904820, “Way forward on L1-RSRP accuracy requirements”, Ericsson, Nokia, CMCC.

# Appendix

## Simulation assumption

General assumption:

- Number of CSI-RS port for CMR (if applicable), and NZP-IMR:

- N=1

L1-SINR accuracy evaluation simulation assumption for CMR only:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

L1-SINR accuracy evaluation simulation assumption for SSB-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1, 3 |
| Number of PRBs for IMR | 48 |
| Propagation condition | AWGN |

L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + NZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | option-1: 0dB, other options not excluded |
| Side condition (SNR) on IMR | option-1: 0dB, other options not excluded |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |

L1-SINR accuracy evaluation simulation assumption for CSI-RS-based CMR + ZP-IMR:

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1, 3 |
| Number of PRBs for CMR/IMR | 48 |
| Propagation condition | AWGN |