3GPP TSG-RAN WG4 meeting #97-e	R4-2015831

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	7.9.4.2.1

Source:	Ericsson

Title:	Simulation results of single-DCI based SDM transmission

Document for:	Information

# 1	Simulation parameters

This contribution provides the simulation results based on WF agreed in RAN4#96-e [1].

| Parameters | Simulation cases 3-1 and 3-2 | Simulation cases 4-1 and 4-2 |
| --- | --- | --- |
| Duplex mode | FDD | TDD |
| TDD UL/DL configuration | N/A | FR1.30-1 (7DS2U, 6D+4G+4U) PDSCH is scheduled in the special slots |
| Timing offset of the second TRP from the first TRP (Note 2) | Positive offset: 2 μs Negative offset: -0.5 μs | Positive offset: 1μs Negative offset: -0.25μs |
| Frequency offset between two TRPs (Note 2) | 200 Hz | 300 Hz |
|  | Common serving cell parameters |  |
| Physical Cell ID | 0 |  |
| SSB position in burst | First SSB in Slot #0 |  |
| SSB periodicity | 20ms |  |
| Transmit TRP of SSB | Only from TRP1 |  |
|  | TRS#1 (configuration for TCI state #1) |  |
| Transmit TRP | TRP#1 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=0 | k0=0 |
| QCL Info | TCI state #0 | TCI state #0 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
|  | TRS#2 (configuration for TCI state #2) |  |
| Transmit TRP | TRP#2 |  |
| CSI-RS periodicity | [20] slots | [40] slots |
| CSI-RS offset | [10] for CSI-RS resources 1 and 2 [11] for CSI-RS resources 3 and 4 | [20] for CSI-RS resources 1 and 2 [21] for CSI-RS resources 3 and 4 |
| Density | 3 | 3 |
| First subcarrier index in the PRB used for CSI-RS | k0=1 | k0=1 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |
| QCL Info | TCI state #0 | TCI state #0 |
|  | TCI State configuration #0 |  |
| Type 1 QCL information | SSB index #0, QCL type C |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #1 |  |
| Type 1 QCL information | CSI-RS resource: TRS#1, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | TCI State configuration #2 |  |
| Type 1 QCL information | CSI-RS resource: TRS#2, QCL type A |  |
| Type 2 QCL information | N/A |  |
|  | PDCCH configuration |  |
| CORESETPoolIndex | 0 (or not configured) |  |
| Symbols for PDCCH | 0, 1 |  |
| K0 | 0 |  |
| AL | 8 (576REs) |  |
|  | PDSCH configuration |  |
| PDSCH resource mapping type | Type A |  |
| Resource allocation type | Type 1 |  |
| DMRS configuration | Type 1, 1+1 |  |
| Antenna ports index ​ | {1000} for TCI#1 and {1002} for TCI#2 |  |
| Start symbol (S) | 2 |  |
| Time duration (L) | 12 |  |
| PRB allocation | Full-overlapping case PDSCH#1: PRB#0 to RPB#51 PDSCH#2: PRB#0 to RPB#51 | Full-overlapping case PDSCH#1: PRB#0 to RPB#105 PDSCH#2: PRB#0 to RPB#105 |
| Note 1: Apply a power scaling factor as 1/sqrt(2) for transmitted signaling from each TRP. Note 2: Using TRP1 is Reference to define timing and frequency offset Note 3: PDSCH is not scheduled in slots where SSB is transmitted (slot #0 with periodicity of 20ms) Note 4: FFS whether PDSCH is scheduled in slots where TRSs are transmitted. |  |  |

| Simulation cases | Duplex mode | BW / SCS | MCS | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | FDD | 10MHz / 15kHz | [64QAM 0.5] MCS19 | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 2us [Test 1a: 200Hz / -0.5us] | 70% of max Tput |
| 3-2 | FDD | 10MHz / 15kHz | [64QAM 0.5] MCS19 | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 2us [Test 1a: 200Hz / -0.5us] | 70% of max Tput |
| 4-1 | TDD | 40MHz / 30kHz | [64QAM 0.5] MCS19 | [TDLA30-10] for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | Test 1b: 0Hz / 1us [Test 1a: 300Hz / -0.25us] | 70% of max Tput |
| 4-2 | TDD | 40MHz / 30kHz | [64QAM 0.5] MCS19 | [TDLA30-10] for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | Test 1b: 0Hz / 1us [Test 1a: 300Hz / -0.25us] | 70% of max Tput |

# 2	Simulation results

## 2.1	FDD SCS=15kHz

Figure 1 shows our ideal simulation results of sDCI-based SDM for FDD SCS=15kHz with different TO/FO setting.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 1	Simulation results of sDCI-based SDM transmission with FDD SCS=15kHz.

## 2.2	TDD SCS=30kHz

Figure 2 shows our ideal simulation results of sDCI-based SDM for TDD SCS=30kHz with different TO/FO setting.

| (a) 2Rx | (b) 4Rx |
| --- | --- |

Figure 2	Simulation results of sDCI-based SDM transmission with TDD SCS=30kHz.

# References

- R4-2012664, “Simulation assumption for PDSCH requirements with Single-DCI SDM scheme and Multi-DCI transmission schemes”, Ericsson.