3GPP TSG-RAN WG4 Meeting #97-e	R4-2015848

Electronic Metting, 2nd – 13th November, 2020

Source:	Ericsson

Title:	Scenario X and additional scenarios Y and Z NR PUSCH UL timing adjustment simulation results

Agenda item:	7.15.3.2.3

Document for:	Discussion

# Introduction

In this contribution, we presented our simulation results for scenario Z NR HST PUSCH UL timing adjustment.

# Simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 for UL timing adjustment. The corresponding ideal simulation results and results with impairments are given in Table 2 and Table 3 respectively.

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Channel model | Scenario X: Stationary UE: AWGN Moving UE (SCS = 15 kHz): TDLC300-400 Moving UE (SCS = 30 kHz): TDLC300-800 | Scenario Y and Z: Stationary UE: AWGN Moving UE: AWGN |  |
| Number of Tx and Rx | 1Tx 2Rx |  |  |
| Number of DMRS symbols | (1+1+1) DM-RS symbols with l0 = 2 for PUSCH mapping type A, and l0 = 0 for PUSCH mapping type B |  |  |
| (A, Δω) | Scenario X: SCS = 15 kHz: (10 µs, 0.04 s-1) SCS = 30 kHz: (5 µs, 0.08 s-1) | Scenario Y: SCS = 15 kHz: (10 µs, 0.13 s-1) SCS = 30 kHz: (5 µs, 0.26 s-1) | Scenario Z: SCS = 15 kHz: (10 µs, 0.18 s-1) SCS = 30 kHz: (5 µs, 0.36 s-1) |
| System bandwidth | Scenario X: SCS = 15 kHz: 5 & 10MHz SCS = 30 kHz: 10 & 40 MHz | Scenario Y: SCS = 15 kHz: 5MHz SCS = 30 kHz: 10MHz | Scenario Z: SCS = 15 kHz: 5MHz SCS = 30 kHz: 10MHz |
| Frequency domain resource | SCS = 15 kHz & 10 MHz CBW: 25 PRBs for each UE SCS = 30 kHz & 40 MHz CBW: 50 PRBs for each UE SCS = 15 kHz & 5 MHz CBW: 12 PRBs for each UE SCS = 30 kHz & 10 MHz CBW: 12 PRBs for each UE |  |  |
| MCS index | 16 |  |  |
| SRS (optional) | SCS = 15 kHz & 10 MHz CBW: single region with length of 40 PRBs SCS = 30 kHz & 40 MHz CBW: single region with length of 80 PRBs SCS = 15 kHz & 5 MHz CBW: single region with length of 20 PRBs SCS = 30 kHz & 10 MHz CBW: single region with length of 20 PRBs |  |  |
| Testing metric | SNR @70% of maximum throughput |  |  |

Table 1. Simulation assumptions for additional UL timing adjustment requirements for scenario X, Y and Z [1].

Table 5. Ideal simulation results for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario X | 15 kHz / 5 MHz | A | 8.3 |
|  |  | B | 8.4 |
|  | 30 kHz / 10 MHz | A | 8.8 |
|  |  | B | 8.7 |
| Scenario X | 15 kHz / 10 MHz | A | 9.3 |
|  |  | B | 9.4 |
|  | 30 kHz / 40 MHz | A | 9.9 |
|  |  | B | 9.8 |
| Scenario Y | 15 kHz / 5 MHz | A | 6 |
|  |  | B | 6 |
|  | 30 kHz / 10 MHz | A | 6 |
|  |  | B | 6 |
| Scenario Z | 15 kHz / 5 MHz | A | 6 |
|  |  | B | 6 |
|  | 30 kHz / 10 MHz | A | 6 |
|  |  | B | 6 |

Table 6. Results with impairments for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 70% of maximum throughput |
| --- | --- | --- | --- |
| Scenario X | 15 kHz / 5 MHz | A | 10.5 |
|  |  | B | 10.8 |
|  | 30 kHz / 10 MHz | A | 11.3 |
|  |  | B | 10.8 |
| Scenario X | 15 kHz / 10 MHz | A | 11.8 |
|  |  | B | 11.9 |
|  | 30 kHz / 40 MHz | A | 12.2 |
|  |  | B | 12.3 |
| Scenario Y | 15 kHz / 5 MHz | A | 8.1 |
|  |  | B | 8.4 |
|  | 30 kHz / 10 MHz | A | 8.2 |
|  |  | B | 8.1 |
| Scenario Z | 15 kHz / 5 MHz | A | 8.3 |
|  |  | B | 8.2 |
|  | 30 kHz / 10 MHz | A | 8.2 |
|  |  | B | 8.5 |

# Conclusion

In this contribution. we summarized our additional 70% of maximum throughput simulation results for scenario X. Y and Z NR UL timing adjustment.

# References

R4-2012676, “WF on Rel-16 NR HST PUSCH BS demodulation requirements”, RAN4#96-e, Nokia, Nokia Shanghai Bell, …